3GPP TSG-WG SA2 Meeting #166	S2-2412299

Orlando, USA, Nov 18 – 22, 2024

Source:	China Mobile

Title:	Conclusions for KI#1 Topology 2 networking --RRC option

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Conclusions for KI#1 are proposed.

# 1. Introduction/Discussion

The RRC option has two architectures. Option A is a direct connection between AIoTF and gNB/AIoT RAN, while Option B is an indirect connection between AIoTF and gNB/AIoT RAN via AMF. From a pure technology perspective, Option A seems to have advantages over Option B due to direct delivery without AMF in the path.  However, from network deployment, network maintenance, and network evolution perspectives, the operator has the following concerns about Option A:

- From a network deployment perspective, AIoTF in Option A needs to handle connection management with gNB/AIoT RAN. Then, it has to be deployed at a low level in the core network (same level as AMF). However, from an ambient IoT service perspective, it is a web-based background service. The AIoTF should be deployed at an upper level in the core network, such as the same level as UDM/NEF, to provide a cost-efficient solution. In addition, the Topology 1 network solution will deploy the AIoTF at an upper level in the core network to manage BS readers via AMF. If Option A in the RRC option is adopted for Topology 2, the AIoTF will be deployed at a lower level in the core network (same level as AMF).  Thus, Operators can’t have the unified AIoTF for both topology 1 and topology 2 .

- From a network maintenance perspective, AIoTFs with huge capacity at an upper level in the core network will be deployed to reduce total numbers and significantly lower maintenance costs. If AIoTFs are deployed at a low level in the core network (the same level as AMF), then the operator must deploy many AIoTFs with small capacity.

- gNB needs to have extra interface with AIoTF.

- From a network evolution perspective, Outdoor application scenarios will be considered in R20. It needs AMF to support mobility management.

Proposal:  gNB/AIoT RAN connect to AIoTF via AMF  in RRC option

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13.

FIRST CHANGE

### 8.1.3	Architecture to Support Topology 2

#### 8.1.3.1	General

Two options will be specified for Topology 2:

-	User-plane option as defined in clause 8.1.3.2.

-	RRC-based option as defined in clause 8.1.3.3.

Editor's note:	Whether a NAS-based option is needed in addition is FFS.

#### 8.1.3.2	User-plane option

The following principles apply:

-	As depicted in Figure 8.1.3.2-1, the UE Reader connects to the AIoTF based on the AIoT Application Protocol (AIoT-AP) using an IP PDU Session between the UE and the UPF as transport. The related protocol stack is shown in Figure 8.1.3.2-2. The AIoT AP protocol will support procedures and information to be exchanged as specified by RAN WG2, RAN WG3 and SA WG2.

Figure 8.1.3.2-1: User-plane architecture for Topology 2

Figure 8.1.3.2-2: Protocol stack for the user-plane architecture for Topology 2

NOTE 1:	Which transport protocol to use for AIoT-AP can be decided by CT WG1 in coordination with RAN WG3.

NOTE 2:	Security for AIoT-AP is assumed to be defined by SA WG3.

Editor's note:	Further details are FFS.

Editor's note:	The terminology, e.g. AIoT RAN is FFS.

#### 8.1.3.3	RRC option

The following principles apply:

-	Messages between the UE Reader and the AIoTF are delivered using RRC and a protocol between gNB/AIoT RAN and AIoTF. The related protocol stack is shown in Figure 8.1.3.3-1.

Editor's note:	How addressing works for UL  is FFS.

Editor's note:	Further details are FFS.

Figure 8.1.3.3-1: Protocol Stack for the RRC option

END OF CHANGES