3GPP SA WG2 Meeting #166 	S2-2412348

Orlando, US, Nov 18 – 22, 2024

# Source:	KPN N.V.

# Title:	New SID on Architecture support of Ambient power-enabled Internet of Things Phase 2

# Document for:	Information

# Agenda Item:	30.8

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Architecture support of Ambient power-enabled Internet of Things Phase 2

Acronym:	FS_AmbientIoT_Ph2

Unique identifier:	TBD

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_AmbientIoT | SA2 | 1020071 | Study on Architecture support of Ambient power-enabled Internet of Things |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

TS 22.369 captures the use cases, scenarios, and performance requirements for ambient power-enabled Internet of Things. The functional service requirements defined in SA1 are organized into six categories:

- Communications aspects;

- Positioning;

- Management;

- Exposure;

- Charging;

- Security and privacy.

The study on Architecture support of Ambient power-enabled Internet of Things in Release 19 addressed the initial architectural enhancements to support security, identification, connection management and Ambient IoT inventory and command services. However, multiple aspects remain to be addressed:

Ambient IoT device and reader association:

When an Ambient IoT device is within reach from multiple readers, it might respond to all of them when prompted, which can increase network load. To minimize this, Ambient IoT device and reader association can be considered to configure the device to respond to a single reader.

Additionally, the performance service requirements as defined in TS 22.369 include scenarios where the device can be moving at different speeds. In case of topology 2, the UE reader can also move away from the Ambient IoT device. In that case, mechanisms are needed to update the device and reader association accordingly.

Roaming support:

Ambient IoT use cases described in TR 22.840 include logistics scenarios where the devices are used to track merchandise that is shipped across countries. The 5G system needs to support the architecture and procedures to enable reachability of Ambient IoT device when moving between one or more networks and countries.

AIoT device management and configuration:

As specified in TS 22.369, mechanisms are required to temporarily or permanently disable Ambient IoT devices. It is also necessary to support mechanisms to provide or update the configuration of one or more Ambient IoT devices to perform services.

Positioning:

Support location services for Ambient IoT devices (e.g., to locate Ambient IoT devices using absolute or relative positioning methods) for tracking.

Energy availability considerations:

The Ambient IoT devices are energy constrained and may receive different amounts of energy from different sources. At the same time, the Ambient IoT service requirements as defined in TS 22.369 include different message sizes, communication ranges and transfer intervals, which means that the amount of required energy to perform a task differs. Thus, mechanisms are needed to support the Ambient IoT services based on energy availability at the device.

# 4	Objective

The following objectives are considered for this study:

- WT#1 Ambient IoT device and reader association.

  - WT#1.1 Study how to support Ambient IoT device association to a reader;

  - WT#1.2 Study how to support Ambient IoT device association update in case of mobility of the device or reader.

- WT#2 Architecture to support Ambient IoT device reachability and communication in roaming scenarios.

- WT#3 Architecture to support AIoT device management and configuration

  - WT#3.1 Study how to support temporary disablement and re-enablement of AIoT devices;

  - WT#3.2 Study how to support configuration provisioning and update of one or a group of AIoT devices;

- WT#4 Architecture to support AIoT device positioning.

- WT#5 Support Ambient IoT services based on energy availability;

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.XXX | Study on Architecture support of Ambient power-enabled Internet of Things Phase 2 | TBD | TBD |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Security aspects are considered by SA3.

Charging aspects are considered by SA5.

RAN aspects are considered by RAN WGs.

# 9	Supporting Individual Members

Supporting IM name

KPN N.V.

TNO