SA WG2 Meeting #S2-166	S2-2412361

18 - 22 November, 2024, Orlando, USA

3GPP TSG-SA3 Meeting #119	S3-245259

# Orlando, US, 11th - 15th November 2024

Title:	LS on Multi-hop U2N Relay Architecture Aspects

Response to:

Release:	Rel-19

Work Item:	FS_5G_ProSe_Ph3_SEC

Source:	SA3

To:	SA2

Cc:	CT1

Contact person:	Samir Ferdi

samir dot ferdi at Interdigital dot com

Send any reply LS to:   3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall description

SA3 has studied security solutions for the support of multi-hop UE-to-Network Relay communications (KI#1 of TR 33.743) and would like to request SA2 feedback on the following architectural aspects to assist SA3 in moving forward its conclusions.

Q1: Intermediate Relay’s PKMF reachability via dedicated DNN for UE-to-Network Relay

Some solutions propose that the Intermediate Relay plays the role of the UE-to-Network Relay during link establishment procedure with the Remote UE (or with a child Intermediate Relay). For that, the Intermediate Relay is assumed to be able to reach its HPLMN’s PKMF via the dedicated DNN for UE-to-Network Relay (after successful connection establishment with the UE-to-Network Relay). The same assumption applies for each Intermediate Relay in the path between the Remote UE and the UE-to-Network Relay. Can SA2 verify the assumption, i.e., whether an Intermediate Relay can use the dedicated DNN for UE-to-Network Relay to communicate with its HPLMN’s PKMF?

Q2: Intermediate Relays and the UE-to-Network Relay belonging to the same PLMN in Model A and Model B discovery.

SA3 discussed a possible simplification of the Model A and Model B discovery with the assumption that the intermediate Relays and the UE-to-Network Relay belong to the same PLMN. This assumption is considered in particular to avoid the need for Remote UE to send or process multiple discovery messages trying different candidates PLMN security material. Can SA2 verify the assumption, i.e., whether it can be assumed for Model A and Model B discovery that the Intermediate Relays and the UE-to-Network Relay belong to the same PLMN?

Actions

To SA2:

ACTION: 	SA3 kindly asks SA2 to provide answers to the questions.

# 3	Dates of next TSG SA WG 3 meetings

SA3#120			14 - 18 February 2025				Athens (Greece)

SA3#121			04 - 07 April 2025				Goteborg (Sweden)