3GPP TSG-SA2 Meeting #166	S2-2412430

# 18th – 22nd November 2024, Orlando FL. USA                           (revision of S2-2411828)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1208 | rev | 10 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the general inference procedure for vertical federated learning to resolve ENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, [ZTE, Samsung, Nokia, Lenovo, Toyota, ICS, Huawei, HiSilicon, Ericsson], Interdigital, Tencent, ETRI. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Editor's note: How the VFL server determines the VFL clients and whether the VFL server may not select all VFL clients is FFS. is removed since clause 6.2H.2.1 describes the VFL discovery. The UE ID(s) is added in the request send from the VFL server to the VFL client to indicate the VFL client which UE will be used as the sample to let VFL client collect the data and perform the VFL inference. The Editor's note: Whether the VFL model correlation ID is needed or using the model ID is FFS. is removed. The local model in each client can be different. Model ID is designed to identiy the sepcfic model. The model ID is same means the model should have the same topology. However, in the VFL case, the topology for each client local model can be different. It is not suitable to use the same model ID for every client. The VFL model correlation ID is perferred to indicate those local model in each client should used for the same VFL task e.g. for the same analytcis ID. The VFL model correlation ID is added when the VFL client send the VFL inference result to the VFL server. VFL model correlation ID is used to let VFL server know which inference result from which client need to be combined to generate the VFL inference result. For the NWDAF as the server case, no NEF new behaviour is needed. Therefore, the Editor's note: If any of the Consumer, Server and clients are untrusted AF(s), how the NEF assists the VFL inference process, and whether the existing or new NEF service should be invoked are FFS. is refined to only capture the service opetion. VFL Server can based on the the clients contributed weight to determine not all the clients need to be selected to perform VFL inference. For example, some of the feature in some clients are not important or has less impact to the VFL inference reuslt accuracy depending on the VFL training result. Therefore, the Editor's note: It is FFS how the origin of analytics results can be traced and explained if not all clients participate and results are not satisfying is removed. The VFL server monitoing is planed to has less priority in this release, therefore, the Editor's note: The details on how the VFL server monitoring the accuracy of the VFL is FFS. is remove in this release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Several ENs are resolved for the VFL inference procedure. Parameter from Analytics subscription such as analytics target and analytics filer is added in the request send from the VFL server to the VFL client and in related replies. The VFL model correlation ID is added when then VFL client send the VFL inference result to the VFL server. Several ENs are refined or removed for the VFL inference procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can not perform VFL inference sucessully if some open issue can not be resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G, 6.2H.1, 6.2H.2.4(new), 6.2H.2.4.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2G	Void

6.2G.1	Void

6.2G.2	Void

6.2G.2.1	Void

# * * * * Next change * * * *

6.2H.1	General

This clause specifies how NWDAF as a VFL server can register and also discover other NWDAFs and/or AFs via NRF

Both the VFL server and VFL client store the VFL model after finishing the VFL training process and use the same VFL local model to perform the VFL inference later based on the VFL correlation ID. The differences between the VFL training and inference are that for the inference there is no check of the labels, nor any server intermediate results are sent and as such only client intermediate results are sent to the server.

Editor's note:	This clause needs to be update to include the description of the subclauses for NWDAF or AF as the server case.

# * * * * Next change * * * *

6.2H.2.4	Inference procedure for vertical federated learning

6.2H.2.4.1	Inference procedure for vertical federated learning when NWDAF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF is acting as VFL server

0.	The analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g., UE IDs, and optionally Analytics Reporting Information=Analytics target period and Analytics Filter) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF determines the VFL Server for the requested analtyics, sends a subscription request to NWDAF VFL server using Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g., UE IDs and optionally Analytics Reporting Information=Analytics target period and Analytics Filter.

2.	Based on the information received in the step 0 or 1, VFL server decides to initiate the VFL inference procedure with the VFL clients. VFL Server selects clients(s) using information stored in the VFL training process. The server may select some or no clients, e.g., depending on the accuracy of the VFL model, the contribution to the training result and the current status of the VFL clients.

When no VFL Clients are selected, the VFL server generates the VFL inference results based only on its local trained ML model associated with the determined VFL correlation ID, skipping the steps 2 – 6 (and 7 if step 1 was also skipped).

VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients including the Target of VFL inference = e.g., UE IDs, VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used and optionlly VFL inference filter.

Editor's note:	It is FFS whether additional parametersare needed to send from the VFL server to VFL client.

2a.	For each NWDAF VFL client, the VFL Server NWDAF sends an Nnwdaf_VFLInference_Subscribe or Nnwdaf_VFLInference_Request to the VFL client.

2b.	For each trusted AF VFL client, the VFL Server NWDAF sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the VFL client.

2c.	For each untrusted AF VFL client, the VFL Server NWDAF sends an Nnef_VFLInference _Subscribe or Nnef_VFLInference _Request to the NEF serving the AF.

2d.	For each untrusted AF VFL client, the NEF converts any internal identifiers to external identifiers and sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the untrusted AF VFL client.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	VFL Client sends the client intermediate results to the VFL server.

The intermediate results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to combine and generate the VFL inference results.

Editor's note:	It is FFS additional parameters are needed to send from the VFL client to VFL server.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

5a.	Each NWDAF VFL client sends an Nnwdaf_VFLInference_Notify or Nnwdaf_VFLInference_Request response to the VFL Server NWDAF.

5b.	Each trusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the VFL Server NWDAF.

5c.	Each untrusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external to internal identifiers and sends an Nnef_VFLInference_Notify or Nnef_VFLInference _Request response to the NWDAF VFL server.

6.	The VFL server may collect its local data and generate the intermediate local inference results. When the VFL Server selected VFL clients to participate in the VFL Inference process, it combines all the intermediate results to generate the VFL inference results based on the VFL correlation ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate results when generates the combined inference output.

7.	Depending on request, the NWDAF VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify to the consumer (i.e NWDAF containing AnLF) including the VFL inference results.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

6.2H.2.4.2	Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

The inference procedure when untrusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggerd by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped. The inference procedure triggerd by internal service logic of the AF acting as VFL server is out of 3GPP scope.

0.	same as step 0 in 6.2H.2.4.1.

1.	same as step 1 in 6.2H.2.4.1, to NEF using Nnef_ Inference_subscribe. NEF forwards the subscription request to AF using Naf_ Inference _subscribe.

Editor's note: When the AnLF determine the VFL server AF is FFS. For example, during the training phase.

Editor's note: The service name between the AnLF and untrusted AF as VFL server is FFS.

2.	Same as step 2 in 6.2H.2.4.1, to NEF using Nnef_ VFLInference _subscribe. An untrusted AF includes the external NWDAF ID and sends the request to the NEF.

NEF converts any received external identifiers to internal identifiers and forwards the subscription request to NWDAF using Nnwdaf_VFLInference_subscribe.

Editor's note:	It is FFS additional parameters are needed to send from the VFL server to VFL client.

3.	Same as step 3 in 6.2H.2.4.1.

4.	Same as step 4 in 6.2H.2.4.1.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	Same as step 5 in 6.2H.2.4.1, to NEF using Nnwdaf_ VFLInference _Notify.

NEF converts any received internal identifiers to external identifiers and forwards the subscription notify to the untrusted AF using Nnef_ VFLInference _Notify.

Editor's note:	It is FFS additional parameters are needed to send from the VFL client to VFL server.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

6.	Same as step 6 in 6.2H.2.4.1.

7.	Same as step 7 in 6.2H.2.4.1, to NEF using Naf_ Inference _Notify.

NEF converts any received internal identifiers to external identifiers and forwards the subscription Notify to NWDAF using Nnef_ Inference _Notify.

8.	Same as step 8 in 6.2H.2.4.1.

# * * * * End of changes * * * *