3GPP TSG SA WG2 Meeting #166	S2-2412910

# 18 – 22 November, 2024, Orlando, USA                                (revision of S2-2412686)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1524 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for supporting of third party user identity information in IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedure details for supporting third party user identity information in IMS to align the conclusions in KI#4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Feature procedural details along in the lines of KI#4 conclusions specified in TR 23.700-77 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specififcation will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AX (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The new clause of this CR is to be implemented after CR1478 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# AX	Provisioning of IMS user specific properties

Support for provisioning of information which can be used for an IMS user in the IMS subsystem (e.g. RCD server address, RCD URL or RCD information for a single IMPU or wildcard IMPU as described in clause XX.2.2.6) is depicted below.

Figure AX.1-1: IMS user specific properties provisioning procedure

- The AF (e.g., located in a third party network) provides information which can be used for an IMS user in the IMS subsystem, e.g., RCD server address, RCD URL or RCD information for a single IMPU or wildcard IMPU using Nnef_ImsPP_Create service operation as described in clause 5.2.6 of TS 23.502 [55].

- NEF after receiving the provisioning information can locate an IMS instance as described in clause XX.2.2.6 and provisions the received parameters to IMS AS using Nimsas_ImsPP_Create service operation.

- IMS AS validates the retrieved information from NEF (e.g. check the syntax and semantics of the information).

- IMS AS stores the IMS user information in the HSS repository data using existing Nhss_ImsSDM_Update service operation.