SA WG2 Meeting #S2-167	S2-2502427

17 - 21 Feb, 2025, Athens, Greece

Title:	Reply LS on emergency call back and paging

Response to:	R2-2411041/S2-2501367

Release:	Rel-17

Work Item:	NR_newRAT-Core, NR_redcap-Core

Source:	SA2

To:	RAN2

Cc:	CT1, RAN3

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu.jinguo@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks RAN2 for the LS on emergency services and eDRX (S2-2500044/R2-2411041). The following question is raised by RAN2 in that LS:

In RAN2#128, companies discussed a scenario where a UE is configured with a RAN eDRX cycle in RRC_INACTIVE state when the UE has a PDU session associated with emergency services. RAN2 notice the below description in TS 23.501 which states that an Idle eDRX cycle should not be used by the UE when it has a PDU session associated with emergency services. However, it is unclear whether it is possible for such UE to be released to RRC_INACTIVE state with a RAN eDRX cycle and if it is so how UE behavior may be.

#### 5.31.7.2 Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE ##### 5.31.7.2.1 Overview ##### // skipped irrelevant part When the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception as defined in clause 5.4.5 and shall not use the extended idle mode DRX. Extended idle mode DRX parameters may be negotiated while the UE has PDU Session associated with emergency services. When the PDU Session associated with emergency services is released, the UE and AMF shall reuse the negotiated extended idle mode DRX parameters in the last Registration Update procedure. // skipped irrelevant part

Based on current specification, a RAN eDRX cycle can be configured by a RAN node when releasing the UE to RRC_INACTIVE state only if the UE is configured with an Idle eDRX cycle by CN.

RAN2 assumes that RAN node should be aware whether a UE has a PDU session associated with emergency services so that it can avoid releasing such UE to RRC_INACTIVE state with a RAN eDRX cycle, or even if it is so, there should be means for the RAN node and the UE to not use the configured RAN eDRX cycle. Regarding how RAN node can be aware of such PDU session, RAN2 understands that CN can already indicate this (e.g. via ARP value) but there is no requirement for CN to do this, so, there is case that RAN node may not be aware of this.

RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed?

SA2 analysed the identified issue above and would like to provide the following feedback:

- Existing specifications have specified that each PLMN/operator reserves a specific ARP value for emergency PDU session and provide to RAN node. See related text in TS 23.501 clause 5.16.4.3, 5.16.4.7, 5.16.4.9 and TS 23.502 clause 6.1.3.10. Therefore the RAN node can be aware of an emergency PDU session based on the reserved ARP value when the PDU session context exist in the RAN node.

- In addition there is inactivity monitoring for the presence of user plane resources for the emergency PDU session in SMF as defined in TS 23.502 clause 4.4.2.2 and TS 23.503 clause 6.1.3.10.

Based on the above, some companies in SA2 believe that the RAN node is capable through configuration to not release the emergency PDU session user plane resources based on existing implementation, and to not release the UE to RRC_INACTIVE state/RRC_IDLE state when the UE has an emergency PDU session. Therefore the issue highlighted in RAN2 LS may not exist. However there is no consensus in SA2 whether the UP of emergency PDU session can be deactivated, or whether to introduce new signalling to NG-RAN.

SA2 would like to ask RAN2 to provide feedback.

2. Actions:

To RAN2 group:

ACTION:  SA2 kindly requests RAN2 to take the above information into account

3. Date of Next SA2 Meetings:

SA2#168                                    7th – 11st April 2025	 		Goteborg, Sweden

SA2#169                                    19th – 23rd May 2025	 		Fukuoka, Japan