SA WG2 Meeting #167	S2-2501363

17 - 21 February, 2025, Athens, Greece

SA WG2 Meeting ##166-Ad-Hoc-e	S2-2500040

20 - 24 January, 2025, Electronic meeting

3GPP TSG-CT WG4 Meeting #126	C4-245437

# Orlando, U.S.; 18th – 22nd November 2024

# Title:	Reply LS on RAN support of QoS monitoring capability

# Response to:	Reply LS (S2-2411247) on RAN support of QoS monitoring capability from SA WG2

# Release:	Rel-19

# Work Item:	TEI19_QME

Source:	CT WG4

To:	SA WG2

Cc:	CT WG3

Contact Person:

Name:	Caixia Qi

Tel. Number:

E-mail Address:	Caixia.qi@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

CT4 thanks SA WG2 for the reply LS on RAN support of QoS monitoring capability. CT4 has discussed it and has the following answers to SA2 questions:

SA2 Question 1: Why is the RAN QoS monitoring capability value "UNKNOWN" needed. What is the expected SMF behaviour compared to receiving "NOT_SUPPORTED" or no indication at all?

CT4 answer: As the QoS monitoring for packet delay function is introduced in previous release, but the new feature QME is introduced in Rel-19. In Inter-AMF mobility with I-SMF/V-SMF change procedure, the target AMF in Rel-18 will not indicate the support of QME feature and cannot send the new attribute qosMonitoringPdSupported to the I-SMF/V-SMF, however the I-SMF/V-SMF cannot interpret that the QoS monitoring for packet delay is "NOT_SUPPORTED" by NG-RAN. "UNKNOWN" value is used in this scenario, and SMF will decide whether the QoS monitoring for packet delay can be triggered based on implementation.

If the SMF receives "NOT_SUPPORTED" value, it means the QoS monitoring for packet delay is not supported by NG-RAN, and SMF will not trigger the QoS monitoring.

No indication means the support of QoS monitoring for packet delay is not changed since last update to the SMF.

SA2 Question 2: According to SA2’s understanding of the CT4 CR, it seems that there is an inconsistency between clause 6.1.6.2.9 and 6.1.6.2.11 in the CT4 agreed CR.

CT4 answer: In PDU session establishment (PduSessionCreateData), if the new attribute qosMonitoringPdSupported is not included, the SMF knows the capability of the NG-RAN is "UNKNOWN", there is no need to indicate it to the SMF explicitly.

In HsmfUpdateData, as replied in Question 1, no indication means the support of QoS monitoring for packet delay is not changed since last update, so the "UNKNOWN" shall be used to indicate the capability of the new NG-RAN is "UNKNOWN".

2. Actions:

To SA WG2 group.

ACTION: 	CT4 kindly asks SA2 to take the above information into account.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm