SA WG2 Meeting #167	S2-2501367

17 - 21 February, 2025, Athens, Greece

SA WG2 Meeting ##166-Ad-Hoc-e	S2-2500044

20 - 24 January, 2025, Electronic meeting

3GPP TSG-RAN WG2 Meeting #128                                                                   R2-2411041

Orlando, USA, Nov 18th – 22nd, 2024

Title:	LS on emergency services and eDRX

Response to:

Release:	Rel-17

Work Item:	NR_newRAT-Core, NR_redcap-Core

Source:	RAN2

To:	SA2, CT1

Cc:	RAN3

Contact person:	Jing Liu

liu.jing30@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

In RAN2#128, companies discussed a scenario where a UE is configured with a RAN eDRX cycle in RRC_INACTIVE state when the UE has a PDU session associated with emergency services. RAN2 notice the below description in TS 23.501 which states that an Idle eDRX cycle should not be used by the UE when it has a PDU session associated with emergency services. However, it is unclear whether it is possible for such UE to be released to RRC_INACTIVE state with a RAN eDRX cycle and if it is so how UE behavior may be.

#### 5.31.7.2 Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE ##### 5.31.7.2.1 Overview ##### // skipped irrelevant part When the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception as defined in clause 5.4.5 and shall not use the extended idle mode DRX. Extended idle mode DRX parameters may be negotiated while the UE has PDU Session associated with emergency services. When the PDU Session associated with emergency services is released, the UE and AMF shall reuse the negotiated extended idle mode DRX parameters in the last Registration Update procedure. // skipped irrelevant part

Based on current specification, a RAN eDRX cycle can be configured by a RAN node when releasing the UE to RRC_INACTIVE state only if the UE is configured with an Idle eDRX cycle by CN.

RAN2 assumes that RAN node should be aware whether a UE has a PDU session associated with emergency services so that it can avoid releasing such UE to RRC_INACTIVE state with a RAN eDRX cycle, or even if it is so, there should be means for the RAN node and the UE to not use the configured RAN eDRX cycle. Regarding how RAN node can be aware of such PDU session, RAN2 understands that CN can already indicate this (e.g. via ARP value) but there is no requirement for CN to do this, so, there is case that RAN node may not be aware of this.

RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed?

# 2	Actions

To SA2, CT1 group

ACTION: 	RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed?

# 3	Dates of next TSG-RAN WG2 meetings

TSG-RAN2 Meeting #129	17 - 21 February 2025	Athens, GR

TSG-RAN2 Meeting #129bis             07 - 11 April 2025	                             China, CN