SA WG2 Meeting #167	S2-2501416

Athens, Greece, 17 – 21 February 2025

Source:	Oracle, Verizon UK Ltd

Title:	WID: TEI20_DAMUP – Deferred AM and UE Policies in the 5GC

Document for:	Information

Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: Deferred AM and UE Policies in the 5GC

## Acronym: TEI20_DAMUP

## Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: Rel-20.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification:

## 3	Justification

The access and mobility related policy control encompasses the management of service area restrictions, the management of the RFSP Index, the management of the UE-AMBR, the management of the UE Slice-MBR, the slice replacement management and the management of SMF selection.

UE policy encompasses the management of ANDSP, URSP, V2XP, ProSeP, RSLPP, A2XP (see 23.503 clause 6.1.2.2). In the context of this document, UE policies refer to URSP only.

The following two policy mechanisms exist(ed) in EPC.

- Policy rule activation/deactivation in a deferred mode – The ability of the policy server to ask the enforcement to enforce policy decisions (session rules or/and pcc rules) in a future time and/or stop enforcing policy decisions at a future time; and the corresponding ability of the enforcement to follow through with that.

- Revalidation time – The ability of the policy server to instruct the enforcement to interact with it (via the generic mechanism of Policy Control Request triggers) within Revalidation time limit set by the policy server. Once this future interaction takes place, the policy server can evaluate policies and install/modify/delete PCC rules.

These two policy mechanisms were carried over as is to 5GC Session management related policy control (Npcf_SMPolicyControl).

Some scenarios however require the use of these mechanisms also for AM or/and UE policy (Npcf_AMPolicyControl or/and Npcf_UEPolicyControl). UE policy (specifically URSP) already has a “built-in” mechanism of activation/deactivation via the time window of the RSD.

Consequently, we need to:

- Add the mechanism of policy rule activation/deactivation in a deferred mode as well as the mechanism of Revalidation time to AM policy.

- Add the mechanism of Revalidation time to UE policy.

The following scenarios have been identified:

Scenario 1: Promotions – An operator can offer a promotion on higher UE Slice-MBR, UE AMBR, different service area restrictions, different RFSP, URSP rule with a dedicated slice/DNN combination etc. Promotions are usually associated with time interval. For example, a promotion may be active for 24 hours or a promotion may be active for the rest of the billing cycle etc. These promotions use rule activation time (i.e. rules becomes active upon start of a promotion), rule deactivation time (i.e. rules stops being active upon end of a promotion) and in some cases revalidation time (i.e. AMF should ask for AM/UE policies upon start or/and end of a promotion). Revalidation time would be used in case PCF needs to re-evaluate policies or current conditions to determine correct settings.

NOTE:	The number of Revalidation timers per AMF would be much smaller than number of Revalidation timers per PCF (one PCF normally serves multiple AMFs), hence trying to keep revalidation state(s) on PCF may not scale up well.

Scenario 2: Time of Day policy actions. For example, during busy hours an operator may want to lower UE Slice-MBR or change the RFSP, while during off peak hours the operator may want a higher UE Slice-MBR/RFSP.

Scenario 3: Planned events - Same as Scenario 2, but wrt/ planned events such as a game or a concert. This may also involve pushing event specific AM policies, e.g. instructing the UE about slice related restrictions, allowed TAIs etc.

## 4	Objective

The objective is to enhance the applicability of ‘PCC rule activation/deactivation in a deferred mode’ mechanism to AM policy and applicability of ‘Revalidation time’ mechanism to AM and/or UE policy.

The scope of this Work Item in the context of both AM and UE policies applies to either non-roaming or roaming. Only impacted NFs are PCF for a UE and AMF.

The expected time for completing this work is 1 TUs. 0.5 time slots in one SA2 meeting to define the functional description and 0.5 time slots in next SA2 meeting to define the procedures and services.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.503 | Define the PCF/SMF impacts. 6.1.2.X(new) - Update Access and mobility policy control such that they will include activation/deactivation of policies in a deferred mode. 6.1.2.Y(new) - Update Access and mobility and UE policy control such that they will include Revalidation time of policies. 6.5 - Add attributes related to activation/deactivation/revalidation of policies in a deferred mode to Access and mobility related policy information table. 6.6.7 - Add attributes related to revalidation of policies in a deferred mode to UE Policy Association supplementary information table. 6.1.2.5 – Add Revalidation Timeout to Policy Control Request Triggers relevant for AMF and both 3GPP and Non 3GPP access type (This PCRT table includes both AM and UE policy PCRTs). | TSG#110 (Dec 2025) |  |

## 6	Work item Rapporteur(s)

Baniel, Uri	uri dot baniel at oracle dot com

## 7	Work item leadership

SA2.

## 8	Aspects that involve other WGs

None.

## 9	Supporting Individual Members

Supporting IM name

Oracle

Verizon UK Ltd

China Telecom

SK Telecom