3GPP TSG-SA2 Meeting #167	 S2-2501420

Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6066 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the network slicing clause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The text in the third paragraph of clause 5.49.1.4 currently is ambiguous as to whether the UE served by the MWAB-gNB or the MWAB-UE PDU session establishment completes |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified the text refers to a UE served by the MWAB-gNB, and the wording of paragraph 2 is improved |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear text, possible misbehaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

#### 5.49.1.4	Support of network slicing for UEs served by a MWAB

A MWAB-gNB can be configured to associate S-NSSAI(s) in the MWAB-Broadcasted PLMN/SNPN to the traffic descriptor for selection of the BH PDU Sessions parameters e.g. use dedicated S-NSSAI in BH PLMN. If no such association exists for a specific S-NSSAIs of the MWAB Broadcasted PLMN/SNPN, the MWAB-gNB associates this S-NSSAI to default traffic descriptors for the selection of BH PDU Session(s) for the related N2 and/or N3 backhauling. The MWAB-gNB requests the BH PDU sessions as specified in clause 5.49.1.2.

When the MWAB-gNB obtains, for a UE it serves, the S-NSSAI of a PDU Sessions from the UE’s PDU session context during PDU session establishment, it checks whether a configured association exists for the S-NSSAI of the PDU session of the UE and, if so, it uses the related BH PDU session for the UE's PDU Session.

If a new BH PDU Session needs to be established for supporting a UE's traffic for a S-NSSAI in the MWAB-Broadcasted PLMN/SNPN, after the completion of the PDU session establishment procedure for the UE served by the MWAB-gNB, the MWAB-UE may trigger a PDU session resource modification procedure to shift the N3 traffic for the UE to the new BH PDU sessions.

NOTE:	Due to the limitation of the maximum number of PDU sessions supported by a UE, it is expected that there can only be a small number of dedicated BH PDU sessions for specific S-NSSAI(s) in the MWAB-broadcasted PLMN.

End of changes