3GPP TSG-SA2 Meeting #167	 S2-2501421

Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6067 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of QoS Clause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is a misleading text implying N3 can be not related to UE PDU sessions. This is not correct as all N3 traffic is related to UE PDU sessions. In addition, the EN that refers to Xn being FFS should be removed as additional considerations on Xn will depend on developments in RAN WGs and we can align to their conclusions if needed using CAT-F CRs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The reference to N3 is removed in last sentence of second paragraph of clause 5.49.1.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification, EN remains |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

#### 5.49.1.3	Support of QoS for UEs served by a MWAB

The MWAB-UE considers the traffic from the MWAB-gNB as application layer traffic and applies the related QoS handling as specified in clause 5.7 for BH PDU Session(s). The MWAB-UE may establish and modify the BH PDU Sessions based on necessary information provided by MWAB-gNB (e.g. according to the QoS requirements identified by the MWAB-gNB based on the QoS information of the PDU Session(s) in the UE contexts of the MWAB-gNB and/or based on OAM configuration) via implementation-based internal communication between MWAB-gNB and MWAB-UE.

Based on configuration and traffic mapping specified in TS 38.414 [195] and clause 5.7.1.5, the MWAB-gNB (in the Uplink direction) and the UPFs (in the Downlink direction) in the MWAB Broadcasted PLMN/SNPN set proper marking information (DSCP) for the N3 GTP-U traffic for the UE(s) served by the MWAB-gNB. The MWAB gNB additionally marks the DSCP value based on configuration for N2, Xn and OAM traffic it sources, i.e. not related to PDU Sessions(s) of the UE.

The MWAB-gNB provides the information to the MWAB-UE for the mapping of DSCP values used for N3 traffic in the MWAB Broadcasted PLMN/SNPN to QoS flows to be used in the BH PLMN/SNPN (including the required Packet Filter Set for filtering of packets) and the corresponding QoS requirements (e.g. QoS Flow level parameters including e.g. 5QI). The MWAB-UE then initiates PDU session establishment or modification procedure to establish or update the BH PDU session(s) for N3 traffic delivery based on the received information.

Based on the information provided by the MWAB-gNB and received during the PDU session establishment/modification, the MWAB-UE processes the UL N3 traffic as described in clause 5.7.1.7. The BH UPF processes the DL N3 traffic as described in clause 5.7.1.6.

The MWAB-UE may indicate to the MWAB-gNB the current BH PDU Session's QoS Flow level QoS parameters, e.g. for admission control of the QoS-Flows of the UEs served by the MWAB.

End of changes