SA WG2 Meeting #167	S2-2501444

# 17 - 21 February 2025, Athens, Greece

Source:	ZTE

Title:	Potential working tasks of AIML in R20 5G-A

Document for:	agreement

Agenda Item:	30.8

Abstract of the contribution: This discussion paper presents some potential working tasks for 5G-A AIML.

Discussion

1. Extension of ML Model Evaluation Metrics in the Core Network

Currently, the standardized ML Model metric only contains accuracy. In the current TS 23.288 specification, the definition of accuracy is only applicable to assess the performance of classification models. However, in some scenarios, the output consists of continuous variables rather than discrete ones. For instance, in scenarios such as NF (Network Function) load analysis, NWDAF (Network Data Analytics Function) may predict continuous variable like the resource utilization (e.g., CPU and disk) in an NF during a future time period, as well as the NF's load.

Figure 1. Comparison of Classification and Continuous Prediction

As shown in Figure 1, continuous prediction and classification prediction are fundamentally different. The accuracy metric currently defined in TS 23.288 is not suitable for evaluating regression models.

On the other hand, for the evaluation of classification models, accuracy alone is not comprehensive. To assess the model's sensitivity, specificity, and performance when dealing with imbalanced datasets, it may be worth considering the introduction of new metrics. In addition to evaluating the quality of the model's output, other aspects of the model's performance are also important: for instance, inference speed (especially in latency-sensitive scenarios, such as user experience analysis), energy consumption during training/inference, and disk overhead during training. Whether and how to evaluate the model's performance in these areas within the core network is also worth exploring.

2. Defining Model Life cycle Capabilities in the Core Network

In the current 3GPP TS 23.288 specification, the model life cycle defined for the core network is not clear. The current specification only defines the procedures related to ML model training and ML model accuracy monitoring, but stages such as training and testing data pre-processing(e.g. data set cleaning, solving unbalance problem, feature engineering), model validation, and model testing are not explicitly defined.  Figure 2 shows a schematic of the ML model life cycle.

Figure 2. Schematic of the Model Life cycle

On the other hand, the computational power of each NWDAF (Network Data Analytics Function) varies at different times: some NWDAFs have more computational power during some specific periods, while others have less. As a result, the types of ML tasks they can perform may vary over time. NWDAFs with more computational power can handle resource-intensive tasks like ML model training, while those with less computational power can handle tasks such as model testing or data preprocessing.

Additionally, the service areas of each NWDAF are different, meaning the model testing data they collect are also different. Therefore, testing the same ML model across multiple NWDAFs with different serving area can evaluate the generalization performance of ML Model before it deployed to some specific area.

3. Enhancement of NWDAF and ADRF Discovery

The processes of model training, validation, testing, and data pre-processing consume significant computational resources. Additionally, the computational power required to generate different analysis based on different requests of different consumer network function requests can vary. NWDAFs (Network Data Analytics Functions) have different amounts of computational power available at different times. However, in the existing standards, the computational power information of NWDAF is not mentioned.

Therefore, it is worth discussing whether computational power-related parameters should be added to the  NF profile of NWDAF when registered into the NRF (Network Repository Function). This could allow the discovery process to match the appropriate NWDAF based on its computational power.

Similarly, the discovery process for ADRF (Analytics Data Repository Function) does not include parameters such as memory information. Since ADRF is a core network element responsible for storing data and models, it may face memory constraints. As a result, it is also worth discussing whether and how ADRF's available memory information should be registered in the NRF. This can be used to match ADRFs with sufficient memory during the discovery process.

- Reinforcement Learning&Online learning in 5GC

- The general reinforcement learning problem can be represented in the following way: The Agent receives the State of the Environment (through the Interpreter) at a certain time (s). Then the Agent selects on action (a) and applies it in the Environment. When this action is applied, the Environment provides a Reward (r) and changes to a new State (s'), the Reward and State provided finally by the Interpreter [1].

Figure 3. Schematic of Reinforcement Learning Algorithm [1]

Reinforcement Learning could help to enhance the outcome quality of the following scenarios in TS 23.288:

- The Analytics accuracy monitoring/ML Model performance monitoring (only between NWDAFs)

- QoS parameter recommendation/assistance by NWDAF (possibly between PCF and NWDAF)

Below is a figure for Analytics accuracy monitoring inside among NWDAF using Reinforcement Learning:

Figure 3. Reinforcement Learning for analytics quality monitoring and ML Model monitoring

- Online learning is a machine learning paradigm where a model is trained incrementally by processing data sequentially, updating itself as each new data point becomes available, without requiring access to the entire dataset at once. In the 5GC, the inference output(analytics output) and ground truth data can be used to update/train the ML Model, to improve the ML Model real-time generalization performance.

- NOTE: Reinforcement learning may be a part of online learning.

- Enhancement of ML Model updating procedure

- In one scenario, an NF consumer (such as a NWDAF containing AnLF or LMF) obtains a ML model from a NWDAF that contains MTLF. However, during inference, the NF consumer notices that the model’s inference output quality has degraded (corresponding to the accuracy monitoring procedure in TS 23.288). Consequently, the NF consumer initiates a model update request to the NWDAF with MTLF. After the NWDAF trains a better ML model, it can only transmit the entire new model file to the NF consumer via a URL or Fully Qualified Domain Name (FQDN).

- Transferring the large model file between the NWDAF and the NF consumer, even if done via a link, imposes a significant burden on the NWDAF containing AnLF due to the "download + delete + store" operations required to handle the model.

- In the scenario described above, a more efficient approach might be to update only the parameters that have changed after re-training, or to use a small model fusion update method.

- 6. ML Model profile

- Model consumers can first request ML Model profiles, select one or more models based on ML Model profiles, and finally request the model source files. This effectively resolves issues of incompatibility and poor inference performance that arise from directly requesting black-box model files.

- 7. Enable RAG technology in 5GC (For R20 5G-A or beyond)

- Retrieval-Augmented Generation (RAG) is a hybrid model architecture that combines two powerful AI techniques: retrieval and generation. It enhances Large language models by incorporating relevant external knowledge during the generation process, making it particularly useful for tasks requiring up-to-date or domain-specific information.

- The RAG technology can largely increase the quality analytics generated by NWDAF by interacting with some domain-specific information in the knowledge base.

- Figure 4. Knowledge base construction for RAG

- Figure 5. Using RAG to generate analytics

- 8. AIML tasks coordination (For R20 5G-A or beyond)

- In the current core network, NWDAF handles a large number of AI-related tasks (such as model training, analytics generation, etc.). Different tasks consume varying amounts of computing power and have different priorities. However, the current TS 23.288 specification lacks measures for coordinating AI/ML tasks, which may lead to issues in 5G networks, such as some NWDAF being overloaded with  insufficient computing power, while others are underloaded but have excess computing resources.

- Working task proposals

- Whether and how to extend ML Model metrics to support comprehensive evaluation of ML Models

- Whether and how to define ML Model life cycles and related procedures

- Whether and how to extend the NWDAF and ADRF profile to be registered to NRF considering the computational power of NWDAF and memory capacity of ADRF

- Whether and how to enhance the ML Model update procedures

- Whether and how to enable RAG into 5GC

- Whether and how to coordinate AI/ML tasks in 5GC

- Whether and how to define ML Model profile and conduct ML Model selection based on ML Model profile

- References

- [1] Ericsson, “New solution PCF as RL Agent and NWDAF as Interpreter S2-2404476”, SA2 #162, available at: https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_162_Changsha_2024-04/Docs/S2-2404476.zip