3GPP TSG-SA2 Meeting #167	S2-2501461

Athens, Greece, 2025-02-17 – 2025-02-21

Source:	Qualcomm (Moderator)

Title:	Moderator report on: Moderated discussion for Enhancements for ePDG based access to 5GC

Document for:	Decision

Agenda Item:	30.8

Work Item / Release:	5G-A/Rel-20

Abstract of the contribution: Moderator summary on Enhancements for ePDG based access to 5GC features NWM discussions held between January 10 – February 05, 2025.

# 1.	Introduction

This report summarizes the discussion/comments provided by 22 companies regarding enhancements proposal for ePDG based access to 5GC.  The NWM report in included in the zipped file of this report.

Summary of the comments on the WTs (simplified):

Originally proposed WTs:

WT-1: Support for IMS Voice over Wi-Fi connected to 5GC in standalone deployments and 3GPP-N3GPP interworking without relying on N3IWF. This includes:

- Architecture to support ePDG connected to AUSF, UDM and NRF, without intermediate 3GPP AAA Server

- Impact to procedures for Attach and PDN Connection Establishment via ePDG when interfacing with AUSF and UDM

- Investigate solutions for 3GPP-N3GPP interworking without the use of N3IWF, aiming to minimize UE impacts and NW impacts in control plane management functions

- Study should focus on use of Voice over Wi-Fi as the most prominent use for MNOs in short term.

- Other use cases and scenarios can be considered with lower priority, e.g., allow use of ATSSS functionality “on top”Support for standalone deployment i.e., in absence of 3GPP link

WT-2: Investigate whether the architecture enhancements to ePDG for Voice over Wi-Fi in 5GC, as defined in WT1, can also be used to enable evolved Residential Gateway (eRG) access to 5GC via ePDG without impact on wireline network. This includes the scenario where the eRG is not equipped with USIM (as per the approved 22.261 CR0805; S1-242538).

No new WTs were proposed.

Moderator views on how to move forward:

WT#1:

- 10 companies supported, 3 companies were neutral and indicated they can support it, and 8 companies indicated they do not support it.

- For WT#1, concerns were expressed about the need for a number of TUs over what is expected to be possible, or the need to consider this as part of Rel. 20 6G work, and several evaluative comments were provided together with technical answers to such comments.

- Explicit comments were raised about the need to incorporate 5G authentication as part of WT#1.

WT#2:

- 2 companies indicated support, while 1 company was neutral and 15 companies do not support.

- The moderator concludes that there is no support for WT#2.

# 2.	Proposal

Based on the analysis of the NWM responses, moderator proposes following way forward:

Proposal 1:

- The moderator suggests drafting a WT#1 description taking into account the technical comments regarding the content and leave evaluation comments for the SID evaluation phase.

- During the WT#1 discussion, we can evaluate if the scope allows for the work to be carried out in the number of TUs we expect can be allocated for the SID.

- WT#1 should include 5G authentication.

Proposal 2:

- The moderator proposes that we do not proceed with WT#2.