3GPP TSG-WG2 Meeting #167	S2-2501481

# 17th – 21st February, Athens Greece

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0669 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures to collect data from the UE and from NG-RAN, resolution of Editor´s Note. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Updates to the clause to collect data from NG-RAN to cover updates on user consent. Changes compared with rev.1. Two alternative options: - User consent for data collection for training covers the initial model training and the maintenance of the model. When the user consent is revoked, there may be internal actions at the LMF (or at NWDAF) to update the ML Model that are out of scope of 3GPP. - Alternatevly, it has also been discussed that there is a framework for user consent for data collection for ML Model training and inference in Annex V of 33.501 that is used by any consumer, including NWDAF or LMF. No extensions should be discussed for a specific consumer. So no extensions to the user consent in UDM are also a reasonable option as a way forward. This means that LMF collects user consent when doing training and nothing is said about user consent and performance monitoring per current specifications. Similar on the actions on user consent revoke. Corresponging EN are removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to the clause to collect data from NG-RAN to cover updates on user consent. Changes compared with rev.1 (if option 2 is preferred this CR needs to include third change only) - A NOTE is to explain that user consent is obatained when the LMF starts training the model and then the LMF uses the user consent during the maintenance of the ML Model. - A NOTE is added to indicate that when the user consent is revoked the LMF may take actions towards the trained model that are internal and out of the scope of 3GPP. - The Editor´s Note related to user consent checking with SA3 is removed as it appears in other clauses and is not anylonger related to the questions that motivated to send an LS to SA3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LMF-based AI/ML positioning is not fully specified in Rel19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** 1st change **************

### 6.22.3	Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements

The procedure for data collection from NG-RAN for LMF-based AI/ML positioning is used to e.g. train the AI/ML Model.

Figure 6.22.3-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements

1.	The LMF determines that data collection is required e.g. to train an AI/ML Model for UE positioning for a number of UEs or to monitor the AI/ML Model performance. The LMF may also initiate the data collection upon the request of an NWDAF containing MTLF as described in step 3 in figure 6.22.4-1.

2.	The LMF may know the SUPI of these UEs, e.g. when training a AI/ML model using a PRU, which are associated to this LMF that executes data collection, the LMF may optionally invoke an Nnrf_NFDiscovery Request service operation to an NRF in order to identify the LMF(s) which has PRU associated to a list of TAIs. The LMF may send an Nlmf_Location_MeasurementData Request service operation to one or more of the other PRU serving LMFs in order to get the data collection and the known PRU location in this case step 6 follows.

If the LMF does not know the SUPIs of those UEs, the LMF try to get the list of SUPIs from the AMF, before that the LMF finds the AMFs via NRF using Nnrf_NFDiscoveryRequest.

3.	For each AMF serving a specific area. the LMF subscribes to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

4.	The AMF send Namf_EventExposure_Subscriber_Response or Namf_EventExposure_Notify ("list of SUPIs in/out the area of interest").

For each SUPI provided by the AMF that is in the area of interest then steps 5 to step 6 are performed.

4.	The LMF checks whether the SUPI provided user consent for data collection for the purpose of training with UDM using Nudm_SDM_Get including data type "User consent".

NOTE 1:	The LMF obtains user consent for data collection for the purpose of model training, i.e. this cover both training and maintenance of the trained model, as such there is no need to obtain user consent again when the LMF checks the performance of the model.

5.	The LMF subscribes to UDM to notifications of changes on subscription data type "User consent" for this SUPI using Nudm_SDM_Subscribe. If user consent is provided, then step 7 follows, otherwise no data is collected for this SUPI, i.e. steps 7 and 8 are not performed.

6.	The LMF request input data for the target UE from NG-RAN.

7.	The LMF receives ground truth data from the UE.

8.	The LMF may determine that the UE is not longer in the area of interest, based on the AMF notification using Namf_EventExposure service, then the LMF performs step 10 and step 11, the LMF may unsubscribe to be notified on user consent updates if the UE is not in the area of interest any longer.

9. The UDM may notify the LMF on changes of user consent at any time after step 5. If user consent is no longer granted for a user for which data has been collected the LMF performs step 10 and step 11. The LMF may unsubscribe to be notified of user consent updates from UDM for each SUPI for which data consent has been revoked.

10.	The LMF request NG-RAN to stop reporting input data for the target UE.

11.	The LMF stop any retrieval of ground truth data from the UE.

The measurements and ground truth data are used for training. The UE location is derived from the measurements data by using LMF-based AI/ML Positioning. The derived UE location and ground truth data are used for performance monitoring.

The LMF may initiate data collection for multiple UEs simultaneously, as such step 8 and 9 may occur in parallel for a number of SUPIs as determined by the LMF.

NOTE 2:	When the user consent for model training is revoked, the LMF can take internal actions such as model re-training, these actions are out of the scope of this specification.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

************** End of changes **************