3GPP TSG-SA WG2#167	S2-2501497

Athens, Greece, 17-21 February 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	Discussion paper on OSAppID usage by AppToken use case

Document for:	Discussion/Approval

Agenda Item:	4.1

Work Item / Release:	TEI19/Rel. 19

Abstract of the contribution: the paper discusses various aspects related to the AppToken scenario that GSMA generated.

# 1.	Introduction

Arguments have been made in SA2#166AHE that SA2 should simply answer GSMA questions and not discuss the merits and applicability of the solution proposed by GSMA. While the solution remains under the purview of GSMA, if the solution impacts the 3GPP functionality of is based on incorrect assumptions with respect to how 3GPP mechanisms work, the it is in the scope of SA2 to provide appropriate questions and comments to GSMA.

Specifically, the LS from GSMA contains the following two points:

- Would 3GPP SA2 prefer an alternate mechanism to preserve end user privacy?

- Consider the architectural implications of the proposed mechanism and provide feedback.

These clearly indicate GSMA welcomes technical feedback from SA2 beyond the scope of the first question in the LS.

# 2.	Discussion

## 2.1 Influencing URSP rules

Regarding the first question in the LS, there is consensus that 3GPP specifications allow URSP population influenced by AF (3rd party app backend).

Proposal 1: indicate to GSMA that 3GPP specifications do allow URSP population influenced by AF (in this case 3rd party app backend).

## 2.2 User Privacy

GSMA states that:

“The App Token is a piece of information that the UE OS can obtain from specific applications and allows to uniquely identify an application across the complete ecosystem (application developer, MNO, subscriber UE) whilst protecting the end user privacy. This means that the UE OS does not need to use the information it has about the identity of an application when comparing against the OSAppID field from the URSP rules received from the MNO, but to use an anonymous identifier received from the MNO through TS.43 interface for that comparison.”

At high level, it is very unclear how the AppToken protects user privacy:

- The actual identity of the application requesting connectivity to the OS cannot be hidden from the OS, since it is the application itself that sends the AppToken to the TS.43 client running in the OS

- The actual identity of the application requesting connectivity to the OS cannot be hidden from the MNO, since if the application provider via its backend requests the MNO to create URSP rules with the “anonymized” AppID, then the MNO knows the application associated to the token.

It seems that if there is a privacy or security concern to be addressed, further study is required by SA3 to identify the threat and the validity of this or alternative solutions.

Proposal 2: indicate to GSMA it is not clear what aspects of end user privacy the solution is addressing.

## 2.3 Signalling Impact Versus MNO Awareness of the app

Analyzing the message flow from the material sent from GSMA, it seems that when the 3rd party app needs connectivity, it triggers the OS to validate the AppToken with the OES, which means the OES (i.e. the MNO) is aware the application is starting and thus it is not protecting the user privacy wrt which application is being used. Moreover, this requires additional signalling everytime the 3rd party app requires connectivity. Moreover, it seems the 3rd party app needs to have the AppToken verified before it can connect to the network, which may lead to additional delay with impact on the user experience.

Proposal 3: ask GSMA to clarify exactly what privacy the mechanism is intended to protect considering the additional signalling being introduced.

## 2.4 Impact on 3GPP NFs

The solution may have impact on the PCF: if the URSP rules are configured on a per subscription basis, PCF would need to know whether the UE is configured with the anonymized App_Token or not, and thus an interaction between the entitlement server and PCF would be required, but this aspect is not addressed at all in the proposal.

Proposal 4: indicate to GSMA that the solution may have impact on PCF, and ask whether they have considered the need for interaction between the entitlement server and the PCF.

## 2.5 Time Validity of AppToken

The proposal is unclear as to whether the AppToken has a limited time validity or it has no expiration. This would need to be clarified to understand the impact on the creation and handling of URSP rules, and it has implications on the UE side in terms of how many timers would need to be run (one per app?). Also, can an App_token be invalidated after being issued? If not, why?

Proposal 5: ask GSMA clarification on the time validity of an AppToken and indicate that time validity has implications on implementation and handling of URSP rules in the UE, and on the contrary lack of a time validity may require a revocation mechanism.

## 2.6 Impact on applications

It is evident that applications need to be modified in order to enable the application to pass the AppToken to the OS. Is this the intention and the assumption?

Proposal 6: ask GSMA why they do consider feasible having to rewrite application software to enable the AppToken.

# 3.	Conclusions

Several questions need to be raised with GSMA and SA3 wrt the proposed solution, and the questions and comments proposed above should be added to an LS response.