3GPP TSG-SA2 Meeting #167	S2-2501500

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0386 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification about MBS service area encoding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5MBS |  |  |  |  |  | Date: |  |  | 2025-01-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Stage 3 TS 29.571 contains. #### 5.9.4.4 Type: MbsServiceArea Table 5.9.4.4-1: Definition of type MbsServiceArea Attribute name; Data type; P; Cardinality; Description. ncgiList; array(NcgiTai); C; 1..N; List of NR cell ids with their pertaining TAIs (NOTE).. taiList; array(Tai); C; 1..N; List of tracking area Ids (NOTE).. NOTE: The MBS Service Area consists of the union of the cells in the tracking areas listed in the taiList IE and the cells listed in the ncgiList IE. At least one of the ncgiList IE and taiList IE shall be present.. #### 5.9.4.5 Type: NcgiTai Table 5.9.4.5-1: Definition of type NcgiTai Attribute name; Data type; P; Cardinality; Description. tai; Tai; M; 1; TAI of the cells in cellList (NOTE). cellList; array(Ncgi); M; 1..N; List of NR cell ids. NOTE: The NcgiTai consists of the list of cells listed in the cellList IE. These cells pertain to the TAI indicated in the tai IE. The TAI may be used e.g. to discover and select an AMF that serves NG-RAN nodes supporting the corresponding cells.. However, stage 2 does not reflect that Cell IDs in the MBS service area are provided together with TAIs to enable the selection of appropriate gNBs and AMFs |
| Summary of change: |  | Stage 3 Alignment: Cell IDs in the MBS service area are associated with a TAI to enable routing related requests to appropriate gNBs and AMFs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Routing based on TAIs is not clearly described, which may confuse implementors (in particular due to related wording for NR_NTN_Ph3-Core beeing suggested for Rel-19) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change: Local MBS service and Location dependent MBS service

### 6.2.1	General

A Local MBS service is an MBS service provided in one MBS service area. A location dependent MBS service is an MBS service provided in several MBS service area(s). An MBS service area is identified by a cell list or a tracking area list. The MBS service area could be geographical area information or civic address information, and NEF/MBSF translates the location information to Cell ID list or TAI list as MBS service area, see clause 7.1.1.2.,

NOTE:	TAIs can be provided separate from cell IDs and then denote that the entire related tracking area is part of the MBS service area, or in combination with Cell IDs and then do not denote the MBS service area but are only used to aid routing related requests to AMFs and RAN nodes handling the related cells.

The MBS service area may be updated by the AF for both multicast MBS sessions and broadcast MBS sessions as specified in clause 7.1.1.6.

For more details, refer to clause 7.2.4 for multicast MBS session and refer to clause 7.3.4 for broadcast MBS session.

End of changes