3GPP TSG-SA2 Meeting #167	S2-2501520

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025										(was S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3883 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mode transition between S&F and Normal Mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The exception sheet in SP-241918 for 5GSAT_Ph3-ARC includes for S&F operation the following: - (2b) – Functional specification of expected UE and network behaviour when transitioning between operation in S&F mode and operation in normal mode Changes are thus needed to define the transitioning between S&F and normal mode in both a UE and network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of UE and network behaviour for transition between S&F mode and normal mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE or network may not be able to handle the transition from normal mode to S&F mode or vice-versa. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.13.9.x	Transition between S&F mode and normal mode

The transition between S&F mode to normal mode or vice versa shall follow the same behaviour as described in clause 4.13.9.1.

- If the UE is registered with a satellite operating in S&F mode, and the satellite changes from S&F to normal mode and if the UE sends TAU or Service Request, and if the ground MME doesn’t have the UE context, then MME shall send TAU or Service Reject with cause implicit detach, so that UE can re-attach in normal mode.

- If the UE is registered with a satellite operating in normal mode and the satellite changes mode from normal to S&F, and if the UE sends TAU or Service Request, and if the MME on-board satellite doesn’t have the UE context, then MME shall send TAU or Service Reject with cause implicit detach or indicating the NAS congestion with optional back-off timer based on MME deployment, so that UE can re-attach in S&F mode, if the UE is capable of S&F.

# * * * * End of changes * * * *