3GPP TSG-WG SA2 Meeting #167	S2-2501157

# 17-21 February, 2025, Athens, Greece		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6081 | rev |  | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Energy saving indicator |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Energy Saving Indicator is an indicator that is send to the gNB related to the specific UE and not as a generic indicator that applies to all UEs. The current name gives the impression that is generic, from the point of view of people participating to the discussion the meaning is crystal clear, but when we move out of Energy-people the risk of misunderstanding Is not negligible. Therefore we propose the change of name to “UE Energy Saving Indicator” to clear indicate that is an indicator per UE. The current specification, there is no principle described for the handling of UEs with this indicator, in particular, when and how often RAN energy saving shall apply. Without such principle, the AMF forwards the Energy Saving indicator for all these UEs to NG-RAN and NG-RAN may often apply energy saving actions for these UEs. This can easily lead to situations where some UEs are consistently impacted and thus unequally treated compared to other UEs. The issue of an unconditional and permanent distribution in the network and especially towards the RAN nodes. It is quite likely that there are end users which are often connected to RAN nodes in energy saving mode and thus will experience a certain degradation of service while other end users may rarely be connected to such RAN nodes. In consequence, some end users are quite unhappy with their contract (containing the Energy Saving indicator) and will change the contract or even the operator. And the RAN node will then have less UEs available for reducing the energy consumption when necessary. So, instead of an unconditional and permanent forwarding, the PCF take QoS decision involving both the PCC rules and whether to send the Energy Saving Indication. In fact the PCF may decide to send both or to send only the indicator or change the PCC rules for a given UE in order for example to avoid or to allow to apply double traffic restrictions the first or to control the traffic restriction to a given level setting a suitable QoS that will be not degraded too much by action taken by gNB. The PCF performs the control sending an indication to the AMF via AM Policy control which enable the AMF to send the Energy Saving indicator via N2 to the gNB where the UE is connected to or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are proposed: - change of the name to UE Energy Saving Indicator, - added the specification that the AMF provides the indication to enabling/disabling the sending of the UE Energy Saving Indicator from AMF to gNB is provided by PCF. T |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AMF send unconditional the Energy Saving Indicator which can cause an unpredictable result for the services provided to the UE cuasing a degradation of User experience |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.5, 5.51.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 5.51.5	Subscription information aspects

The Subscription data for a UE may include an UE Energy Saving Indicator  indicating that the UE is subject to network operation aiming to save energy. The UE Energy Saving Indicator is stored in UDM. The AMF retrives the UE Energy Saving Indicator in the UE Subscription data. The AMF forwards the UE Energy Saving Indicator to the PCF. The AMF forwards the UE Energy Saving Indicator to the NG-RAN only if the AMF receives from the PCF the indication to send such indicator as described in TS 23.503 clause 6.1.2.1.1

Editor's note:	Whether the indicator is provided to the NG-RAN and how it is used in the NG-RAN is FFS.

#### 5.51.6	Policy control aspects

During AM policy association establishment procedure, the AMF sends to the PCF the UE Energy Saving indicator as described in clause 5.51.5. Based on the UE Energy Saving indicator the PCF may make policy decisions reusing the existing parameters based on the operator policy. The PCF send in AM policy to AMF the indication whether the UE Energy Saving Indicator shall be sent by AMF to NG-RAN as described in TS 23.503 clause 6.1.2.1.1.

Editor's note:	A notification for the energy related information, if any, should not be provided from the UDM/UDR. Whether the notification for the energy related information from the EIF is provided to the PCF and what are the notification details, are FFS.

End of changes