3GPP TSG-SA WG2 Meeting #167	S2-2501561

17-21 February, 2025, Athens, Greece

Agenda item:	19.4.2

Source:	Huawei

Title:	Discussion on event reporting

WI/Release:	EnergySys/Rel-19

Document for:	Discussion and Decision

# 1	Discussion

At SA2#166AH meeting we had 2 proposal regarding the event reporting:

- to extend the usage of the User Data Usage Measures IE (S2-2500547)

- to define a new event  ID ((S2-2500812)

The discussion took place via email, but in this discussion paper we would resume the 2 approaches.

The proposal is to use the User Data Usage Measures for use plane from UPF via SBI interface to  SMF/EIF. This message has been defined in TS 23.502 in clause 4.15.4.5 with the scope to provide information of user data usage of the User PDU Session.  This event has been defined in the context of retrieving information from  NWDAF/AF/NEF. AT the subscription phase the consumer send the information from which the information is requested and the SMF select the correct UPF. The current specification allows to inform the consumer via SMF or directly from the selected UPF.

The current information returned is defined in TS 29.564 clause 5.2.1.3.3.

Consideration #1 It shall be noted that this event provides statistics related to user data usage of a PDU session of a specific UE or a group of UEs or any UE:

The measurement results provided are:

-	Volume Measurement: measurements of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall) determined for the requested Granularity of Measurements.

-	Throughput Measurement: measurements of data throughput (UL and DL) determined for the requested Granularity of Measurements.

-	Application related Information: URLs and/or Domain information (Domain name and protocol) detected in the target traffic identified by the information included in the subscription request, e.g. an application id.

When the granularity of the measurement is per data flow, the notification shall include the packet filter set and the Applications Identifier if available.

For energy purpose the information to be retrieved by  the EIF in order to be able to perform the calculation are per TS 23.501 per clause 5.51.2.2

| > UL/DL Data Volume of UPF | The UL/DL Data Volume of a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a QoS flow (UE ID, S-NSSAI, DNN, IP 5-Tuple). |
| --- | --- |
| > (I-)UPF ID(s) | Identifier of any (I-)UPF(s) associated to a reported data volume used by a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a QoS flow (UE ID, S-NSSAI, DNN, IP 5-Tuple). |
| > gNB serving the UE | The gNB which serving the UE. |

Consideration #2: The Data volume information can be mapped to the Volume measurement within the User Data Usage Measures IE.

Consideration #3: The serving gNB ID is not known to the UPF which just gets the N3 Info from the SMF, therefore only the SMF can provide it to the consumer. Hence the Nupf_EventExposure Service from the UPF can not be used, since the UPF  can not includes the gNB ID information.

User Data Usage Measures (see clause 4.15.4.5): SMF conveys the subscription to UPF on behalf of the consumer. Consumer receives the events directly from UPF. For certain UE(s), the SMF conveys the subscription to I-SMF on behalf of the consumer, and the I-SMF conveys the subscription to UPF on behalf of SMF. Consumer receives the events directly from UPF; and

The service which includes the User Data Usage Measures is the Nupf_EventExposure Service from the UPF, which can not be used since it can not include the gNB ID information.

Consideration #4: The gNB ID shall be reported by SMF , i.e, within the  Nsmf_EventExposure_Notify.

Consideration #5: The Nsmf_EventExposure_Notify service does not include the User Data Usage Measures IE.

The extension of the User Data Usage Measures impacts also the other consumer which are not interested to such information.

Consideration #6: The Nsmf_EventExposure_Notify service needs to be extended with a IE for providing the gNB ID information since the other information are already included in IE of the notify message.

As last considerations the  UPF needs to be instructed to perform the measurements and reporting for different granularity, Furthermore it needs to counts the traffic for the overall UPF and for each N3 interfaces, i.e. towards the specific gNB and store toward which gNb that traffic has been consumed. Current N4 Instruct for URR can address the needs, while the reporting based on User Data Usage Measures does not address such requirement/ Therefore the combination of SMF reporting plus UPF reporting is preferable.

It can be also considered that within the measurement time the UPF may change, this is an additional motivation for preferring the SMF reporting.

Consideration #7: The UPF may change during the measurement time, therefore the SMF reporting is preferable to manage the change of SBI interface which needs to be used to provide the info to EIF.

The I-UPF is also an additional case where the SMF reporting is preferable in respect the direct reporting via I-UPF which add complexity to the SBI reporting to EIF.

Consideration #7: The presence of I-UPF (including addition and removal)  makes  the SMF reporting preferable to the SBI interface  from the I-UPF to the EIF.

# 2	Conclusions

It is proposed that the Proposal here below are agreed.

Proposal#1: The Nupf_EventExposure Service from the UPF can not be used, since the UPF does not know the gNB ID information

Proposal#2: The SMF reports the requested information to the consumer within the Nsmf_EventExposure_Notify message

Proposal#3: The gNB ID requires to extend the Event Notification (i.e. TS 23.503 clause and TS 29.508 clause 5.6.2.5)