3GPP TSG-SA WG2#167	S2-2501568

Athens, Greece, 17-21 February 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	Emergency PDU session and “delayed” paging

Document for:	Discussion/Approval

Agenda Item:	19.50

Work Item / Release:	TEI19/Rel-19

Abstract of the contribution: Analyses the interaction of emergency call back and delayed paging features.

# 1.	Discussion

SA2 in recent discussions to draft alignment CRs for the “LP-WUS” feature in SA2#166 captured the following Editor’s Note in TS 23.501:

Editor's note:	The use of LP-WUSPS for emergency sessions and emergency call back is FFS.

In SA2#166AHE SA2 received LS R2-2411041/S2-2500044 from RAN2 “LS on emergency services and eDRX” that is related also to how to handle emergency call back, in this case for UE in RRC_INACTIVE state with eDRX.

This paper analyzes the optimal approach for handling emergency callbacks for UEs configured with features that may delay received paging.

# 2.	Background

# First some background for emergency callback. Emergency call back as defined in TS 23.167 and based on service requirements defined in TS 22.101 is treated as “normal call” based on “local regulation”.

Extract from TS 23.167 cl. 4.1:

12.	The architecture shall enable emergency centres and PSAPs to request a PSAP call back to a UE with which the Emergency centres or PSAPs had an emergency session. The serving network of the UE shall use the appropriate call termination procedures e.g. IMS if the UE is available for voice over PS, or ICS if the user is available over CS. PSAP call back is subject to local regulation.

NOTE 3:	PSAP call back sessions are treated as normal calls.

NOTE 4:	Subject to local regulation, any supported media can be used during a call back attempt from a PSAP.

Extract from TS 22.101 cl. 10.1.3:

### 10.1.3	Call-Back Requirements

Subject to local/regional regulations the network shall support a call-back from a PSAP.

It shall be possible to supply the user's Directory Number/MSISDN/SIP URI as the CLI to the PSAP to facilitate call-back. The CLI used on call-back shall allow the PSAP to contact the same terminal that originated the emergency call.

If the incoming call can be identified by the core network as a call-back to an emergency call (i.e. coming from a PSAP) then supplementary services at the terminating party shall be handled as described in TS 22.173 [40] for Multimedia Telephony (e.g. Communication Diversion, Communication Hold, Communication Barring).

NOTE: There is no specific callback requirement for CS supplementary services.

A call-back may be attempted for a period of time defined by local regulations after the emergency call release. In case of a UE in limited service state, call-back is not required.

Observation 1: The definition of “normal call” for the case of IMS we can assume is:

- there is no support for emergency call back when the UE is in limited service

- therefore the UE has to have active IMS PDU session and normal IMS registration to deliver the call

- in case of roaming such IMS PDU session in most cases will terminate in HPLMN of the UE

- there is no priority treatment e.g. special ARP, 5QI, RRC establishment cause that can allow the 5GS (CN and RAN) to distinguish between emergency call back and any other mobile terminated call

In addition though:

- There is inactivity monitoring for the presence of user plane resources for the emergency PDU session in SMF:

Extract from TS 23.502, clause 4.4.2.2:

The SMF initiates release of an emergency PDU Session when the UPF reports detection of PDU Session inactivity for a specified period.

Extract from TS 23.503, clause 6.1.3.10:

[…] the SMF shall start a configurable inactivity timer (e.g. to enable PSAP Callback session). When the configured period of time expires the SMF shall terminate the PDU Session serving the IMS Emergency session as defined in TS 23.502

- In SA2#166AHE, Ericsson provided further investigation related to the RLF case and see how UE will behave that indicates that the UE will try to setup the UP resource during SR procedure for the EMC PDU session if the RLF happens when inactivity timer for the EMC PDU session is started but not expired yet. See below text from TS 24.501:

5.6.1.1          General

The UE shall invoke the service request procedure when:

j)   the UE, in 5GMM-CONNECTED mode over 3GPP access or in 5GMM-CONNECTED mode with RRC inactive indication, receives a fallback indication from the lower layers (see subclauses 5.3.1.2 and 5.3.1.4) and the UE has pending uplink user data for PDU session(s) with user-plane resources already established but no pending NAS procedure; the UE, in 5GMM-CONNECTED mode over 3GPP access, receives a "RRC Connection failure" indication from the lower layers and the UE has pending uplink user data for PDU session(s) with user-plane resources already established but no pending NAS procedure;

For case j) in subclause 5.6.1.1:

a)   the UE shall include the Uplink data status IE in the SERVICE REQUEST message indicating the PDU session(s) for which the UE has uplink user data pending and the PDU session(s) for which user-plane resources were active prior to receiving the fallback indication or "RRC Connection failure" indication from the lower layers, if any; and

Observation 2: Considering the above and the inactivity timer handling in SMF (regardless of whether the inactivity is triggered by UPF or gNB), we can consider it’s normal that the emergency PDU session will have user plane resources always (i.e. the network will remove the UP together with the removal of the entire PDU session).

Since rel.13 though when 3GPP introduced for first time extended DRX for idle mode in EPS, text was added in TS 23.401 for the UE to disable extended DRX when it has active bearers for emergency services:

When the UE has bearers for emergency bearer services, the UE and MME follow regular discontinuous reception as defined in clause 5.13 and shall not use the extended idle mode DRX. Extended idle mode DRX parameters may be negotiated while the UE has bearers for emergency bearer services. When the bearers for emergency bearer services are released, the UE and MME shall reuse the negotiated extended idle mode DRX parameters in the last TAU/Attach procedure.

Similar text was added to TS 23.501 when extended DRX was introduced in 5GS

When the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception as defined in clause 5.4.5 and shall not use the extended idle mode DRX. Extended idle mode DRX parameters may be negotiated while the UE has PDU Session associated with emergency services. When the PDU Session associated with emergency services is released, the UE and AMF shall reuse the negotiated extended idle mode DRX parameters in the last Registration Update procedure.

The question is: what is the use case which this text was intended to cover? Given what is mentioned in Observations 1 and 2, it will not cover either the emergency session or emergency call back since it will not be delivered over the emergency PDU session and is handled as “normal call”. Nevertheless, when “Paging Early Indication with Paging Subgrouping Assistance” (PEIPS) was introduced similar text was added:

When the UE has an active emergency PDU Session:

-	The UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

And since the design of LP-WUS follows PEIPS the aforementioned Editor’s Note was added.

Observation 3: Current specifications require the UE to “disable” idle mode extended DRX and not signal Paging Subgrouping Support Indication when it has active PDU session for emergency services.

# 3. 	Next steps

Given observations 1, 2 and 3, it makes no sense (adds no benefit) to disable features that may impact paging (eDRX, PEIPS, LP-WUS) since there is no possibility of sending MT signalling within the emergency PDU session when the UE is in CM_IDLE state.

Observation 4: there is no benefit to disable features that may impact paging (eDRX, PEIPS, LP-WUS) when emergency PDU session is active.

Considering this text exists in specifications up to rel.18 and there are test cases defined, see below:

- In TS 36.523 cl 9.2.2.1.3, this is a test case to validate UE is not requesting eDRX during emergency session

- In TS 38.523 cl 11.4.1a, there is a test case to validate UE is not using PEIPS assistance information (they test paging the UE while emergency PDU session is active)

Observation 5: For UE not requesting eDRX and not using PEIPS when it has active emergency PDU session there are existing text cases.

It is proposed that CRs are approved from rel.19 in order not to create backwards incompatibilities for existing UE implementations.

Proposal 1: it is proposed to approve CR S2-2501569, that removes this text from rel.19 version of TS 23.501

The same analysis “mutatis mutandis” applies also to EPC.

Proposal 2: it is proposed to approve CR S2-2501570, that removes this text from rel.19 version of TS 23.401

# 4.	How to handle LS R2-2411041“ LS on emergency services and eDRX”

The LS R2-2411041 “LS on emergency services and eDRX” relates also to the analysis in previous clauses of this paper. Namely, when the UE is RRC_INACTIVE state with extended DRX it can only receive emergency call back over IMS PDU session (see observation 1). It is therefore proposed to answer the LS of RAN2 accordingly.

Proposal 3: Answer to the RAN2 LS on emergency services and eDRX (R2-2411041) as proposed in S2-250abcd.

# 5.	Conclusions

The following proposals are requested to be agreed:

Proposal 1: it is proposed to approve CR S2-2501569, that removes this text from rel.19 version of TS 23.501

Proposal 2: it is proposed to approve CR S2-2501570, that removes this text from rel.19 version of TS 23.401

Proposal 3: Answer to the RAN2 LS on emergency services and eDRX (R2-2411041) as proposed in S2-2501572.