SA WG2 Meeting #167	S2-2501595

Athens, Greece, 17 – 21 February 2025

Source:	Ericsson, Huawei, HiSilicon, Samsung

Title:	MWAB-UE HO considering preventable multi-hop

Document for:	Agreement

Agenda Item:	19.5

Work Item / Release:	VMR_Ph2 / Rel-19

Abstract of the contribution: This contribution discusses the MWAB-UE HO considering the preventable multi-hop.

# 1.Background

RAN3 has provided LS reply in S2-2500052/R3-247909 which addressed the MWAB Access Control aspect during the handover procedure with the following:

RAN3 has agreed the following solution to prevent multi-hop backhauling upon WAB-MT HO:

For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling.

Observation 1: RAN3 indicates that the target MWAB-gNB is responsible for the failure of HO preparation based on supported S-NSSAIs which are dedicated for the BH PDU session and not configured/supported in a MWAB-gNB.

# 2.Discussion on MWAB-UE HO with BH PDU sessions

## 2.1 Handover scenarios for slice based multi-hop control

For the MWAB-UE handover procedure, it includes both Xn based handover (e.g. when there is no change of the AMF) and N2 based handover (e.g. including the case of AMF change with or without PLMN change)

For the Xn based handover, CN is only involved in the execution phase. It’s clear that target RAN node (e.g. MWAB-gNB) is in responsible of resource preparation and fail the HO procedure of the MWAB-UE if the corresponding slice(s) of the BH PDU Session(s) of the MWAB-UE are not configured/supported in target RAN (e.g. MWAB-gNB). This is re-use of the existing slice-based resource control during handover preparation in a target gNB side. No special handling specifically for the MWAB-gNB.

Observation 2: During Xn handover procedure. Target RAN (e.g. MWAB-gNB) node is responsible for the resource preparation and fail the HO procedure based on existing slice control logic.

However, for the N2 based handover, CN is also involved during the handover preparation, e.g. CN needs to prepare the CN side information for the N3 tunnel before the sending of NGAP Handover Request message to target RAN. This is clearly stated in step 4 of clause 4.9.1.3.2 of TS 23.502 where AMF needs control the PDU sessions based on slice. AMF is aware of the supported TAIs/Slices based on the NGAP Setup procedure when a gNB is connected to AMF.

4.	[Conditional] T-AMF to SMF: Nsmf_PDUSession_UpdateSMContext (PDU Session ID, Target ID, T-AMF ID, N2 SM Information).

For each PDU Session indicated by S-RAN, the AMF invokes the Nsmf_PDUSession_UpdateSMContext Request to the associated SMF. However, if the S-NSSAI associated with PDU Session is not available in the T-AMF, the T-AMF does not invoke Nsmf_PDUSession_UpdateSMContext for this PDU Session.

PDU Session ID indicates a PDU Session candidate for N2 Handover. Target ID corresponds to Target ID provided by S-RAN in step 1. SM N2 Info includes the Direct Forwarding Path Availability if the direct data forwarding is available between the S-RAN and the T-RAN and has been inserted by the S-RAN.

If the (T-)AMF detects that the UE moves into a non-allowed area based on Service area restrictions, the (T-)AMF notifies each NF consumer which has subscribed for UE reachability event (e.g. SMFs corresponding to the list of PDU Sessions received in UE Context from (S-)AMF via Namf_EventExposure_Notify that the UE is only reachable for regulatory prioritized services.

Observation 3: During N2 handover procedure. CN is also involved the resource preparation and controls the PDU sessions resource preparation based on existing slice control logic.

Furthermore, the NGAP Handover Request message for N2 handover requires the PDU session information (e.g. N3 tunnel info) as mandatory IE based on TS 38.413. This means N2 Handover procedure will continue with at least one PDU session that can be accepted by the CN side.

In case CN fails all the PDU sessions of a UE, e.g. due to slice control, and the N2 handover procedure cannot be continued, the existing service operation between target AMF and old AMF support already such case based on TS 29.518 with proper error code.

#### 9.2.3.4	HANDOVER REQUEST

This message is sent by the AMF to the target NG-RAN node to request the preparation of resources.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| Handover Type | M |  | 9.3.1.22 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | M |  | 9.3.1.58 |  | YES | reject |
| Core Network Assistance Information for RRC INACTIVE | O |  | 9.3.1.15 |  | YES | ignore |
| UE Security Capabilities | M |  | 9.3.1.86 |  | YES | reject |
| Security Context | M |  | 9.3.1.88 |  | YES | reject |
| New Security Context Indicator | O |  | 9.3.1.55 |  | YES | reject |
| NASC | O |  | NAS-PDU 9.3.3.4 | Refers to either the “Intra N1 mode NAS transparent container” or the “S1 mode to N1 mode NAS transparent container”, the details of the IE definition and the encoding arespecified in TS 24.501 [26]. | YES | reject |
| PDU Session Resource Setup List |  | 1 |  |  | YES | reject |
| >PDU Session Resource Setup Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>S-NSSAI | M |  | 9.3.1.24 |  | - |  |
| >>Handover Request Transfer | M |  | OCTET STRING | Containing the PDU Session Resource Setup Request Transfer IE specified in subclause 9.3.4.1. | - |  |
| >>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.3.1.94 | Expected UE Activity Behaviour for the PDU Session. | YES | ignore |
| Allowed NSSAI | M |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network. | YES | reject |
| … | … |  | … | … | … | … |

##### 6.1.6.2.43	Type: UeContextCreateError

Table 6.1.6.2.43-1: Definition of type UeContextCreateError

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| error | ProblemDetails | M | 1 | Represents the detailed application error information. The application level error cause shall be encoded in the "cause" attribute. |  |
| ngapCause | NgApCause | C | 0..1 | This IE shall be present, if available. When present, it shall represent the NGAP Cause received from RAN. |  |
| targetToSourceFailureData | N2InfoContent | C | 0..1 | This IE shall be present if a "Target to Source Failure Transparent Container" has been received from the target NG-RAN. When present, this IE shall contain this container. | NPN |

#### 6.1.7.3	Application Errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| NF_CONSUMER_REDIRECT_ONE_TXN | 307 Temporary Redirect | The request has been asked to be redirected to a specified target. |
| HANDOVER_FAILURE | 403 Forbidden | Creation of UE context or relocation in the target AMF failed during Handover procedure causing a failure of handover. |
| … | … | … |

Observation 4: During N2 handover procedure. Handover procedure can be failed by the CN side based on existing slice control logic.

Proposal 1: Specify that target side, both target CN and target RAN, can fail the BH PDU sessions of the MWAB-UE based on existing slice control logic.

## 2.2 PDU sessions of MWAB-UE

MWAB-UE will use dedicated S-NSSAIs in the backhaul PLMN/SNPN to setup BH PDU sessions. However, the MWAB-UE is not restricted to establish normal PDU sessions as normal UE as long as it’s allowed by the subscription of the MWAB-UE.

During the Xn/N2 handover procedure, when a MWAB-UE with both BH PDU sessions and normal PDU sessions try to access another MWAB-gNB, the BH PDU sessions will be checked by the target side as described in clause 2.1 above based on existing slice control logic and BH PDU session resources will not be setup if the corresponding dedicated S-NSSAIs are not supported by the target MWAB-gNB/AMF, and the Allowed NSSAI for the MWAB-UE will be updated to not include the dedicated S-NSSAIs accordingly. For the normal PDU sessions, it may be entitled for the resource setup and the Handover procedure can be successful if the corresponding S-NSSAI is supported by the target MWAB-gNB/AMF. However, during this handover procedure, the MWAB-UE becomes a normal UE. From concept point of view, the target side prevented the MWAB-UE connects to a MWAB-gNB, but it allows the normal UE be connected via the MWAB-gNB. There is no need of change to the existing slice control logic, but clarification with a NOTE is needed.

Proposal 2: Add a NOTE to clarify that MWAB-UE may handover to MWAB-gNB if it has normal PDU sessions other than the BH PDU sessions.

# Proposal

Based on the above discussion, observations, and proposals, it is recommended to agree the following:

1) SA2 specifies that target side controls the BH PDU sessions based on existing slice logic. Please see CR S2-2501596.

2) Add a NOTE to clarify that target side with MWAB-gNB can accept MWAB-UE as normally UE if there is normal PDU sessions.