SA WG2 Meeting #167	S2-2501604

17 – 21 February 2025, Athens Greece	(revision of S2-25xxxxx)

Title:	[Draft] Reply LS for OSAppID usage by AppToken use case

Response to:	LS (S2-2501377) from GSMA TSGNS: OSAppID usage by AppToken use case

Release:	Rel-19

Work Item:	-

Source:	SA2

To:	GSMA TSGNS

Cc:	SA3

Contact Person:	belen.pancorbo@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

# 1	Overall description

SA2 thanks for the LS from GSMA on “OSAppID usage by AppToken use case”. SA2 discussed the questions from GSMA, and provide the following responses:

Question 1: Do the existing 3GPP specifications allow the network to use application identities that are generated by an application backend to populate URSP in step 2?

Response to Question 1: The procedure described in TS 23.502, clause 4.15.6.10 “Application guidance for URSP determination” enables an Application Function to provide Service Parameters including an application traffic description in the form of an Application identifier that is specific for an operating system in the UE. The AF provides OSId and OSAppID. There are no assumptions or constrains on how the AF obtains the OSAppID or the OSId.

Question 2: Would 3GPP SA2 prefer an alternate mechanism to preserve end user privacy?

Response to Question 2: This is SA3 scope.

Question 3: Consider the architectural implications of the proposed mechanism and provide feedback.

Response to Question 3: In general, there are no impacts on the 3GPP SA2 architecture from the proposal by GSMA in S2-2501377. More in detail:

- the proposed mechanism includes provisioning of URSP Rules to the UE including OSAppID within the Traffic description that is aligned with 23.503. The PCF may provide URSP Rules that are different for different registered UEs, as such the OSAppId provided to the UE may depend on the provided OSid, and the RSD may depend on the Route Selection parameters provided by the Application Function, and operator policies.

- The Operator entitlement server to 3rd party App back-end interface is not in 3GPP SA2 scope.

- The UE internals are not specified by 3GPP SA2 either.

# 2. Actions

To GSMA TSGNS group.

ACTION: 	SA2 kindly asks GSMA TSGNS group to take the above comments into account.

# 3. Date of Next SA2 Meetings

SA2#168                           7th April- 11th April 2025	Gothenburg, Sweden

SA2#169                         19th May – 23rd May 2025	Fukuoka, Japan