SA WG2 Meeting #167	S2-2501605

Athens, Greece, 17 – 21 February 2025

Source:	Ericsson

Title:	Energy Attribution sensitivity and robustness

Document for:	Agreement

Agenda Item:	19.4.2

Work Item / Release:	EnergySys / Rel-19

Abstract of the contribution: This contribution discusses the sensitivity and robustness of the current formulas for attribution of energy consumption to a UE or finer granularity.

# 1.Background

In our system, the network energy consumption cannot be directly measured for each UE or at finer granularities. Instead, it is attributed to a UE, or more detailed levels, using other measurements and a model e.g. formula. To produce usable results the model/formula for energy consumption attribution needs to be robust and sensitive to the changes in the UE (or finer granularity) energy consumption.

# 2.Discussion on energy consumption attribution

For this discussion I will use a simple analogy of a hotel that use a model to attribute energy consumption to each guest. This model is very similar to our current formulas for energy attribution from gNB and UPF to per-UE or finer granularities.

Imagine a hotel that has a capacity of 100 guests (single rooms).

- The hotel consumes 1000 Units of energy (uE) without guests: idle consumption

- Each guest adds an average of 10 uE (room cooling/heating, hot water, share of laundry, share of cooking breakfast buffe etc)

Model: Energy attribution to each guest = (Idle consumption + 10 * no of guests)/ no of guests

With 50% occupation the energy attribution per guest is = (1000+10*50)/50 = 30 uE

How sensitive is the per guest energy consumption attribution to changes in a single guest halving of doubling his energy consumption?

Half: (1000+10*50-5)/50 = 29.9 uE

Double: (1000+10*50+10)/50 = 30.2 uE

How sensitive is the per guest energy consumption attribution to changes in the number of guests:

30% occupation: (1000+10*30)/30 = 43.3 uE

70% occupation: (1000+10*70)/70 = 24.3 uE

Observation 1: This model is very sensitive to changes in the number of guests at the hotel but it’s not sensitive at all to dramatic changes in single guests’ contribution to the energy consumption.

If we compare the model above with the energy attribution models we have defined in rel-19 we can see that for each gNB or UPF the model is similar to the model above. The total data volumes data volumes (DVgNB and DVUPF) play the same role as the total number of guests in the example above.  The idle energy consumption and the added energy consumption per added data volume may differ a bit but we estimate that using real world data in the currently defined formulas would show even more sensitive the total data volumes and less sensitive to the UE, or finer granularity, data volume. I would invite others to repeat the analysis with real world numbers.

Observation 2: Our current models for attribution of network energy consumption for a UE or finer granularity are not robust as they are very sensitive to changes in total data volumes for the NFs (DVgNB and DVUPF).

As the current models for energy consumption attribution to a UE (or finer granularity) are not robust, the results they produce are not suitable for credit control, policy control or analytics.

Proposal 1: Agree that our current energy attribution formulas are not robust and does not produce results that are suitable for credit control, policy control or analytics.

# Proposal

It’s proposed to:

- Endorse Proposal 1.

- Agree 23.501CR5900

In addition, proposal 1 and the observations should be taken into account when discussing other CRs for Rel-19 and the Study item for Rel-20