3GPP TSG-SA2 Meeting #167	S2-2501656

# Athens, Greece, February 17 – 21, 2025        			             			(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6096 | rev |  | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the support of regulatory service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the following NOTE in clause 5.49.8, NOTE: If the MWAB is configured to support emergency services for the UE, it is assumed that the MWAB Broadcasted PLMN/SNPN and BH PLMN/SNPN are in the same country. There are some issues need to be clarified, as follows: - Issue#1: MWAB-UE roaming If the MWAB-UE is roaming, besides the PLMNs mentioned in the NOTE, there is another PLMN exists, i.e. the MWAB-UE HPLMN. In this case, whether it is required or not that the MWAB-UE HPLMN shall in the same country as the two PLMNs in the NOTE. This CR is proposed to clarify that in this case, the MWAB-UE HPLMN should also in the same country as the two PLMNs mentioned in the NOTE. - Issue#2: the information considered by OAM to determine the correct configuration related to emergency service. Based on the description in clause 5.49.8, the OAM is responsible to determine the configuration related to emergency services. Based on the NOTE above, if the configuration is supporting emergency services, it is required that the MWAB Broadcasted PLMN/SNPN and BH PLMN/SNPN are in the same country. However, the OAM has no idea of the BH PLMN/SNPN ID, thus it is impossible for OAM to determine whether the two PLMNs/SNPNs above are in the same country or not. So it is proposed that the MWAB-gNB provides the BH PLMN/SNPN ID and the MWAB-UE HPLMN ID to the OAM. Based on the PLMN ID, the OAM can determine whether the PLMNs are in the same country or not, and then determine the configuration related to the emergency services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Support the MWAB-UE roaming scenario for OAM configuration related to emergenc services. - Clarify the OAM determines the configuration related to emergency services based on the BH PLMN/SNPN ID and/or MWAB-UE HPLMN ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how does OAM determine whether the MWAB Broadcasted PLMN/SNPN and BH PLMN/SNPN are in the same country and it is not clear whether the MWAB-UE roaming impacts the OAM configuration related to emergency service or not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

5.49.8	Support of regulatory service via MWAB

Based on the OAM configurations, an MWAB may support emergency services for UEs connected to the MWAB-gNB.

When the MWAB determines that the UE connects to the MWAB-gNB has initiated an emergency PDU Session, e.g. via AS layer signalling, or based on the ARP of the PDU Session context, the following handling applies:

-	The MWAB may decide to reuse the existing BH PDU Session if the existing BH PDU Session has sufficient QoS to support high availability of the N2/N3 interface.

-	The MWAB may decide to establish a new BH PDU Session, if the HPLMN of MWAB-UE has configured it with specific S-NSSAI and DNN for BH PDU Session(s) to serve emergency PDU Session for UEs accessing this MWAB.

-	The emergency PDU Session establishment procedure for the UE(s) should not be interrupted by the establishment of the new BH PDU Session. To avoid interruption of the establishment of the emergency PDU Sessions, the MWAB-gNB should complete of the emergency PDU Session establishment by using the existing BH PDU Sessions for N3 traffic and then trigger PDU Session resource modification procedure for the emergency PDU Sessions of the UE(s) to associate the emergency PDU Session N3 interface to the new BH PDU Session to serve emergency PDU Session for UEs accessing this MWAB.

-	If there are UEs with emergency services, the MWAB-gNB will not stop operating(e.g. due to de-authorization) as MWAB until it handovers the UEs to other cells as described in clause 5.49.3.

-	Based on configuration for the specific S-NSSAI and DNN for BH PDU Session to serve emergency PDU Sessions, the BH PLMN/SNPN and the MWAB attempt to guarantee the resources of the BH PDU session(s) to serve emergency PDU Session in the BH PLMN/SNPN so that they are not be released when MWAB is serving the emergency services.

NOTE:	If the MWAB is configured to support emergency services for the UE, it is assumed that the MWAB Broadcasted PLMN/SNPN, BH PLMN/SNPN and MWAB-UE HPLMN which is considered when the MWAB-UE is roaming are in the same country. The OAM in the Broadcasted PLMN/SNPN determines the configuration related to emergency service by considering the BH PLMN/SNPN ID and/or MWAB-UE HPLMN ID provided by MWAB-gNB. The MWAB-gNB requests the BH PLMN/SNPN ID and/or MWAB-UE HPLMN ID from the MWAB-UE via the implementation based internal communication between the MWAB-gNB and the MWAB-UE.

* * * End of Change * * *