3GPP TSG-SA2 Meeting #167	S2-2501657

Athens, GREECE, February 17 – 21, 2025 	(revision of S2-25xxxxx)

Source:	CATT

Title:	Discussion on user plane connection modification

Document for:	Discussion

Agenda Item:	9.6.2

Work Item / Release:	5G_eLCS_Ph3 / Rel-18

Abstract of the contribution: this discussion paper is proposed to discuss the issue related to the user plane connection modification procedure and the associated way forward.

1. Background

1.1	Internal logic of UE with user plane positioning capability

The illustration of internal logic of UE with user plane positioning capability is shown in Figure 1.

Figure 1: Internal logic of UE with user plane positioning capability

Based on the following description in TS 24.572, LPP messages and specific supplementary services messages can be transferred via user plane connection. So the LCS shall be enhanced to store the user plane connection status and support the function of user plane and control plane positioning selection, e.g. CP is selected when UP is released, CP is selected when the SS message is 5GC-MO-LR request.

The main function of the Location Services User Plane Protocol (LCS-UPP) is to support generic transport of the following messages for positioning related protocols, via an LCS secured user plane connection between the UE and the LMF:

a)	LPP messages, which are specified in 3GPP TS 37.355 [13]; and

b)	LCS supplementary services messages, only including:

1)	messages for EventReport operations (see 3GPP TS 24.080 [11]);

2)	messages for PeriodicTriggeredInvoke operations (see 3GPP TS 24.080 [11]); and

3)	messages for MSCancelDeferredLocation operations (see 3GPP TS 24.080 [11]).

1.2	User plane connection modification

The clause 6.18.3 in TS 23.273 describes the procedure to modify the user plane connection between UE and LMF. And the procedure may be performed in the following two scenarios:

-	Scenario#1: When LMF receives an event report via user plane from UE and the LMF discovers a need to change LMF.

-	Scenario#2: When AMF relocation happens, the target AMF receives the UE LCS-UP context which includes the source LMF from the source AMF. To store the correct information, the target AMF determines whether the LMF relocation is needed or not based on UE location. If needed, the target AMF triggers user plane connection modification.

The related description in TS 23.2773 is as follows:

1a.	[Conditional] The LMF discovers a need to change LMF or re-establish the user plane connection between UE and LMF or terminate the user plane connection, e.g. after receiving an event report via user plane from UE or after receiving an AMF relocation information from a target AMF (target AMF obtains from source AMF the UE LCS-UP context which indicates UE has maintained LCS-UP connection with the source LMF, the target AMF may inform the source LMF about the AMF change using the Nlmf_Locatoion_UPConfig request). The LMF sends an Nlmf_Location_UPNotify message that includes connection move (termination and establishment) or termination is required and if connection move is requested then message may include target LMF identification. The address of the AMF was provided to LMF as a "Notification Target Address" in latest Nlmf_Location_UPConfig message or Nlmf_Location_UP Subscribe message. If the LMF is going to terminate the user plane connection, step 1a, step 3, step 6 and step 7 are performed and other steps are skipped.

2. Question Analyses

2.1	Event Report Triggered User Plane Connection Modification

When the user plane connection modification procedure is triggered by event report, the UE behaviour and network behaviour is illustrated in Figure 2.

Figure 2: UE and network behavior when user plane connection modification procedure is triggered by Event Report

After the step 6 in Figure 2, the UE stores the deferred routing ID of target LMF. When the UE sends the next Event Report, if the user plane is selected, the LCS layer provides the Event Report and the deferred routing ID of target LMF to the LCS-UPP which is responsible to generate the UL LCS-UP transport message including the Event Report and the deferred routing ID of target LMF to the target LMF. The target LMF handles the UL LCS-UP transport as usual. So if the user plane connection modification is triggered by Event Report. The existing mechanism works well.

Observation#1: The existing mechanism works well if the user plane connection modification is triggered by source LMF when the Event Report message is received.

2.2	AMF Relocation Triggered User Plane Connection Modification

When the user plane connection modification procedure is triggered by AMF relocation, the UE behaviour and network behaviour is illustrated in Figure 3.

Figure 3: UE and network behavior when user plane connection modification procedure is triggered by AMF relocation

Based on the step 6 in Figure 3, after the user plane connection modification procedure, the UE LCS layer still stores the deferred routing ID of the source LMF. However the UE LCS-UPP layer has been updated to associate with the IP address of the target LMF in step 3 in Figure 3. So the status between UE LCS layer and UE LCS-UPP layer is misaligned. Furthermore, after step 4-5 in Figure 3, the source LMF removes the UE location context which is maintained in target LMF, but the UE has no idea of this information, so the status between UE and network is also misaligned.

In this case, when the UE detects an event happens and sends the Event Report, the UE LCS layer sends the Event Report and the deferred routing ID of source LMF to UE LCS-UPP layer if user plane is selected. The UE LCS-UPP layer generates the UL LCS UP transport message including the Event Report and the deferred routing ID of source LMF and sends the message to the Target LMF. When the deferred routing ID of other LMF is received by Target LMF, it is not clear the behavior of the Target LMF. Furthermore, it is not clear how to solve the status misalignment issues above.

Observation#2: If the LMF relocation happens during the user plane connection modification procedure triggered by AMF relocation, after the procedure, there are status misalignments between different layers in UE and between UE and network.

Observation#3: If the status between UE LCS layer and UE LCS-UPP layer is misaligned, the UE LCS-UPP layer generates the UL LCS UP transport message including the Event Report and the deferred routing ID of the LMF other than the serving LMF. When the Target LMF which is the only serving LMF of UE receives the deferred routing ID of other LMF, the behaviour of the Target LMF is not clear.

3. Solutions and Evaluation

To solve the issue described in clause 2.2, the possible solutions and the evaluation is shown in Table 1.

Table 1: Solutions and evaluation

| Solution | Description | Evaluation |
| --- | --- | --- |
| Sol#1 | Introduce new interaction(s) between LMF and UE to align the status between UE and LMF. | The UE and the LMF are impacted. And it is not feasible to discuss and introduce new function for Rel-18 which has been frozen for a long time. |
| Sol#2 | When the LMF receives the UL LCS UP transport message including the Event Report and the deferred routing ID of other LMF, the LMF considers the UE status is not aligned with the network. In this case, the LMF generates the Event Report ACK including the LMF relocation indication and the new deferred routing ID referring to the LMF and sends the message to UE. The LMF sends DL LCS UP transport message including the Event Report ACK and a deferred routing ID to the UE via user plane connection. In this case, the deferred routing ID should be the one provided by UE in the UL LCS UP transport message. Because the UE LCS layer can only identify this deferred routing ID which is also used by the UE to associate the Event Report ACK with the correct deferred 5GC-MT-LR context. When the UE receives the DL LCS UP transport message, the UE behaviour is the same as the existing UE behaviour, as follows: - Based on the deferred routing identifier, the UE can associate the Event Report Acknowledgement message with the correct deferred 5GC-MT-LR procedure. - Based on the Event Report Acknowledgement message, the UE knows the LMF relocation happens and updates the stored deferred routing identifier with the received one. | This solution only impacts the LMF and existing mechanism is re-used as much as possible. However, whether this solution can apply to the multiple LMFs needs further evaluation. |
| Sol#3 | Remove the scenario of AMF relocation triggered user plane connection modification. | By removing the scenario, the status misalignment does not happen no matter one LMF case or multiple LMF case. But without the LMF relocation, the target AMF stores the source LMF ID in the user plane connection context. |

Based on the evaluation in Table 1, the observations are as follows:

-	Sol#1 introduces new function(s) which requires study firstly, so this solution is not feasible in Rel-18 which has been frozen for a long time.

-	Sol#2 only impacts the LMF and reuses the existing mechanisms as much as possible. However, when the feature of multiple user plane connections is introduced, whether the solution can still be applied needs further evaluation.

-	Sol#3 prevents the issue by removing an existing trigger for user plane connection modification, i.e. AMF relocation. Without the LMF relocation, the target AMF still stores the source LMF ID in UE context. When the upcoming location request(s) is received by the target AMF, it may select the source LMF by considering the LMF ID in the user plane connection context and send the location request(s) to the source LMF. In this case, because the source LMF is not the suitable LMF to serve the current UE location, it may not obtain the valid measurements which leads to the reduction of location accuracy.

4. Proposal

Based on the evaluation in clause 3, the sol#2 has less impact, so the following proposals are proposed.

Proposal#1: In Rel-18, only one user plane connection is supported.

Proposal#2: Clarify the LMF behavior that when the LMF receives the UL LCS UP transport message including the Event Report and the deferred routing ID of other LMF, the LMF considers the UE status is not aligned with the network. In this case, the LMF generates the Event Report ACK including the LMF relocation indication and the new deferred routing ID referring to the LMF and sends the message to UE.