3GPP TSG-SA2 Meeting #167	S2-2501658

# Athens, Greece, February, 17 – 21, 2025        			             			(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0674 | rev |  | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the user plane connection modification procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the description in clause 6.18.3, the user plane connection modification procedure can be triggered by AMF relocation. The target AMF may further triggers LMF relocation. In this case, for the UE with ongoing deferred 5GC-MT-LR procedure, there is no way to notify UE about the LMF relocation. So the UE still stores the deferred routing identifier of the source LMF. But the user plane connection has been updated which is between UE and the target LMF. So there are status misalignments between different layers in UE and between the UE and the network. In this case, when the UE sends an Event Report, it sends the Event Report and the deferred routing identifier of the source LMF via user plane connection to the target LMF. When the target LMF receives the deferred routing identifier of other LMF, the behaviour of the target LMF is unclear. Furthermore, it is not clear how to solve the status misalignments issue above. To solve the issues above, it is proposed to clarify that when the target LMF (i.e. the existing serving LMF) receives the Event Report and the deferred routing ID referring to other LMF, it generates the Event Report ACK including the LMF relocation indication and the new deferred routing ID referring to the LMF and sends the message to UE. When the UE receives the message, the UE updates the stored deferred routing ID using the received one. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the LMF behaviour to identify the status misalignment between UE and the LMF and to solve the issue by providing the UE with the Event Report ACK including the LMF relocation indication and the new deferred routing ID referring to the LMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Status misalignments between different layers in UE and between the UE and the network. And it is not clear the LMF behaviour when it receives the deferred routing ID of other LMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

6.18.3	Modification of User Plane Connection between UE and LMF

The figure 6.18.3-1 shows how a secure user plane connection between UE and LMF is modified. The flow describes change of LMF but applies also when source and target LMF is the same. The procedure can also be used to terminate the user plane connection to Source LMF not selecting any Target LMF.

Figure 6.18.3-1: Connection modification between UE and LMFs

1a.	[Conditional] The LMF discovers a need to change LMF or re-establish the user plane connection between UE and LMF or terminate the user plane connection, e.g. after receiving an event report via user plane from UE or after receiving an AMF relocation information from a target AMF (target AMF obtains from source AMF the UE LCS-UP context which indicates UE has maintained LCS-UP connection with the source LMF, the target AMF may inform the source LMF about the AMF change using the Nlmf_Locatoion_UPConfig request). The LMF sends an Nlmf_Location_UPNotify message that includes connection move (termination and establishment) or termination is required and if connection move is requested then message may include target LMF identification. The address of the AMF was provided to LMF as a "Notification Target Address" in latest Nlmf_Location_UPConfig message or Nlmf_Location_UP Subscribe message. If the LMF is going to terminate the user plane connection, step 1a, step 3, step 6 and step 7 are performed and other steps are skipped.

If the request is to terminate the user plane connection, the AMF releases the LCS-UP context.

NOTE 1:	The LMF change procedure is independent from the SSC mode of the PDU Session with dedicated DNN used for positioning. For SSC mode 2/3, PSA UPF connection with the LMF can be relocated with UE movement and LMF can discover the need to change LMF to reduce the user plane path latency.

1b.	[Conditional] AMF based on target LMF identification received from source LMF for user plane positioning, or AMF may perform LMF reselection if UE moves to a new location (which may be out of serving area of source LMF and in serving area of target LMF) and select the target LMF for the current UE location based on LMF service area, LMF user plane positioning capability information and other information listed in clause 5.1. LMF needs to be capable to establish a user plane session for positioning with the UE. After AMF relocation, the target AMF may tigger the LMF reselection and LCS-UP connection modification procedure, e.g. if the source LMF does not perform LMF reselection.

2.	[Conditional] Steps 3-10 of figure 6.18.2-1 are performed between AMF (or Target AMF), UE, and Target LMF with addition that UE also terminate connection to Source LMF.

3.	[Conditional] If the user plane connection between UE and Target LMF is established in step 2, the AMF sends an Nlmf_Location_UPConfig Request towards the source LMF. The message may include a request for the Source LMF to terminate a specific user plane connection to the UE and the Target LMF identification. Alternatively, it may include information about AMF reallocation.

NOTE 2:	AMF relocation does not necessarily always cause LMF reselection, the AMF relocation information can keep the LMF informed.

4-5.[Conditional] The source LMF may invoke an Nlmf_Location_LocationContextTransfer Request service operation towards the target LMF to provide the current location context(s) of the UE, if there is user plane based periodic and triggered UE location events report context(s). The target LMF informs source LMF of the location context transfer operation results.

When the target LMF receives the Event Report and the deferred routing identifier from the UE via user plane connection, if the deferred routing identifier indicates other LMF, the target LMF sends the Event Report Acknowledgement message and the deferred routing identifier received from UE to the UE via user plane connection. The Event Report Acknowledgement indicates a change of LMF and includes a deferred routing identifier indicating the target LMF.

NOTE 3:	When the UE receives the Event Report Acknowledgement message and the deferred routing identifier via user plane, based on the deferred routing identifier, the UE can associate the Event Report Acknowledgement message with the correct deferred 5GC-MT-LR procedure. Based on the Event Report Acknowledgement message, the UE knows the LMF relocation happens and updates the stored deferred routing identifier with the received one.

6.	[Conditional] If the user plane connection to source LMF is still active, the source LMF terminates the connection to the UE.

7.	The LMF sends Nlmf_Location_UPConfig Response message to AMF to confirm connection termination or acknowledge change of AMF. If this procedure is used for termination, the AMF will release the LCS-UP context after receiving the response message.

* * * End of Change * * *