3GPP SA WG2 Meeting #167	 S2-2501665

# Athens, Greece, 17 - 21 February, 2025									(revision of S2-2500292)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1546 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification on general description for standalone IMS DC session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. The wording “user concern” is incorrect. 2. The application readable name may be obtained from DCSF via application list, instead of from the application itself. 3. Some unclear part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Correct the wording of “user concern” to “user consent”. 2. Change “information from application” to “information for the application” 3. Some editorial change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change* * *

# AC.10	Standalone IMS Data Channel Sessions

## AC.10.1	General

A standalone IMS data channel session is applied to establish data channels without MMTel media (e.g. audio, video, messaging). A standalone IMS data channel session with bootstrap data channels is used for downloading application list and applications from the DCSF. Application data channels between two UEs may be initiated with bootstrap data channels simultaneously or not during the standalone IMS data channel session establishment.

NOTE 1:	The remote UE can reject the IMS data channel session without downloading the application.

There are two types of standalone IMS data channel session:

a)	IMS session with only bootstrap data channel towards PSI, which cannot add MMTel media.

b)	IMS session with only bootstrap and application data channels towards peer UE, which can add MMTel media.

In the case of b), the terminating UE sends 180 Ringing only when the terminating UE successfully downloaded the DC application indicated by the originating UE. The IMS data channel session is able to be answered by the terminating UE after the DC application for communication is allowed to run on the terminating UE. It is assumed early media is used during the process.

NOTE 2:	The alert on terminating UE for an incoming standalone IMS data channel session may occur at proper time for user consent on, e.g. incoming standalone IMS data channel session, application downloading, and/or application running, etc. The terminating UE may be able to be configured by the user to automatically allow some specific performance, e.g. application downloading. The alert may be a ringtone accompanied with additional display of the information for the application, e.g. the human readable application name, the brief introduction of the application, the required permissions of the application, etc. The detail of information shown to the user is out of scope of this specification.

NOTE 3:	The time between alerting the terminating user and the download of the application needs to conform to SIP Session timers.

The standalone IMS data channel session towards peer UE can be updated by adding MMTel media to the session. All MMTel media can be removed from an IMS data channel session towards peer UE if standalone IMS data channel session is supported.

When standalone IMS data channel session is supported, based on operator policy, additional subscription information for standalone IMS data channel service may be required to be included in service subscription. When the additional subscription information is needed, it is used by the IMS AS to determine whether to accept or reject a standalone IMS data channel session, e.g.

if a UE that has not subscribed for standalone IMS data channel session, the IMS AS rejects the request of standalone IMS data channel session establishment.

Standalone IMS data channel sessions use SIP session timer as per RFC 4028 [106] to avoid hanging resources in the UE and the network. Usage of SIP session timer in IMS is specified in TS 24.229 [10a]. Establishment of standalone IMS data channel sessions does not require SIP precondition.

The standalone IMS data channel session is terminated by the existing session release procedure specified in clause AC.7.6. The session release may be triggered by the expiry of the session timer or by sending a SIP BYE request.

* * * End of Changes * * *