3GPP TSG-SA WG2 Meeting #167	S2-2501666

# Athens, Greece, 17 - 21 February, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1585 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on UE-Satellite-UE communication determination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedures for USU at call setup, and USU at call setup with early media are agreed, the feature description on USU needs to be update accordingly. There’s a EN related to subscription for USU, considering USU is not aware by user, no need to consider subscription for USU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update AE.3.2 and AE.4.2 according to the procedures Editorial modification in AE.1 Remove ENs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not consistent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AE.1, AE.3.2, AE.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change ---

# AE.1	General

This annex describes IMS architecture enhancements to support UE-Satellite-UE communication in IMS, i.e., optimized media routing via IMS user plane on-board satellite.

In this Release of the specification, this feature is supported only for IMS voice/video service and for UEs belonging to the same PLMN and in the non-roaming scenario.

The term and the 5GS architecture for UE-Satellite-UE communication is defined in TS 23.501 [93]. The optimized media routing described in this annex refers to a routing of media between UEs under the coverage of the same or of different serving satellites, using the 5GS network as the IP-CAN and with the media not transiting through any network elements on the ground. The ground fallback routing refers to the case with the media transiting through the ground segment.

The P-CSCFs serving the MO UE and the MT UE negotiate with each other to determine whether to activate UE-satellite-UE communication, that is, whether it is possible to apply media routing that only relies on IMS user plane on-board satellite.

## ---Start of the 2nd Change ---

## AE.3.2	At call setup

The P-CSCF receives from the PCF the identifier of the satellite serving the UE and receives from the P-CSCF serving the remote UE the identifier of the satellite serving the remote UE. Subject to operator policy, the P-CSCF uses this information to determine whether or not activate optimized media routing.

If the P-CSCF determines the activation of optimized media routing, the P-CSCF informs the identifier of the satellite serving the UE to the P-CSCF serving the remote UE when needed, and instructs the PCF to authorize the necessary resources according to clause 5.6.2 and clause 5.7.2, so that the PCF proceeds to establish a 5GS user plan path for optimized media routing.

## ---Start of the 3rd Change ---

## AE.4.2	At call setup

The P-CSCF serving the MO UE selects an IMS AGW on ground at call setup according to TS 23.334 [74]. The P-CSCFs serving the MO UE and MT UE proceed with the determination of whether UE-Satellite-UE communication is possible as described in clause AE.3.2. If the P-CSCF determines the activation of optimized media routing is possible, when deciding to use the optimized media routing (e.g. with consideration of whether early media is in progress or not), the P-CSCF releases the IMS AGW on ground if allocated and selects instead an IMS AGW on satellite according to TS 23.334 [74].

## ---End of the Change---