SA WG2 Meeting #167 	S2-2501687

# 17 - 21 February, 2025, Athens , GR														(was)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3886 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transition between S&F and normal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some clarification is needed for UE and NW behaviour under transition between SF and normal mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If a UE is in ECM-CONNECTED state with a satellite operating in S&F mode (i.e., the accessing eNB that is broadcasting the indication of operating in S&F Mode), for upcoming available feeder link, the MME on the satellite operating in S&F mode detaches the UE. If a UE is in ECM-CONNECTED state with a satellite operating in non-S&F mode (i.e., the accessing eNB that is not broadcasting the indication of operating in S&F Mode), for upcoming loss of the feeder link, the MME on the satellite operating in S&F mode detaches the UE. NOTE A: How the MME determines the feeder link availability upcoming is up to MME implementation. If a UE is registered state for S&F operation (i.e., the UE was provided the S&F capability during the attach) and in ECM-IDLE state, when detects the eNB of the camping cell that does not broadcast the indication of operating in S&F Mode, the UE initiates the attach procedure to the network. If a UE is registered state not for S&F operation (i.e., the UE is registered to TN or NTN not operating S&F) and in ECM-IDLE state, when detects the eNB of the camping cell that broadcasts the indication of operating in S&F Mode, the UE initiates the attach procedure to the network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear for the UE and NW behaviour when transfer between SF and normal |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# FIRST CHANGE

4.13.9.x	Transition between S&F mode and normal mode

If a UE is in ECM-CONNECTED state with a satellite operating in S&F mode (i.e., the accessing eNB that is broadcasting the indication of operating in S&F Mode), for upcoming available feeder link, the MME on the satellite operating in S&F mode detaches the UE.

If a UE is in ECM-CONNECTED state with a satellite operating in non-S&F mode (i.e., the accessing eNB that is not broadcasting the indication of operating in S&F Mode), for upcoming loss of the feeder link, the MME on the satellite operating in S&F mode detaches the UE.

NOTE A:	How the MME determines the upcoming feeder link availability is up to MME implementation.

If a UE is registered state for S&F operation (i.e., the UE was provided the S&F capability during the attach) and in ECM-IDLE state, when detects the eNB of the camping cell that does not broadcast the indication of operating in S&F Mode, the UE initiates the attach procedure to the network.

If a UE is registered state not for S&F operation (i.e., the UE is registered to TN or NTN not operating S&F) and in ECM-IDLE state, when detects the eNB of the camping cell that broadcasts the indication of operating in S&F Mode, the UE initiates the attach procedure to the network.

# End of CHANGEs