3GPP TSG-WG SA2 Meeting #167	S2-2501697

Athens, GR， 17 – 21 Feb 2025

Source:	China Mobile

Title:	Conclusions for KI#1 on the aggregation aspects

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusions for KI#1 on the aggregation aspects are proposed.

# 1. Introduction/Discussion

This contribution is prepared to update the conclusions for KI#1 on the aggregation aspect related to the EN below.

Editor's note:	How the aggregation can be done is FFS.

The AF or AIOTF decides whether to enable the aggregation per service operation.  For example, if the AF decides to inventory one category of goods, from the AF's perspective, it will know the estimated number of expected ambient IOT Devices.  Based on the estimated number, the AF can make the decision to enable the aggregation or not for the service operation.  Alternatively, AF sends the estimated number of expected ambient IOT Devices to AIOTF, and then the AIOTF can make the aggregation decision based on the number.

Besides, if the service operation needs to involve multiple AIOT Nodes, the AIOTF can aggregate the AIOT Data received from multiple AIOT Nodes.

Proposal 1: The AIOTF can determine whether to enable aggregation based on the aggregation indication parameter or the approximate number of AIOT devices from the AF per service operation, and the AIOTF can aggregate the AIOT Data received from multiple AIOT Nodes if needed.

How does AIOTF inform the AIOT RAN to perform the aggregation, AIOTF may transfer the assistant aggregation indication parameter to AIOT RAN, or AIOT RAN decide by itself.

Proposal 2: The AIOTF may provide the assistant aggregation indication to the AIOT RAN to assist in aggregating AIOT data received from AIOT device(s).

NOTE X: If the AIOTF does not provide the assistant aggregation indication, the aggregation in AIOT RAN is up to RAN implementation.

Typically, there are three options for AIOT RAN to perform aggregation:

Option 1: Response: [AIOT RAN1 (data from dev1, data from dev2, …)]

Option 2: Response: [AIOT RAN1 (data from dev1, reader 1),  (data from dev2 , reader 1), (data from dev3, reader 2)  …]

Option 3: Response: [AIOT RAN1 {Reader1 (data from dev1, data from dev2), Reader2 (data from dev3, …)}, …]

Comparison among three options:

Option 1: AIOTF can’t get the serving BS reader information for each responded ambient device

Option 2: AIOTF can get the serving BS reader information for each responded ambient device, but there are some issues with transportation efficiency

Option 3: AIOTF can get the serving BS reader information for each responded ambient device with efficient transportation.

In ambient IOT solutions, the AIOTF needs to know the last known BS reader information for each ambient IOT device in the following features:

- Optimize BS reader selection

- decide the appropriate device number per BS reader

If the data is aggregated per reader, AIOTF can quickly determine which devices responded through which readers. This improves data organization and possibly efficiently obtains the serving reader information for each responded ambient IOT device.

Proposal 3: the AIOTF can obtain the serving BS reader information for each responded ambient device in the aggregation operated by the AIOT RAN.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13.

FIRST CHANGE

## 8.1	Conclusion on Key Issue #1

### 8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

In this Release, normative work will take place for Topology 1 and no normative work will take place for Topology 2.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

1.	A new core network function is introduced to support Ambient IoT (e.g. AIOTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIOTF registers itself in the NRF with its NF profile, this is to enable the discovery of AIOTF instances e.g. by an NEF. The NF profile at least includes the AIOTF ID/address, NF type, information used to allow the NEF to discover AIOT instances, e.g. based on the target area information in AF request. Other details of the NF profile, if needed, will be completed in normative phase.

b.	For topology 1, the AIOTF selects AIOT RAN nodes and optionally a list of the BS readers. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

For topology 1, NGAP is terminated at an AIOT RAN node. An AIOT RAN node that supports one or more BS readers, may report its supported BS reader information e.g. BS reader IDs and their corresponding service areas. The OAM may configure the AIOTF with BS Reader information including e.g. their corresponding service area when the BS Readers are integrated in the network.

NOTE 2:	Whether RAN3 will specify BS reader information (e.g. BS reader ID, service area) reporting to AIOTF over NGAP depends on RAN3 decision and if so SA WG2 may include this feature in SA WG2 specifications.

NOTE 3:	If the request from the AIOTF to the AIOT RAN does not include a BS reader list, then AIOT RAN can use all available BS readers.

NOTE 4:	The BS reader’s service area needs to coordinate with RAN WG(s).

c.	The AIOTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIOTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

e.	The AIOTF may provide the following assistance information to AIoT RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

NOTE 5:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 6:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

NOTE 7:	Further assistance information can be added during the normative phase and in cooperation with other WGs if necessary.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

f.  The AIOTF can determine whether to enable aggregation based on the aggregation indication parameter or the approximate number of AIOT devices from the AF, and the AIOTF can aggregate the AIOT Data received from multiple AIOT Nodes if needed.

The AIOTF may provide the assistant aggregation indication to the AIOT RAN to assist in aggregating AIOT data received from AIOT device(s).

The AIOTF can obtain the serving BS reader information for each responded ambient device in the aggregation operated by the AIOT RAN.

NOTE X1: The AIoT service operations being aggregated from the AIOT devices share the same service transaction identifier.

NOTE X2: If the AIOTF does not provide the aggregation indication, the aggregation in AIOT RAN is up to RAN implementation.

g.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 8:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIOTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

5.	The AIOTF may store and manage the AIoT device related information (also known as device context information) locally that includes e.g. the AIOT device permanent ID, the last known reader information of the AIoT device. The last known reader information can be used to support the AIOTF to select the serving reader to forward the message towards the specific AIoT device(s).

NOTE 9:	The storage of security related information at the AIOTF is up to the coordination with SA WG3 later.

END OF CHANGES