3GPP TSG-WG SA2 Meeting #167	S2-2501713

# Athens, Greece, 17 – 21 February 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5351 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on EIF services to solve the Editor's notes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Editor's note: Whether there are other NFs invoking EIF Service is FFS. Regrads to the above EN, according to the clause 5.51.4 in TS 23.501, the PCF may make policy decisions based on energy related information. This energy related information should be provided from EIF. It is proposed to add the PCF as the consumer of EIF service in Table 5.2.28.1-1. - Editor's note: More detailed input/output parameters of the service operations are FFS. Regrads to the above EN, it is proposed that the AF could provide the reporting threshold for the threshold-based reporting. From the AF point of view, it may only need to be notified when the threshold is reached. This is beneficial to reduce the signalling and needs to be supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 5.2.28.1, the PCF is added as the consumer NF for EIF services to solve the Editor's note. - In clause 5.2.28.2.2, regards to the Editor's note, it is proposed that the AF may provide the reporting threshold for Neif_EventExposure _Subscribe. - Other editorial changes are proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Editor's notes are not solved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.28.1, 5.2.28.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.2.28.1	General

The following table shows the EIF Services and EIF Service Operations.

Table 5.2.28.1-1: NF services provided by the EIF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Neif_EventExposure | Subscribe | Subscribe/Notify | NEF, AF, PCF |
|  | Unsubscribe | Subscribe/Notify | NEF, AF, PCF |
|  | Notify | Subscribe/Notify | NEF, AF, PCF |

# * * * * Second change * * * *

5.2.28.2.2	Neif_EventExposure_Subscribe service operation

Service operation name: Neif_EventExposure _Subscribe

Description: The consumer subscribes to receive the energy consumption information from the network, or the subscription is updated if the same subscription is already existing in EIF.

Inputs, Required: NF ID, Target of Event Reporting as defined in clause 5.2.28.2.1, Event ID defined in clause 5.2.28.2.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1.

Inputs, Optional: Event Filter (s) associated with each Event ID, as defined in clause 5.2.28.2.1; Time period for information collection, reporting period and reporting frequency as described in clause 5.51 of TS 23.501 [2]. If the consumer NF is NEF/AF, the reporting threshold may be provided.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding energy related information report is included, if available.

# * * * * End of changes * * * *