3GPP TSG-WG SA2 Meeting #167	S2-2501718

# Athens, Greece, 17 – 21 February 2025					                  (revision of S2-2500771)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5999 | rev | 1 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on subscription information and policy control aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Editor's note: The indicator is expected to select an energy saving strategy for the UE in the system. Whether there are multiple values of the indicator is FFS. Regards to this EN, considering that the network may deploy different network energy saving actions, the value of the indicator could correspond to certain energy saving action. This would give the subscriber the flexibility to indicate which network energy saving action he would like to support in the subscription. Of cause, this would depend on the operator deployments. So it is proposed to add the descriptions to sovle this EN. - Editor's note: A notification for the energy related information, if any, should not be provided from the UDM/UDR. Whether the notification for the energy related information from the EIF is provided to the PCF and what are the notification details, are FFS. Regards to the above EN, the energy saving indicator is not enough for the PCF to make policy control. The PCF also need to take the energy consumption information of the UE into consideration to better decide the policies for network energy saving. So this paper proposes to add the notification from EIF to PCF to sovle this EN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 5.51.5, it is proposed to add the descriptions related with the energy saving indicators. - In clause 5.51.6, regards to the Editor's note, the paper proposes that the EIF notification to PCF is needed for better policy control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Editor's notes are not solved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.5, 5.51.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S2-2500771 |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.51.5	Subscription information aspects

The Subscription data for a UE may include an Energy Saving indicator that the UE is subject to network energy saving operation. The AMF receives the Energy Saving indicator in the UE Subscription data. The AMF forwards the Energy Saving indicator to the NG-RAN and to the PCF. The indicator is used by the PCF to select an energy saving strategy for the UE. Based on the operator’s deployment, the Energy Saving indicator may be single or multiple values and correspond to certain network energy saving action or strategy.

NOTE:	The details of Energy Saving indicator (e.g., encoding, exact number of the indicator) are defined in stage 3 specifications.

Editor's note:	Whether the indicator is provided to the NG-RAN and how it is used in the NG-RAN is FFS.

Editor's note:	Whether the Energy Saving indicator is also stored in the UDR is FFS.

5.51.6	Policy control aspects

During AM policy association establishment procedure, the AMF sends to the PCF the Energy Saving indicator as described in clause 5.51.5. The PCF may also subscribe the notification from EIF for energy consumtion information. Based on the Energy Saving indicator, EIF notification and operator policy, the PCF may make policy decisions reusing the existing parameters.

# * * * * End of changes * * * *