3GPP TSG-SA2 Meeting #167	S2-2501737

# 17 - 21 February, 2025, Athens, Greece	 							   		(Revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1587 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on downloading Avatar representation in UE centric rendering mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is not clear how the avatar representation is downloaded to the UE in UE entric rendering mode. This contribution proposes the following updates: - Add rendering mode in avatar media specifications to indicate the rendering mode when the application DC is established. - clarifies to download the representation to the MF using resource URL which can point to DC AS or BAR. - clarifies the representation to UE who requests the representation based on UE centric rendering mode. - This proposal also applies to network centric rendering mode, which can be aligned further. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add rendering mode in avatar media specifications to indicate the rendering mode when the application DC is established. - clarifies to download the representation to the MF using resource URL which can point to DC AS or BAR. - clarifies the representation to UE who requests the representation based on UE centric rendering mode. - Editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.3.2, AA.2.5.2.2, AC.11.1, AC.11.3.2.1, AC.11.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### AA.2.4.3	Nimsas_MediaControl Service

#### AA.2.4.3.1	General

Service description: This service enables the consumer to control IMS AS handling of media flow within an IMS session. The service can be used by the consumer after receiving and before responding to a Nimsas_SessionEventControl_Notify request.

#### AA.2.4.3.2	Nimsas_MediaControl_MediaInstruction service operation

Service operation name: Nimsas_MediaControl_MediaInstruction

Description: This operation provides instructions to the IMS AS for all media flows a consumer wants to control based on its policies for the received IMS session event, and that may require resource reservation in media resource e.g. MF.

For the case when a specific media flow needs to be terminated in MF media resource (i.e. termination of a Data Channel media descriptor offered by one of the IMS subscribers) or originated by the MF (i.e. origination of a data channel media descriptor to be offered towards one of the IMS subscribers), the consumer must provide a complete MF media specification including information required by MF to know how to terminate or originate the media flow.

Inputs, Required: Session ID, Media instruction set

Session ID specifies the IMS session for which the MediaInstruction operation applies.

Media instruction set includes a set of instructions for each media flow to control. Each instruction includes:

-	media ID: used by the producer (IMS AS) and the consumer for referencing purposes. The consumer reuses the media ID it received from the IMS AS for referencing the same media. This field will be null for instructions related to originating new media.

-	Media resource capability: Identify the Media Resource capabilities the Media instruction is intended for (e.g. DC, AR).

-	Media instruction: includes instructions to the producer (IMS AS) for handling the media. The following instructions are supported:

-	"TerminateMedia": Terminate the offered media descriptor of the UE in the mediaResource, i.e. this media descriptor will not be exposed to the other UE.

-	"OriginateMedia": Originate and offer a media descriptor from the mediaResource to the UE. The media ID representing the new media flow will be provided by the IMS AS in the response.

-	"TerminateAndOriginateMedia": Terminate the offered media flow in the mediaResource from the UE and originate the same media flow from the mediaResource to the other UE, i.e. the offered media descriptor of the originating UE will be replaced by the mediaResource provided media descriptor, which is sent towards the other UE.

-	"UpdateMedia": Update a media flow of the mediaResource previously allocated by the instructions "TerminateMedia", "OriginateMedia" and "TerminateAndOriginateMedia"

-	"DeleteMedia": Delete a media flow of the mediaResource previously allocated by the instructions "TerminateMedia", "OriginateMedia" and "TerminateAndOriginateMedia".

-	"RejectMedia": Remove an offered media flow, i.e. the offered media descriptor will be removed both from the offer sent to the remote UE and from the answer returned to the initiator of the offer.

-	DC Media Specification: Description of additional media specification information needed for data channel media stream from application layer, which includes:

-	Media proxy configuration (HTTP or UDP) applicable to the media flow.

-	MDC1/MDC2 media endpoint address of the application layer.

-	Replacement HTTP URL per stream ID allocated by the application layer representing the application list (e.g. graphical user interface) provided to the IMS subscriber via the MDC1 interface.

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

-	AR Media Specification: Description of additional media specification information needed for AR communication services from application layer, which includes:

-	Media Processing Specification: It specifies how the media stream should be processed.

-	Avatar Media Specification: Description of additional media specification information needed for avatar communication services from application layer, which includes:

-	Avatar representation Resource URL: URL that allocated by DC AS for MF to retrieve the avatar representation.

-	Rendering mode: identifies the mode of rendering avatar media, i.e. UE centric, network centric.

-	Media Processing Specification: It specifies how the media stream should be processed.

NOTE:	DC Media Specification and AR Media Specification are optional depending on the scenario and media type.

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: operation result indication.

Outputs, Optional: media resource information set:

This set includes entries corresponding to each instruction in the received media instruction set. Each entry in the set includes the media ID (same as received or new for new media) and includes the media type according to SDP. Each entry may optionally contain Avatar Media Specification, DC Media Specification and/or AR Media Specification. The DC Media Specification shall exist only with media type "DC" and contains additional media resource information, e.g. allocated media address for MDC2 interface when an offer must be provided to Data Channel Application Server. The AR Media Specification contains media processing instruction for MF on how to process the media. The Avatar Media Specification contains media processing instruction for MF on how to perform network rendering for avatar communication.

# * * * * Second change * * * *

#### AA.2.5.2.2	Nmf_MRM_Create service operation

Service operation name: Nmf_MRM_Create

Description: The consumer NF requests the MF to create a media context including one or multiple media terminations and reserve media resources for anchoring one or multiple media streams of Mb interface in each termination. The consumer NF may also include media application function specification information requested by the application layer, e.g. DCSF, to be applied on the media stream by the MF.

Inputs, Required: List of Media Termination Descriptors.

Each Media Termination Descriptor of the list includes:

-	Termination ID: A unique identity of the termination within the media context resource. The identity is allocated by the producer and must be set to value 'null' by the consumer when adding a termination to a context.

-	List of Media Stream Descriptors belonging to the Termination.

Each Media Stream Descriptor of the list includes:

-	Media ID: A unique identity of the media stream within the media context instance. The MediaID value is set by the consumer.

NOTE 1:	Different Termination IDs associated with the same Media ID represent the media stream flow for data forwarding.

-	Remote Mb Specification: Media Specification specifying SDP parameters of a remote media endpoint e.g. media stream IP address and ports allocated at the remote UE.

NOTE 2:	In the case of reserving termination media resource before receiving media negotiation information from the UE, Remote Mb Specification should be set to Null.

-	Media Function Specifications: Specification of the media function to be applied to the media stream as requested by the application layer, which includes:

-	Media Function Type: The value is set based on the type of the media function required, e.g. "DC media function type or AR media function type.

-	Media Function Description: Application specific media function description specifying how the media function shall handle and process the media stream. Data content depends on type of media function.

If the Media Function Type is "DC Media Function", then the Media Function Description incudes:

-	Media proxy configuration (HTTP or UDP) applicable to the media flow.

-	Remote MDC1/MDC2 media endpoint address.

-	Replacement HTTP URL per Stream ID allocated by the application layer representing the application list (e.g. graphical user interface) provided to the IMS subscriber via the MDC1 interface.

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Security Setup: This attribute identifies the security setup of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity

NOTE 3:	Some of the above attributes are optional depending on the scenario and proxy configuration.

If the Media Function Type is "AR Media Function", then the Media Function Description includes:

-	AR Media Specification: Description of additional media specification information needed for AR communication services from application layer, which includes:

-	Media Processing Specification: It specifies the how the media stream should be processed.

NOTE 4:	The detail of Media Processing Specification is defined by SA4 and needs to be further aligned.

-	Avatar Media Specification: Description of additional media specification information needed for avatar communication services from application layer, which includes:

-	Avatar representation Resource URL: URL that allocated by DC AS for MF to retrieve the avatar representation.

-	Rendering mode: identifies the mode of rendering avatar media, i.e. UE centric, network centric.

-	Media Processing Specification: It specifies how the media stream should be processed.

NOTE 5:	Avatar Media Specification are optional depending on the scenario.

Inputs, Optional: None.

Outputs, Required: Result indication for the entire list, Media Resource Context ID, List of allocated Media Termination Descriptor Resource information. For each Media Termination Descriptor; the Termination ID, and the list of media stream descriptors corresponding to the input list of media stream descriptor:

Media Resource Context ID uniquely identifies the media resources created by the MF. This identity shall be used as reference when updating or deleting the created media resource.

List of allocated Media Termination Descriptor Resource information includes one or multiple Media Termination Descriptor Resources allocated. The list will include same number of terminations, in the same order as provided in the request. Each Media Termination Descriptor Resource includes:

-	Termination ID: Unique identity allocated by the consumer. The identity shall be used when updating/deleting the Media Termination.

-	List of Media Stream Descriptor Resource information elements allocated within the media termination. The list will include same number of media streams, in the same order as provided in the request.

Media Stream Descriptor Resource information includes:

-	Media ID: Unique identity of the media stream within the media context instance allocated by the consumer. This is identical to the Media ID used in the input.

-	Local Mb Specification: Media specification specifying SDP parameters representing the media endpoint of the provider e.g. media stream IP address and port.

-	Media Application Function Output Description: Optional media application function description including information required by the application layer. The Media Application Function Output Description shall be relayed to the DCSF by the consumer NF. Data content depends on type of media function. If the Media type is "DC media function", the Media Application Function Output Description includes:

-	MDC1/MDC2 media point information (IP address/port number, etc.) which shall be relayed to the DCSF by the consumer NF.

Outputs, Optional: None.

# * * * * Third change * * * *

# AC.11	Support of Avatar Communication

## AC.11.1	General

This clause describes the enhancements to the IMS architecture supporting data channel services to additionally support Avatar communication services.

For Avatar communication over IMS data channel, the list of Avatar ID(s) is downloaded to the UE by following options:

-	Pre-configured in the UE: The Avatar ID List is provisioned or downloaded to the UE before a data channel for avatar call is setup. The Avatar Representations may be downloaded together. The UE and the BAR may interact by means out of the scope of 3GPP.

-	Through bootstrap data channel: The Avatar ID List is fetched by the DC AS from the BAR, and transferred from the DC AS to the DCSF, and downloaded to UE through bootstrap data channel.

-	Through application data channel: The Avatar ID List is fetched by the DC AS from the BAR, and downloaded to the UE through application data channel.

Avatar ID List contains the list of Avatar IDs that are identifying the Avatar Representation and/or the associated information. The associated information may include the information for the user selection in the avatar calls (e.g. the metadata associated with the Avatar such as thumbnails, URLs, the associated data channel applications).

Editor's note:	Whether the personal asset information (e.g. identities of accessories, garments) can be provided with Avatar IDs and Avatar Representation is FFS.

For downloading of Avatar ID List through bootstrap data channel, the DC AS URL(s) may be pre-configured in the DCSF, or fetched by DCSF. When the downloading of Avatar ID List is requested through bootstrap data channel, the DCSF requests DC AS to fetch the Avatar ID List from the BAR for the authorized user of the UE.

Editor's note:	How the DCSF fetches DC AS URLs is FFS.

The BAR and/or the DC AS may be deployed either in the trusted domain or untrusted domain of operator's network.

The Avatar Representation is identified by Avatar ID and is stored in the BAR. It is downloaded via application data channel for the avatar communication application using the Avatar Representation. After selection of Avatar ID to use in the avatar call from the Avatar ID list by the user of the UE, the Avatar Representation identified with the selected Avatar ID is provided from the BAR to MF directly or through the DC AS, using the Avatar representation resource URL as defined in clause AA.2.4.3.2. The Avatar Representation may be transferred to the local UE (if not locally available) and/or to the remote UE according to the UE cenrtic rendering mode via application data channel if needed.

The avatar media is rendered by processing the Avatar Representation with the animation data.

-	The avatar media rendering process may be performed by the sending UE, the receiving UE, or the network according to the rendering mode.

-	The animation data may be generated by the sending UE, the receiving UE, or the network (e.g. from audio, video, and/or metadata such as UE sensors collected data).

Editor's note:	Whether the Avatar Representation can be downloaded through bootstrap data channel is FFS.

# * * * * Fourth change * * * *

#### AC.11.3.2.1	Sending UE centric procedure

AC.11.3.2.1

Figure AC.11.3.2.1-1: Procedures of Sending UE centric IMS Avatar communication

Figure AC.11.3.2.1-1 depicts a typical call flow procedure of sending UE centric IMS Avatar communication. The main steps in the call flow are as follows:

1.	IMS session and bootstrap data channel have been established. UE1 downloads avatar communication application. The avatar ID list is downloaded as described in clause AC.11.3.1. Then UE1 chooses an avatar ID and runs the avatar communication application. If the avatar representation is locally available on the UE1, then step 2-6 can be skipped.

2.	P2A application data channel(s) are established between UE1 and DC AS which is used for avatar representation transmission. Avatar representation Resource URL is provided from DC AS to MF in this step. The rendering mode is set to 'UE centric' by the DC AS.

3.	UE1 requests Avatar representation from DC AS using avatar ID and UE1's identity (e.g., IMSI or MSISDN) through the established application data channel.

4-5.	The MF forwards the request to the Avatar representation Resource URL received in step 2 to retrieve Avatar representation.

-	If the URL points to the DC AS, the DC AS further request the Avatar representation from the corresponding BAR and returns it to the MF;

-	If the URL points to the BAR, the MF directly download the Avatar representation from the BAR.

The BAR should authenticate and authorize whether the UE1 is allowed to use the requested Avatar representation and  responses with avatar representation data if allowed.

6.	The MF sends the Avatar representation back to UE1 in the response to UE1’s request based on 'UE centric' rendering mode.

7.	UE1 performs avatar animation.

8.	UE1 transmits the animated video stream over RTP to UE2.

#### AC.11.3.2.2	Receiving UE centric procedure

Figure AC.11.3.2.2-1: Procedures of Receiving UE centric IMS Avatar communication

Figure AC.11.3.2.2-1 depicts a typical call flow procedure of receiving UE centric IMS Avatar communication. The main steps in the call flow are as follows:

1.	IMS session and bootstrap data channel have been established. UE1 downloads avatar communication application. The avatar ID list is downloaded as described in clause AC.11.3.1. Then UE1 chooses an avatar ID and runs the avatar communication application.

2.	P2A2P application data channel(s) are established between UE1, DC AS and UE2.  Avatar representation Resource URL is provided from DC AS to MF in this step. The rendering mode is set to 'UE centric' by the DC AS.

3.	UE1 decides to request peer UE to perform avatar animation based on its status such as power, signal, computing power, internal storage, etc.

4.	UE1 performs avatar animation negotiation with the DC AS and UE2. The negotiation includes usage of the UE1's avatar ID and the animation data types (e.g. text and/or facial expression) supported by UE1.

Editor's note:	The negotiation procedure needs to be further defined in SA WG4, and cooperation with SA WG4 is needed.

5.	UE2 requests avatar representation from DC AS using the avatar ID received from UE1 through the established application data channel in step 2.

6-7.	The MF forwards the request to the Avatar representation Resource URL received in step 2 to retrieve Avatar representation.

-	If the URL points to the DC AS, the DC AS further request the Avatar representation from the corresponding BAR and returns it to the MF;

-	If the URL points to the BAR, the MF directly download the Avatar representation from the BAR.

The BAR should authenticate and authorize whether the UE2 is allowed to use the requested Avatar representation and responses with UE1's avatar representation data is allowed. The BAR should authorize the UE2 download request based on the negotiation result from step 4.

NOTE:	How the DC AS notifies the negotiation result to the BAR is out of 3GPP scope.

8.	The MF sends the Avatar representation back to UE2 in the response to UE2’s request based on 'UE centric' rendering mode.

9.	UE1 may send metadata (e.g. facial expression) to UE2 through the established application data channel and audio/video through RTP channel between UE1 and UE2.

10.	UE2 performs avatar animation with the UE1's avatar representation based on the metadata received from UE1.

# * * * * End of changes * * * *