SA WG2 Meeting #S2-167	S2-2501749

17 - 21 February, 2025, Athens, Greece	(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6104 | rev | - | Current version: | 16.20.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | epoch for 5G Residence time calculation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The difference between TSi (ingress timestamping) and TSe (egress timestamping) is considered as the calculated residence time spent within the 5G system for this gPTP message expressed in 5GS time. The TSi and TSe are not absolute time, they are the time offset since the epoch. There are several well-known epochs, e.g. GPS epoch, PTP epoch, … The issue is, how to guarantee the DS-TT and NW-TT use the same epoch for the TSi/TSe calculation. As indicated in the discussion paper S2-250xxxx, there are two alternatives for normative clarification: 1) which epoch is used by DS-TT and NW-TT for TSi/Tse is configured by operator’s policy/configuration 2) Explicitly clarify, the epoch is used by DS-TT and NW-TT for TSi/Tse is GPS epoch, or PTP epoch. This CR provides both options. It depends on the meeting discussion to select one. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying epoch used by DS-TT and NW-TT for the TSi/TSe calculation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to guarantee the DS-TT and NW-TT use the same epoch for the TSi/TSe calculation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 5.27.1.2.2	Distribution of TSN grandmaster clock and time-stamping

The mechanisms for distribution of TSN GM clock and time-stamping described in this clause are according to IEEE Std 802.1AS [104].

NOTE 1:	It means Externally-observable behavior of the 5GS bridge needs to comply with IEEE Std 802.1AS [104].

The epoch used by DS-TT and NW-TT to create time-stamping (i.e. TSi and TSe) is locally configured by operator’s policy/configuration.

The epoch used by DS-TT and NW-TT to create time-stamping (i.e. TSi and TSe) is PTP/GPS epoch.

NOTE 2:	The epoch is the origin of the timescale of a time domain.

Upon reception of a downlink gPTP message the NW-TT makes an ingress timestamping (TSi) for each gPTP event (Sync) message and uses the cumulative rateRatio received inside the gPTP message payload (carried within Sync message for one-step operation or Follow_up message for two-step operation) to calculate the link delay from the upstream TSN node (gPTP entity) expressed in TSN GM time as specified in IEEE Std 802.1AS [104]. NW-TT then calculates the new cumulative rateRatio (i.e. the cumulative rateRatio of the 5GS) as specified in IEEE Std 802.1AS [104] and modifies the gPTP message payload (carried within Sync message for one-step operation or Follow_up message for two-step operation) as follows:

-	Adds the link delay from the upstream TSN node in TSN GM time to the correction field.

-	Replaces the cumulative rateRatio received from the upstream TSN node with the new cumulative rateRatio.

-	Adds TSi in the Suffix field of the gPTP packet as described in Annex H.

UPF then forwards the gPTP message from TSN network to the UEs via all PDU sessions terminating in this UPF that the UEs have established to the TSN network. All gPTP messages are transmitted on a QoS Flow that complies with the residence time upper bound requirement specified in IEEE Std 802.1AS [104].

NOTE 3:	The sum of the UE-DS-TT residence time and the PDB of the QoS Flow needs to be lower than the residence time upper bound requirement for a time-aware system specified in IEEE Std 802.1AS [104].

A UE receives the gPTP messages and forwards them to the DS-TT. The DS-TT then creates egress timestamping (TSe) for the gPTP event (Sync) messages for external TSN working domains. The difference between TSi and TSe is considered as the calculated residence time spent within the 5G system for this gPTP message expressed in 5GS time. The DS-TT then uses the rateRatio contained inside the gPTP message payload (carried within Sync message for one-step operation or Follow_up message for two-step operation) to convert the residence time spent within the 5GS in TSN GM time and modifies the payload of the gPTP message that it sends towards the downstream TSN node as follows:

-	Adds the calculated residence time expressed in TSN GM time to the correction field.

-	Removes Suffix field that contains TSi.

Before forwarding gPTP messages received from NW-TT to downstream TSN nodes, DS-TT also sets the sourcePortIdentity header information in the gPTP messages received from NW-TT based on the Time domain configuration table received in PMIC as follows: DS-TT selects the entry for which the domainNumber in the gPTP message received from NW-TT matches the domainNumber in the Time domain configuration table entry and then sets the sourcePortIdentity value in the gPTP messages to the portIdentity value of the selected entry.

END OF CHANGES