3GPP TSG-SA2 Meeting #167	S2-2501754

Athens, Greece, February 17 – 21, 2025	(revision of S2-2501317)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6020 | rev | 3 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolvement of EN in AF Requested modification of the Set of Network Slice(s) for a UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, [ZTE, Nokia, Ericsson] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_SliceSel |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are several EN remaining in clause 5.15.5.2.2a Editor's note: The value of Network Slice Replacement Type is FFS. Proposal: Remove this EN directly. The reason is that the AMF needs to differentiate the Rel-18 network slice replacement feature and Rel-19 feature. The details of value of the Network Slice Replacement Type will be defined in stage 3. Editor's note: Which PCRF to be used to request Notification on outcome of Network Slice Replacement request is FFS. Proposal: Propose to remove the EN directly It needs to be clarified that the AMF can update the Allowed NSSAI via the UCU procedure to include the Replaced S-NSSAI. This will also remove the Replaced S-NSSAI from the Rejected NSSAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve the remainging ENs There are other issues - Align the terminology: use the “terminate’ instead of “stop” Add clarification that when the AF terminiates the network slice replacement, the AMF updates the Allowed NSSAI to include the Replaced S-NSSAI In r03 Resolves editori's note on Configured NSSAI and clarifies URSP rule configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.4.1.1, 5.15.5.2.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

##### 5.15.4.1.1	UE Network Slice configuration

The Network Slice configuration information contains one or more Configured NSSAI(s). A Configured NSSAI may either be configured by a Serving PLMN and apply to the Serving PLMN, or may be a Default Configured NSSAI configured by the HPLMN and that applies to any PLMNs for which no specific Configured NSSAI has been provided to the UE. There is at most one Configured NSSAI per PLMN.

NOTE 1:	The value(s) used in the Default Configured NSSAI are expected to be commonly decided by all roaming partners, e.g. by the use of values standardized by 3GPP or other bodies.

The Default Configured NSSAI, if it is configured in the UE, is used by the UE in a Serving PLMN only if the UE has no Configured NSSAI for the Serving PLMN.

The Configured NSSAI of a PLMN may include S-NSSAIs that have standard values or PLMN-specific values.

The Configured NSSAI for the Serving PLMN includes the S-NSSAI values which can be used in the Serving PLMN and may be associated with mapping of each S-NSSAI of the Configured NSSAI to one or more corresponding HPLMN S-NSSAI values. The following applies:

-	In the non-roaming case and if the Serving PLMN ID is the same as the PLMN ID of the SUPI, the network shall not provide any mapped S-NSSAI to the UE with the Configured NSSAI.

-	In the non-roaming case and if the Serving PLMN ID is not the same as the PLMN ID of the SUPI, when the Configured NSSAI of Serving PLMN is provided to the UE the network shall provide the mapped S-NSSAI(s) of the HPLMN to the UE if different values are used for S-NSSAI in Serving PLMN and mapped S-NSSAI in HPLMN.

NOTE 2:	It is recommended that the Serving PLMN and the HPLMN use the same S-NSSAI values in this case.

-	In the roaming case, the AMF shall provide to the UE the mapping of each S-NSSAI of the Configured NSSAI for the Serving PLMN to the corresponding S-NSSAI values of the HPLMN when providing NSSAI information, as described in TS 24.501 [47].

A UE subscription may contain Network Slice Simultaneous Registration Group (NSSRG) information. If so, the UE configuration is performed as described in clause 5.15.12.2.

The UE may be pre-configured with a Default Configured NSSAI associated with the SUPI. The UE may be provisioned/updated by the HPLMN with a Default Configured NSSAI, determined by the UDM in the HPLMN, by using the UE Parameters Update via UDM Control Plane procedure defined in clause 4.20 of TS 23.502 [3]. If the HPLMN configures a UE with the Default Configured NSSAI, each S-NSSAI in the Default Configured NSSAI shall also be one of the Subscribed S-NSSAI(s). If the Subscribed S-NSSAI(s) which are also present in the Default Configured NSSAI are updated (i.e. a Subscribed S-NSSAI is added or removed from the UE subscription or a Subscribed S-NSSAI is added to (or removed from) the Default Configured NSSAI), the UDM should update the Default Configured NSSAI in the UE.

NOTE 2:	The Subscribed S-NSSAIs marked as default S-NSSAI are not required to be part of the Default Configured NSSAI if one is configured for the UE.

In the HPLMN, the S-NSSAIs in the Configured NSSAI provided as described in clause 5.15.4.2, at the time when they are provided to the UE, shall match the Subscribed S-NSSAIs for the UE except a case when Alternative S-NSSAI(s) that are not subscribed S-NSSAI(s) are temporarily used and provided to the UE in the Configured NSSAI as described in clause 5.15.19 or when Replaced S-NSSAI is replaced by AF request as described in clause 5.15.5.2.2a.

When the Subscribed S-NSSAI(s) are updated (i.e. some existing S-NSSAIs are removed and/or some new S-NSSAIs are added) and one or more are applicable to the Serving PLMN the UE is registered in, as described in clause 5.15.3, or when the associated mapping is updated the AMF shall update the UE with the Configured NSSAI for the Serving PLMN and/or Allowed NSSAI and Partially Allowed NSSAI and/or the associated mapping to HPLMN S-NSSAIs (see clause 5.15.4.2). When there is the need to update the Allowed NSSAI or Partially Allowed NSSAI, the AMF shall provide the UE with the new Allowed NSSAI or Partially Allowed NSSAI and the associated mapping to HPLMN S-NSSAIs, unless the AMF cannot determine the new Allowed NSSAI (e.g. all S-NSSAIs in the old Allowed NSSAI have been removed from the Subscribed S-NSSAIs), in which case the AMF shall not send any Allowed NSSAI to the UE but indicate to the UE to perform a Registration procedure. If the UE is in a CM-IDLE state, the AMF may trigger Network Triggered Service Request or wait until the UE is in a CM-CONNECTED state as described in clause 4.2.4.2, TS 23.502 [3].

When providing a Requested NSSAI to the network upon registration, the UE in a given PLMN only includes and uses S-NSSAIs applying to this PLMN. The mapping of S-NSSAIs of the Requested NSSAI to HPLMN S-NSSAIs may also be provided (see clause 5.15.4.1.2 for when this is needed). The S-NSSAIs in the Requested NSSAI are part of the Configured and/or Allowed NSSAIs applicable for this PLMN, when they are available. If the UE has received NSSRG information together with the Configured NSSAI, it only includes in the Requested NSSAI S-NSSAIs that all share a common NSSRG. If the UE has stored Pending NSSAI and the UE is still interested in the Pending NSSAI then all the S-NSSAIs in the Requested NSSAI and the Pending S-NSSAI shall share a common NSSRG. If no Configured NSSAI and Allowed NSSAI for the PLMN are available, the S-NSSAIs in the Requested NSSAI correspond to the Default Configured NSSAI, if configured in the UE. Upon successful completion of a UE's Registration procedure over an Access Type, the UE obtains from the AMF an Allowed NSSAI or Partially Allowed NSSAI for this Access Type, which includes one or more S-NSSAIs and, if needed (see clause 5.15.4.1.2 for when this is needed), their mapping to the HPLMN S-NSSAIs. These S-NSSAIs are valid for the current Registration Area and Access Type provided by the AMF the UE has registered with and can be used simultaneously by the UE (up to the maximum number of simultaneous Network Slice instances or PDU Sessions).

The UE might also obtain from the AMF, one or more rejected S-NSSAIs with cause and validity of rejection. An S-NSSAI may be rejected:

-	for the entire PLMN;

-	for the current Registration Area; or

-	partially in the current Registration Area. Such S-NSSAI rejected partially in the current Registration area is associated with a list of TAs where the S-NSSAI is not supported.

The AMF may also reject the use of an S-NSSAI due to congestion as described in clause 5.19.7.4.

While the UE remains RM-REGISTERED in the PLMN and regardless of the Access Type, the UE shall not re-attempt to register to an S-NSSAI rejected for the entire PLMN until this rejected S-NSSAI is deleted as specified below.

While the UE remains RM-REGISTERED in the PLMN, the UE shall not re-attempt to register to an S-NSSAI rejected in the current Registration Area until it moves out of the current Registration Area.

While the UE remains RM-REGISTERED in the PLMN, the UE shall not re-attempt to register to an S-NSSAI rejected partially in the RA until the UE moves into a TA which is not part of the list of TAs associated with the S-NSSAI.

NOTE 3:	The details and more cases of S-NSSAI rejection are described in TS 24.501 [47].

The S-NSSAIs that the UE provides in the Requested NSSAI which are neither in the Allowed NSSAI nor in the Partially Allowed NSSAI, nor provided as a rejected S-NSSAI, shall, by the UE, not be regarded as rejected, i.e. the UE may request to register these S-NSSAIs again next time the UE sends a Requested NSSAI.

The UE stores (S-)NSSAIs as follows:

-	When provisioned with a Configured NSSAI for a PLMN and/or a mapping of Configured NSSAI to HPLMN S-NSSAIs and possibly NSSRG information for each S-NSSAI in the Configured NSSAI (if applicable and supported by the UE), or when requested to remove the configuration due to network slicing subscription change, the UE shall:

-	replace any stored (old) Configured NSSAI for this PLMN with the new Configured NSSAI for this PLMN (if applicable); and

-	delete any stored associated mapping of this old Configured NSSAI for this PLMN to HPLMN S-NSSAIs and, if present and applicable, store the mapping of Configured NSSAI to HPLMN S-NSSAIs; and

-	delete any stored associated NSSRG information for each S-NSSAI of the Configured NSSAI and, if present, store the associated NSSRG information for each S-NSSAI of the Configured NSSAI; and

-	delete any stored rejected S-NSSAI for this PLMN;

-	keep the received Configured NSSAI for a PLMN (if applicable) and associated mapping to HPLMN S-NSSAIs (if applicable) and associated NSSRG information for each S-NSSAI of the Configured NSSAI (if applicable and supported by the UE) stored in the UE, even when registering in another PLMN, until a new Configured NSSAI for this PLMN and/or associated mapping are provisioned in the UE, or until the network slicing subscription changes, as described in clause 5.15.4.2. The number of Configured NSSAIs and associated mapping to be kept stored in the UE for PLMNs other than the HPLMN is up to UE implementation. A UE shall at least be capable of storing a Configured NSSAI for the serving PLMN including any necessary mapping of the Configured NSSAI for the Serving PLMN to HPLMN S-NSSAIs and the Default Configured NSSAI.

-	The Allowed NSSAI received in a Registration Accept message or a UE Configuration Update Command applies to a PLMN when at least a TAI of this PLMN is included in the RA/TAI list included in this Registration Accept message or UE Configuration Update Command. If the UE Configuration Update Command contains an Allowed NSSAI but not a TAI List, then the last received RA/TAI list applies for the decision on which PLMN(s) the Allowed NSSAI is applicable. If received, the Allowed NSSAI for a PLMN and Access Type and any associated mapping of this Allowed NSSAI to HPLMN S-NSSAIs shall be stored in the UE. The UE should store this Allowed NSSAI and any associated mapping of this Allowed NSSAI to HPLMN S-NSSAIs also when the UE is turned off, or until the network slicing subscription changes, as described in clause 5.15.4.2:

NOTE 4:	Whether the UE stores the Allowed NSSAI and any associated mapping of the Allowed NSSAI to HPLMN S-NSSAIs also when the UE is turned off is left to UE implementation.

-	When a new Allowed NSSAI for a PLMN and any associated mapping of the Allowed NSSAI to HPLMN S-NSSAIs are received over an Access Type, the UE shall:

-	replace any stored (old) Allowed NSSAI and any associated mapping for these PLMN and Access Type with this new Allowed NSSAI; and

-	delete any stored associated mapping of this old Allowed NSSAI for this PLMN to HPLMN S-NSSAIs and, if present, store the associated mapping of this new Allowed NSSAI to HPLMN S-NSSAIs;

-	If received, a Partially Allowed NSSAI received in a Registration Accept message or a UE Configuration Update Command message applies to the current Registration Area. The UE stores the Partially Allowed NSSAI in the same way as described for the Allowed NSSAI (see also clause 5.15.17).

-	If received, an S-NSSAI rejected for the entire PLMN shall be stored in the UE while RM-REGISTERED in this PLMN regardless of the Access Type or until it is deleted.

-	If received, an S-NSSAI rejected for the current Registration Area shall be stored in the UE while RM-REGISTERED until the UE moves out of the current Registration Area or until the S-NSSAI is deleted.

-	If received, an S-NSSAI rejected partially in the RA shall be stored in the UE while RM-REGISTERED until the UE moves out of the current Registration Area or until the S-NSSAI is deleted (see also clause 5.15.17).

NOTE 5:	The storage aspects of rejected S-NSSAIs are described in TS 24.501 [47].

-	If received, the Pending NSSAI shall be stored in the UE as described in TS 24.501 [47].

-	If received, the S-NSSAI validity time information shall be stored in the UE in the UE as described in TS 24.501 [47].

-	If received, the S-NSSAI location availability information shall be stored in the UE as described in TS 24.501 [47].

-	If received, the mapping of old S-NSSAI to the Alternative S-NSSAI and associated Access Type shall be stored in the UE as described in TS 24.501 [47].

UE configuration to guide UE selection of a N3IWF/TNGF that supports the S-NSSAIs needed by the UE is defined in clause 6.3.6 and clause 6.3.12 respectively.

* * * * Start of 2nd Change * * * *

5.15.5.2.2a	AF Requested modification of the Set of Network Slice(s) for a UE

In non-roaming cases, an authorized AF or an AF subject to authorization via the NEF, may request to replace a certain S-NSSAI (Replaced S-NSSAI) for a UE with another S-NSSAI (Alternative S-NSSAI) which is part of the UE subscription. The request in either case is sent to the PCF for the UE handling the impacted subscriber. Similarly, an authorized AF or an AF subject to authorization via the NEF, may request to terminate the replacement then request to use the Replaced S-NSSAI instead of the Alternative S-NSSAI for a UE again. The AF is notified on the outcome of the AF requested Network Slice Replacement.

Only a single AF can request the modification of the Set of Network Slice(s) for one replaced S-NSSAI.

NOTE 1:	It is assumed that in the area where the AF requested Network Slice Replacement feature described in this clause is expected to operate, the AMFs supports AF requested Network Slice Replacement notifications handling from the PCF. How the AF obtains the S-NSSAI to be replaced and the Alternative S-NSSAI is out of scope of this specification.

The PCF for the UE based on operator policies considers the AF request. Consequently the PCF for the UE triggers the Network Slice Replacement at the AMF as defined in TS 23.503 [45]. The PCF sends Slice replacement management in the access and mobility management policies to the AMF per implicitly subscription, as described in TS 23.503 [45]. The Slice replacement management policy includes the Replaced S-NSSAI and the corresponding Alternative S-NSSAI as provided in the AF request, PCRT to request Notification on outcome of Network Slice Replacement request, together with a Network Slice Replacement Type. The Network Slice Replacement Type indicates that the Network Slice Replacement is requested by an AF. The AMF handles the procedure as following:

-	The AMF shall not handle the Network Slice Replacement procedure as described in clause 5.15.19.

-	If the Alternative S-NSSAI is not within the UE subscription, or is not supported in the UE Registration Area, the AMF shall notify the PCF that the Network Slice Replacement is not triggered with a cause value. The PCF further notify the AF.

NOTE 2:	The AMF may update the UE Registration Area to ensure that the Alternative S-NSSAI is supported in the UE Registration Area.

-	If the Alternative S-NSSAI is within the UE subscription and is supported in the UE Registration Area, the AMF includes the Alternative S-NSSAI in the Allowed NSSAI (if not present).

-	The AMF removes the Replaced S-NSSAI from the Allowed NSSAI and removes the Replaced S-NSSAI from the Configured by a UE Configuration Update procedure.

-	The AMF and UE release the PDU sessions that were using the Replaced S-NSSAI.

-	The AMF stores in the UE context the access and mobility management policy information as received from PCF and transfers to new AMF during inter AMF mobility.

-	After performing the above steps, the AMF reports to the PCF the outcome of the network slice replacement (e.g. the S-NSSAI has been replaced by an Alternative S-NSSAI) as defined in TS 23.503 [45].

NOTE 3:	Operator policies in the PCF ensures that the UE is configured with a URSP rule that contains the Alternative S-NSSAI to establish a new PDU Session. The precedence value of RSD with the Replaced S-NSSAI should be lower than the precedence value of RSD with the Alternative S-NSSAI, which enables the UE to prioritize using the Replaced S-NSSAI when both Replaced S-NSSAI and Alternative S-NSSAI are included in the Allowed NSSAI.

The trusted AF or a NEF may send request to PCF for the UE to terminate the Network Slice Replacement. In this case the PCF for the UE sends access and mobility management policies to the AMF, as described in TS 23.503 [45]. The AMF removes the Alternative S-NSSAI from the Allowed NSSAI and adds the Replaced S-NSSAI to the Configured NSSAI by a UE Configuration Update procedure. The UE may request to register the Replaced S-NSSAIs again next time the UE sends a Requested NSSAI.

* * * * End of Changes * * * *