3GPP TSG-SA2 Meeting #167	S2-2501757

Athens, Greece, February 17 – 21, 2025

Title:	[Draft] LS on NR Femto cell shared by PLMN and PNI-NPN

Response to:	-

Release:	Rel-19

Work Item:	5G_Femto, NR_WAB_5GFemto-Core

Source:	SA2

To:	RAN3

Cc:	-

Contact person:	Myungjune Youn

m.youn@lge.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

Overall description

SA2 discussed access control description captured in RAN3 BL CR to TS 38.300 (R3-247936).

- The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG. - The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. - The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

Among above three options, second option requires further impact in stage-2 and stage-3 specifications because current specifications do not allow a UE to access a CAG cell when the UE does not support CAG e.g. as described below.

// From TS 24.501 ##### 5.5.1.2.8 Abnormal cases on the network side The following abnormal cases can be identified: …… j) Based on operator policy, if the initial registration request from a UE not supporting CAG is rejected due to CAG restrictions, the network shall reject the initial registration with a 5GMM cause value other than the 5GMM cause #76 (Not authorized for this CAG or authorized for CAG cells only). ……

Before proceeding further work, SA2 kindly asks RAN3 to provide answers for the following questions.

Q1.	How the AMF knows whether a UE is accessing 5GC via a cell shared by PLMN and PNI-NPN?

In EPS, CSG is used for access control of Femto cell (HeNB) and there is a hybrid cell, which is similar with an NR Femto cell shared by PLMN and PNI-NPN. When a UE accesses via hybrid cell, based on subscription of the UE, an MME indicates whether the UE is a CSG member to RAN and S-GW/P-GW/PCRF for differentiated treatment and charging between CSG and non-CSG members.

From SA2 point of view, an NR Femto cell shared by PLMN and PNI-NPN needs to support differentiated handling between a CAG supporting UE and a CAG non-supporting UE. Otherwise, it seems that there is no difference between a normal PLMN cell and an NR Femto cell shared by PLMN and PNI-NPN.

Q2.	SA2 would like to ask RAN3 opinion about differentiated handling between a CAG supporting UE and a CAG non-supporting UE in an NR Femto cell shared by PLMN and PNI-NPN.

Actions:

To RAN3:

SA2 kindly asks RAN3 to provide answers for above questions.

Date of Next TSG-SA2 Meetings:

SA WG2#168	07 – 11, April 2025	Goteborg, Sweden

SA WG2#169	19 – 23, May 2025	Fukuoka, Japan