SA WG2 Meeting #167	S2-2501760

Athens, Greece, 17-21 February, 2025

# Title:	AIoT Device Information Configure Service

# Source:	InterDigital Inc.

# Document for:	Approval

Agenda Item:	19.14.2

Abstract:	Proposal for additional NEF service for AIoT device information configuration.

1. Discussion

In S2-2500734, NEF services for normal AIoT service operations (Inventory, Command, Notify) have been proposed. In addtion to those services, it is also necessary to provide the NEF service that allows the AF to configure the device information (i.e. device profile data such as device identifier, device location, device frequency band, etc) in the 5GC (i.e. in AIoT UDR).

2. Proposal

It is proposed to adopt the proposed text for Clause 7.4 “NEF services” in the draft TS 23.xyz.

* * * * First Change (All new text) * * * *

## 7.4	NEF services

### 7.4.1	General

In addition to those defined in TS 23.501 [3] clause 7.2.8 and TS 23.502 [4] clause 5.2.6, the following table illustrates additional NEF services to support AIoT.

Table 7.4.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AIoTDeviceInfoConfigure | Insert | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |

### 7.4.2	Nnef_AIoTDeviceInfoConfigure

Service operation name: Nnef_AIoTDeviceInfoConfigure

Description: The consumer configures AIoT device information in the AIoT UDR or AIOTF via the NEF.

Input, Required: AF Identifier, AIoT device identifier(s).

Input, Optional: AIoT device status, AIoT device location, AIoT device frequency band, AIoT device memory capcity.

Output, Required: Operation execution result indication.

Output, Optional: None.

* * * * End of Change * * * *