3GPP TSG-WG SA2 Meeting #167	S2-2501762

17th – 21st February 2025, Athens, Greece	(revision of 2500585)

Source:	Lenovo

Title:	KI#1 Addressing EN on AIoT service responses aggregation

Document for:	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to resolve the EN for the general part of the KI#1 conclusions, especially on the AIoT service responses aggregation.

1	Discussion

This paper mainly focuses on solving the remaining ENs in the General part for KI#1 conclusions, i.e., the following ones on aggregation.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor's note:	How the aggregation can be done is FFS.

1] For the first Editor’s note, the coordination with the RAN WG is needed in the future, and this issue can be further studied and revisited when the topology 2 study restarts. So, as for now, this EN can become a NOTE as below:

NOTE: For RRC based solution of topology 2, whether the aggregation can be performed by the RAN can be revisited and studied later, and coordination with RAN WGs is needed.

2] For the second Editor’s note about how to perform the aggregation, after the discussion and modification during the last SA2#166 ADH E-Meeting, some observations can be concluded as follows from the almost agreed paper (S2-2500463):

Observation 1: Some of the companies believe that the aggregation should be done based on the AIOTF and AIoT reader implementation, while some others believe that it is essential that the AIoT AF provides some aggregation assistance information, or control information, or guidance to the AIOTF and AIoT reader, in order to guide how to perform the aggregation.

For example, the aggregation assistance information can include time/period based aggregation (aggregate the responses for a duration), group of devices based aggregation (aggregate the responses for a maximum number of devices, e.g., 5,000), etc.

Proposal 1: It is suggested that the AIOTF, AIoT RAN and BS reader can perform the aggregation either based on its own implementation, or based on the aggregation assistance information provided by the AIoT AF.

Observation 2: The basic principle for AIoT service operation responses aggregation is that the responses being aggregated, shall share the same correlation identifier that is used to identify the AIoT service and response. The correlation identifier may be in the form of service identifier, or task identifier and so on.

Proposal 2: The AIoT service operation responses being aggregated from the AIoT devices shall share the same Correlation identifier.

With the discussion above, these two Editor’s notes can be removed accordingly.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-13.

# * * * * First change * * * *

8.1	Conclusion on Key Issue #1

### 8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

In this Release, normative work will take place for Topology 1 and no normative work will take place for Topology 2.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

1.	A new core network function is introduced to support Ambient IoT (e.g. AIOTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIOTF registers itself in the NRF with its NF profile, this is to enable the discovery of AIOTF instances e.g. by an NEF. The NF profile at least includes the AIOTF ID/address, NF type, information used to allow the NEF to discover AIOT instances, e.g. based on the target area information in AF request. Other details of the NF profile, if needed, will be completed in normative phase.

b.	For topology 1, the AIOTF selects AIOT RAN nodes and optionally a list of the BS readers. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

For topology 1, NGAP is terminated at an AIOT RAN node. An AIOT RAN node that supports one or more BS readers, may report its supported BS reader information e.g. BS reader IDs and their corresponding service areas. The OAM may configure the AIOTF with BS Reader information including e.g. their corresponding service area when the BS Readers are integrated in the network.

NOTE 2:	Whether RAN3 will specify BS reader information (e.g. BS reader ID, service area) reporting to AIOTF over NGAP depends on RAN3 decision and if so SA WG2 may include this feature in SA WG2 specifications.

NOTE 3:	If the request from the AIOTF to the AIOT RAN does not include a BS reader list, then AIOT RAN can use all available BS readers.

NOTE 4:	The BS reader’s service area needs to coordinate with RAN WG(s).

c.	The AIOTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIOTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF either based on the aggregation assistance information provided by the AIOT AF, or determined locally based on operator policies and configuration, or perform based on its own implementation. The AIOTF may further provide the aggregation assistance information to the AIOT RAN and/or BS reader on how to perform the aggregation of the service operation responses from the AIoT devices, which may include, for example:

-  Aggregation per time window;

-  Aggregation per number of AIoT devices (e.g. for inventory, threshold for the maximum number of AIoT Device(s) required for reporting)

-  Aggregation per service area of the AIoT RAN and/or BS reader.

-  Aggregation per data type stored in the AIoT devices, e.g., temperature, humidity.

-  Any combination of the above.

NOTE X: It is recommended that the AIOTF, the AIOT RAN, and the BS reader perform the aggregation based on the aggregation assistance information (if provided and available). If not, the aggregation will be performed based on its own implementation.

NOTE Y: The AIoT service operation responses being aggregated from the AIoT devices shall share the same Correlation identifier.

NOTE Z: For RRC based solution of topology 2, whether the aggregation can be performed by the NG-RAN (AIoT enabled) will be revisited and studied later, and coordination with RAN WGs is needed.

e.	The AIOTF may provide the following assistance information to AIoT RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

NOTE 5:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 6:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

NOTE 7:	Further assistance information can be added during the normative phase and in cooperation with other WGs if necessary.

.

f.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 8:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIOTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

5.	The AIOTF may store and manage the AIoT device related information (also known as device context information) locally that includes e.g. the AIOT device permanent ID, the last known reader information of the AIoT device. The last known reader information can be used to support the AIOTF to select the serving reader to forward the message towards the specific AIoT device(s).

NOTE 9:	The storage of security related information at the AIOTF is up to the coordination with SA WG3 later.

# * * * * End of changes * * * *