3GPP TSG-WG SA2 Meeting #167	S2-2501763

17th – 21st February 2025, Athens, Greece	(revision of 25XXXXX)

Source:	Lenovo

Title:	pCR to TS 23.xyz: AIOTF Functionality

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to add the descriptions of the functional entities in the clause 4.5 of the TS 23.xyz for Ambient IoT.

- Introduction

This paper is prepared for the normative work of Ambient IoT, to add the descriptions about the functional descriptions of the AIOTF. The texts are based on the conclusions captured in the TR 23.700-13 V1.3.0.

2. Text Proposal

It is proposed to capture the following changes to 3GPP TS 23.xyz for Ambient IoT.

# * * * * First change * * * *

4.5	Functional Entities

4.5.3	AIOTF

The AIOTF supports the following functions:

1)	Communicate with the AIoT device(s) via the AIoT NAS layer, that supports inventory response and command request and response.

2)	Connect with AIoT RAN via direct interface, or via an AMF through indirect interface.

3)	Register itself in the NRF with its NF profile, including at least the AIOTF instance ID, NF type, FQDN or IP address of AIOTF, the location information or the serving area information of the AIOTF, available data types of the AIoT devices within its serving area, available AIoT device information and its associated serving AIoT reader information within its serving area, AIoT service customer information (i.e. to indicate that the AIOTF is used for a dedicated service customer, e.g. a name like DNN may be used), etc.

4)	Receive an AIoT service request from the AF and authorize the AF request for Ambient IoT service based on the AF subscription data stored in the UDM, as described in the clause 5.6. The AIOTF sends a response to the AF to inform the result of the AIoT service request and the AIOTF stores a context for the AIoT service request from the AF until the service operation is completed. If the AF has requested periodic AIoT service operation (i.e., inventory and read), the AIOTF shall store the AIoT service request context in order to create the periodic service requests to the AIoT RAN as long as the periodic service operations shall be performed, and sends the periodic service responses to the AF, based on the service request context stored locally.

5)	Select the AIoT RAN or BS reader to perform the Ambient IoT service as described in clause 5.3, and trigger the AIOT RAN or BS Readers to perform AIoT service operations towards the AIoT Devices(s) by sending the service request to AIoT RAN or BS readers.

6)	Construct a correlation identifier to correlate the Ambient IoT service response to the service request.

7)	Determine and provide the assistance information to the AIoT RAN to perform AIoT services, as described in the clause 5.4.

8)	Perform the AIoT service operation responses (received from the AIoT devices) aggregation either based on the implementation, or based on the aggregation assistance information, which is either provided by the AF or based on local configuration and policies, and send the aggregated service results to AF. The AIOTF may further send the aggregation assistance information to the AIOT RAN and BS reader.

9)	Store and manage the AIoT device related information (also known as AIoT device context information), including e.g., the AIoT device permanent ID, the last known reader information of the AIoT device. AIOTF can use the last known reader information to select the serving reader to forward the message towards the specific AIoT devices.

NOTE X: The storage of security related information at the AIOTF is up to the coordination with SA WG3 later.

10)	Retrieve the AIoT device profile that includes the AIoT device subscription data from the entity storing the AIoT device profile, using device ID, local configuration, and 3rd party related context.

11)	Perform the Ambient IoT Device ID validation by interacting with the Authentication server (within operator network or 3rd party).

NOTE Y: The involvement of the AIOTF in the authentication procedure of the AIoT devices is up to the coordination with SA WG3.

# * * * * End of changes * * * *