3GPP TSG-WG SA2 Meeting #167	S2-2501764

17th – 21st February 2025, Athens, Greece	(revision of 25XXXXX)

Source:	Lenovo

Title:	pCR to TS 23.xyz: AF authorization to the Ambient IoT Services

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to add the support of AF Authorization to Ambient IoT Services for 3rd party AF.

1	Discussion

This paper aims to capture the support of AF authorization to the Ambient IoT services, based on the latest agreed conclusion principles in the TR 23.700-13.

2. Text Proposal

It is proposed to capture the following changes to TS 23.xyz for Ambient IoT.

# * * * * First change * * * *

5.6	AF authorization to the Ambient IoT Services

The information/data needed to support the authorization of the AF for performing the Ambient IoT service is stored as the subscription data for 3rd party AF in the UDM within the 5GC network.

The items within the subscription data for 3rd party AF on Ambient IoT service include at least the AF ID (that is used to index and identify the AF) and the following:

- Allowed service operations for the AF=> Indicate the allowed service operation(s) for the specific AF, e.g., inventory, command (read, write, permanently disable).

- Allowed targeting area=> Indicate the allowed area for the specific AF to perform the services.

- Allowed target AIoT devices=> Indicate the allowed AIoT devices for the specific AF to perform the AIoT services.

The authorization to the AF for the Ambient IoT services mainly includes two parts:

- NEF authorizes the Ambient IoT service requests from the AF based on the service layer agreement (SLA) between the 3rd party AF and the 5GS, and the local operator policy.

- AIOTF authorizes the Ambient IoT service requests from the AF via NEF, by checking and optionally retrieving the subscription data stored in the UDM, using at least the AF ID, that includes:

- check whether the AF is authorized for the requested service operation.

- check whether the AF is authorized for the requested target area.

- check whether the AF is authorized for the requested target AIoT device(s).

If the AIOTF determines that the AF service request is not allowed based on the AF authorization information stored in the UDM, the AIOTF rejects the AIoT service request from the AF.

# * * * * End of changes * * * *