3GPP TSG-WG SA2 Meeting #167	S2-2501766

17th – 21st February 2025, Athens, Greece	(revision of 25XXXXX)

Source:	Lenovo

Title:	pCR to TS 23.xyz: AF and NEF Functionality

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to add the descriptions of the AF and NEF functionality in the TS 23.xyz regarding the Ambient IoT service.

1	Discussion

This paper intends to document the standard texts based on the agreed conclusion principles in TR 23.700-13, regarding the functional descriptions about the NEF, AF and NRF.

2. Text Proposal

It is proposed to capture the following changes to TS 23.xyz for Ambient IoT.

# * * * * First change * * * *

4.5.4	NEF

In addition to the functions defined in clause 6.2.5 of TS 23.501 [x], the NEF further supports the following functions about Ambient IoT service:

- Support the interaction with AF for the AIoT services, including inventory, command (read, write and permanently disable), e.g., receiving the service request from the AF and exposing the service response, and other AIoT service related information to the AF, e.g., AIoT reader information.

- Select the serving AIOTF entity for the AIoT service request based on the information provided by the AF, including e.g., target area, AIoT reader information.

- Send the Ambient IoT service request to the selected AIOTF and receives the service responses from the AIOTF.

4.5.5	AF

In addition to the functions defined in clause 6.2.10 of TS 23.501 [x], the AF further supports the following functions about Ambient IoT service:

- Support the interaction with 5GS for the AIoT services, including inventory, command (read, write and permanently disable).

# * * * * End of changes * * * *