SA WG2 Meeting #167	S2-2501770

# Athens, Greece, February 17 – 21, 2025                        (revision of S2-25xxxxx)

Source: 	CATT

Title: 	pCR to TS 23.xyz: Procedure for Inventory

Document for: 	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: This paper introduces the Procedure for Inventory to TS 23.xyz.

1	Proposal

Based on TR 23.700-13 conclusions, it is proposed to introduce the Procedure for Inventory to TS 23.xyz.

# * * * * First change (all text new)* * * *

5.6	Procedure for Inventory

Depicted in Figure 5.6-1 is the procedure for Inventory.

Figure 5.6-1: Procedure for Inventory

1. An AIOT RAN needs to be authorized before it performs Inventory Service. The AIOT RAN reports AIoT support capability to AMF and is authorized by AMF during NG setup procedure.

2. AF sends AIoT Inventory Service Request message to NEF, and the Request message includes Device filter info and Area info, etc.

3. The NEF authorizes the AF request whether the AF is authorized to perform the Inventory Service.

4. The NEF discovers and selects AIOTF based on the Area info received from the AF.

5. The NEF sends AIoT Inventory Service Request message to the AIOTF, and the Request message includes Device filter info and Area info, etc.

6. The AIOTF discovers and selects AIOT RAN based on the Area info received from the NEF.

7. The AIOTF sends AIoT Inventory Service Request message to the AIOT RAN, and the Request message includes Device filter info, etc.

8. The AIOT RAN sends Initial Trigger Message (Device filter info) to AIoT Device.

NOTE 1:	The step 8 is to be defined by RAN2.

9. The AIoT Device compares the received Device filter info with its Device ID.

10. If the AIoT Device matches the Device filter info, the AIoT Device performs Random Access procedure.

NOTE 2:	The step 10 is to be defined by RAN2.

11. The AIoT Device sends AIoT Inventory Service Response NAS message to the AIOTF via the AIOT RAN and AMF.

12. The authentication and authorization procedure is performed for AIoT Device.

NOTE 3:	The step 12 is to be defined by SA3.

13-14. The AIOTF sends AIoT Inventory Service Response to the AF via the NEF.

# * * * * End of Changes * * * *