3GPP SA WG2 Meeting #167	S2-25xxxxx

Greece, Athens, 17 - 21 February, 2025

Source:	KDDI

Title:	Draft CRs for efficient authentication of IMS resiliency

Document for:	Discussion

Agenda Item:	30.8

Work Item / Release:	T.B.D / Rel-20

Abstract: This paper provides draft CRs based on the potential solutions for efficient authentication of IMS resiliency.

1. Introduction

The contribution provides draft CRs based on the potential solutions for efficient authentication of of IMS resiliency discussion paper.

2. Draft CRs for efficient authentication of IMS resiliency

The additional updates are highlighted in blue.

2.1	Solution #1,2: Efficient USIM authentication

The parts enclosed in red in the table below are introduced in this clause.

2.1.1	UDM > AMF > UE: Procedure enhancements for TS 23.501 (SA2)

Adding following new clause to clause 5.16.3 of TS 23.501:

-------------------------------------

5.16.3.x (new)	S-CSCF name delivery

For the UE registration procedures, the UDM may check the UE registration status of the IMS. If already registered, the UDM shall retrieve the registered S-CSCF name for the UE, process the registered S-CSCF name using one of the following security methods and send it to the AMF.

- Opt.1: Message authentication (e.g. HMAC, CMAC).

- Opt.2: Encryption.

- NOTE:	 Other security methods are FFS.

If the AMF receives the registered S-CSCF name in Step 14a, the AMF shall include it in the Registration Accept message.

-------------------------------------

Adding following description to clause 5.16.3.1:

-------------------------------------

5.16.3	IMS support

5.16.3.1	General

IP-Connectivity Access Network specific concepts when using 5GS to access IMS can be found in TS 23.228 [15].

5GS supports IMS with the following functionality:

-	Indication toward the UE if IMS voice over PS session is supported.

-	Capability to transport the P-CSCF address(es) to UE.

-	Paging Policy Differentiation for IMS as defined in TS 23.228 [15].

-	IMS emergency service as defined in TS 23.167 [18].

-	Domain selection for UE originating sessions.

-	Terminating domain selection for IMS voice.

-	Support of P-CSCF restoration procedure (clause 5.16.3.9).

-	NRF based P-CSCF discovery (clause 5.16.3.11).

NOTE:	The NRF based P-CSCF discovery has no impact on the UE, i.e. the UE does not need to know how P-CSCF IP address(es) is discovered in the network.

-	NRF based HSS discovery (clause 5.16.3.12).

- Capability to transport the S-CSCF name to UE (clause 5.16.3.x).

-------------------------------------

2.1.2	UDM > AMF > UE: Procedure and IE enhancements for TS 23.502 (SA2)

Adding the following procedures to clause 4.2.2.2.2 (General Registration) of TS 23.502:

-------------------------------------

- Step 14a: The UDM may check the UE registration status of the IMS (S-CSCF). If already registered, the UDM shall retrieve the registered S-CSCF name for the UE, processe the registered S-CSCF name with the designated security method such as message authentication (e.g. HMAC, CMAC) and encryption and send it to the AMF with Nudm_UECM_Registration Response.

- Step 21: If the AMF receives the registered S-CSCF name in Step 14a, the AMF shall include it in the Registration Accept message.

-------------------------------------

Adding a registered S-CSCF name to UE with Registration Accept in step 21 of clause 4.2.2.2.2 (General Registration).

-------------------------------------

21. New AMF to UE: Registration Accept (5G-GUTI, Registration Area, [Mobility restrictions], [PDU Session status], [Allowed NSSAI], [Mapping Of Allowed NSSAI], [Partially Allowed NSSAI], [Mapping Of Partially Allowed NSSAI], [TAI List for S-NSSAIs in Partially Allowed NSSAI], [Configured NSSAI for the Serving PLMN], [Mapping Of Configured NSSAI], [NSSRG Information], [NSAG Information], [rejected S-NSSAIs], [TAI List for any rejected S-NSSAI Partially in the RA], [Pending NSSAI], [Mapping Of Pending NSSAI], [Periodic Registration Update timer], [Active Time], [Strictly Periodic Registration Timer Indication], [LADN Information], [MICO Indication], [IMS Voice over PS session supported Indication], [Emergency Service Support indicator], [Accepted DRX parameters for E-UTRA and NR], [Accepted DRX parameters for NB-IoT], [extended idle mode DRX parameters], [Paging Time Window], [Network support of Interworking without N26], [Access Stratum Connection Establishment NSSAI Inclusion Mode], [Network Slicing Subscription Change Indication], [Operator-defined access category definitions], [List of equivalent PLMNs], [Enhanced Coverage Restricted information], [Supported Network Behaviour], [Service Gap Time], [PLMN-assigned UE Radio Capability ID], [PLMN-assigned UE Radio Capability ID deletion], [WUS Assistance Information], [AMF PEIPS Assistance Information], [Truncated 5G-S-TMSI Configuration], [Connection Release Supported], [Paging Cause Indication for Voice Service Supported], [Paging Restriction Supported], [Reject Paging Request Supported], [Paging Restriction Information acceptance / rejection], ["List of PLMN(s) to be used in Disaster Condition"], [Disaster Roaming wait range information], [Disaster Return wait range information], [Forbidden TAI(s)], [List of equivalent SNPNs], [Registered NID], [Unavailability Period Support], [MBSR authorization information], [Return To Coverage Notification Not Required], [Unavailability Period Duration], [Start of Unavailability Period], [S-NSSAI location availability information], [Mapping Of Alternative NSSAI], [Slice Usage Policy], [Maximum Time Offset], [S-CSCF name]).

2.1.3	UDM > AMF: Procedure and IE enhancements for TS29.503 (UDM Services; Stage 3) (CT4)

Adding the following procedures to step 2a of clause 5.3.2.2.2 (AMF registration for 3GPP access) of TS 23.503:

-------------------------------------

5.3.2.2.2	AMF registration for 3GPP access

Figure 5.3.2.2.2-1 shows a scenario where the AMF sends a request to the UDM to update the AMF registration information for 3GPP access (see also 3GPP TS 23.502 [3] figure 4.2.2.2.2-1 step 14). The request contains the UE's identity (/{ueId}) which shall be a SUPI and the AMF Registration Information for 3GPP access.

Figure 5.3.2.2.2-1: AMF registering for 3GPP access

2a.	On success, the UDM updates the Amf3GppAccessRegistration resource by replacing it with the received resource information, and responds with "200 OK" or "204 No Content".

UDM shall invoke the Deregistration Notification service operation towards the old AMF using the callback URI provided by the old AMF.

When AMF indicates there are no ongoing event subscriptions (e.g. during UE initial registration), but UDM has ongoing event exposure subscriptions stored (e.g. in UDR), UDM shall invoke one Namf_EventExposure Subscribe Service operations (see clause 5.3.2.2 of 3GPP TS 29.518 [36]) on behalf of NEF per subscription stored. When the AMF identified that event synchronization is needed but AMF and/or UDM are not supporting event synchronization (see clause 5.3.2.4.2 of 3GPP TS 29.518 [36]), the AMF may also ask the UDM to re-subscribe the stored events by indicating there is no ongoing event subscriptions in the UECM operation.

If ueReachableInd IE is received indicating that the UE is not reachable, the UDM shall not trigger Reachability Report for the UE upon this UECM registration. The AMF shall subsequently generate UE Reachability Report when it detects the UE is reachable. If the If ueReachableInd IE is received indicating that the UE reachability is unknown, the UDM may trigger Reachability Report for the UE when needed based on operator policy.

When UDM confirms the UE Registration, it generates a proof of USIM authentication and sends them to AMF with Nudm_UEContextManagement response.

-------------------------------------

Adding a proof of USIM authentication to Table 6.2.6.2.2-1 (Definition of type Amf3GppAccessRegistration).

-------------------------------------

6.2.6.2.2	Type: Amf3GppAccessRegistration

Table 6.2.6.2.2-1: Definition of type Amf3GppAccessRegistration

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| amfInstanceId | NfInstanceId | M | 1 | The identity the AMF uses to register in the NRF. |
| : : |  |  |  |  |
| ueSnpnSubscriptionInd | boolean | C | 0..1 | This IE shall only be included for the case when the AMF determines that the ME does not support SOR-SNPN-SI (i.e., it may be included during registration from AMF if sorSnpnSiSupported is set to false or not included. If this attribute is included, it shall indicate: - true: the UE supports equivalent SNPNs, the ME does not support SOR-SNPN-SI and the UE is in an equivalent SNPN of the subscribed SNPN. - false: otherwise. |
| proofOfUsimAuth | T.B.D | O | 0..1 | Contains the proof of USIM authentication (i.e., a token valid for about 1 minute) |
| NOTE 1: The urrpIndicator attribute shall only be exposed over the Nudr SBI, and it shall not be included by the AMF. NOTE 2: Regardless of the Dual Registration Flag, the SGSN, if any, is required to be cancelled (see 3GPP TS 23.502 [3] clause 4.11.5.2) |  |  |  |  |

2.1.4	AMF > UE: Procedure enhancements for TS24.501 (NAS protocol for 5GS; Stage 3) (CT1)

Adding the following procedure to clause 5.5.1.3 (Registration procedure for mobility and periodic registration update) of TS 24.501:

- AMF sends a registered S-CSCF name to the UE in Registration accept or TAU message.

Adding a registered S-CSCF name to Table 8.2.7.1.1 (REGISTRATION ACCEPT message content).

-------------------------------------

Table 8.2.7.1.1: REGISTRATION ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
| : : |  |  |  |  |  |
| 63 | RAT utilization control | RAT utilization control 9.11.3.110 | O | TLV | 4 |
| x | S-CSCF name | T.B.D | T.B.D | T.B.D | T.B.D |

-------------------------------------

2.1.5	AMF > UE: IE enhancements for TS24.301 (NAS protocol for EPS; Stage 3) (CT1)

Adding a registered S-CSCF name to Table 8.2.26.1 (TRACKING AREA UPDATE ACCEPT message content) of TS 24.301.

-------------------------------------

Table 8.2.26.1: TRACKING AREA UPDATE ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
| : ; |  |  |  |  |  |
| xx | RAT utilization control | RAT utilization control 9.9.3.3x | O | TLV | 4-n |
| yy | S-CSCF name | T.B.D | T.B.D | T.B.D | T.B.D |

2.1.6	UE > P/I-CSCF, HSS/UDM: Procedure enhancements for TS23.228 (IMS; Stage 2) (SA2)

When a UE receives a S-CSCF name from AMF, the UE contains the S-CSCF name in SIP message and sends it to the P/I-CSCF with SIP Registration request. If the I-CSCF receives the S-CSCF name from P-CSCF, it can skip the Cx Query procedures.

For current description in clause 5.2.2.3 and clause 5.2.2.4 Registration information flow of TS 23.228, Cx-Query request in Step 3 is mandatory. Thus, following optional description (similar to Cx-put/Cx-pull of step 6) is added:

NOTE: Optionally as an optimisation, the I-CSCF can detect the S-CSCF name provided by the P-CSCF and omit the Cx-Query request.

Based on the following NOTE as described in clause 5.2.2 of TS23.228, security related terminologies such as message authentication and encryption can be excluded.

NOTE: These flows do not take into account security features such as user authentication. The description of the impact of IMS security features is done in TS 33.203 [19].

Adding the following procedures to step 1, 2 and 3 of clause 5.2.2.3 (Registration information flow – User not registered):

-------------------------------------

5.2.2.3	Registration information flow – User not registered

The application level registration can be initiated after the registration to the access is performed, and after IP connectivity for the signalling has been gained from the access network. For the purpose of the registration information flows, the user is considered to be always roaming. For user roaming in their home network, the home network shall perform the role of the visited network elements and the home network elements.

Figure 5.1: Registration – User not registered

1.	After the UE has obtained IP connectivity, it can perform the IM registration. To do so, the UE sends the Register information flow to the proxy (Public User Identity, Private User Identity, home network domain name, UE IP address, Instance Identifier, GRUU Support Indication, S-CSCF name).

2.	Upon receipt of the register information flow, the P-CSCF shall examine the "home domain name" to discover the entry point to the home network (i.e. the I-CSCF). The proxy shall send the Register information flow to the I-CSCF (P-CSCF address/name, Public User Identity, Private User Identity, P-CSCF network identifier, UE IP address, S-CSCF name). A name-address resolution mechanism is utilised in order to determine the address of the home network from the home domain name. The P-CSCF network identifier is a string that identifies at the home network, the network where the P-CSCF is located (e.g. the P-CSCF network identifier may be the domain name of the P-CSCF network).

3.	The I-CSCF shall send the Cx-Query/Cx-Select-Pull information flow to the HSS (Public User Identity, Private User Identity, P-CSCF network identifier).

The HSS shall check whether the user is registered already. The HSS shall indicate whether the user is allowed to register in that P-CSCF network (identified by the P-CSCF network identifier) according to the User subscription and operator limitations/restrictions if any.

NOTE: Optionally as an optimisation, the I-CSCF can detect the S-CSCF name provided by the P-CSCF and omit the Cx-Query request.

Adding the following procedures to step 1, 2 and 3 of clause 5.2.2.4 (Re-Registration information flow – User currently registered):

-------------------------------------

5.2.2.4	Re-Registration information flow – User currently registered

Periodic application level re-registration is initiated by the UE either to refresh an existing registration or in response to a change in the registration status of the UE. A re-registration procedure can also be initiated when the capabilities of the UE have changed or the IP-CAN has changed. The UE should perform IMS re-registration when the IP-CAN used by the UE changes between 3GPP access and WLAN access.

Re-registration follows the same process as defined in clause 5.2.2.3 "Registration Information Flow – User not registered". When initiated by the UE, based on the registration time established during the previous registration, the UE shall keep a timer shorter than the registration related timer in the network.

NOTE 1: if the UE does not re-register, any active sessions may be deactivated.

Figure 5.2: Re-registration - user currently registered

1.	The UE initiates a re-registration. For periodic registration, the UE initiates a re-registration prior to expiry of the agreed registration timer. To re-register, the UE sends a new REGISTER request. The UE sends the REGISTER information flow to the proxy (Public User Identity, Private User Identity, home network domain name, UE IP address, capability information, Instance Identifier, GRUU Support Indication, S-CSCF name).

2.	Upon receipt of the register information flow, the P-CSCF shall examine the "home domain name" to discover the entry point to the home network (i.e. the I-CSCF). The proxy does not use the entry point cached from prior registrations. The proxy shall send the Register information flow to the I-CSCF (P-CSCF address/name, Public User Identity, Private User Identity, P-CSCF network identifier, UE IP address, S-CSCF name). A name-address resolution mechanism is utilised in order to determine the address of the home network from the home domain name. The P-CSCF network identifier is a string that identifies at the home network, the network where the P-CSCF is located (e.g. the P-CSCF network identifier may be the domain name of the P-CSCF network).

NOTE 2:	The P-CSCF may force the UE to attempt initial registration with another P-CSCF, instead of forwarding its re-registration request. This is useful e.g. to move users from a P-CSCF to another P-CSCF without interrupting the service to these users.

3.	The I-CSCF shall send the Cx-Query information flow to the HSS (Public User Identity, Private User Identity and P-CSCF network identifier).

NOTE: Optionally as an optimisation, the I-CSCF can detect the S-CSCF name provided by the P-CSCF and omit the Cx-Query request.

-------------------------------------

Adding the following procedures to clause 5.2.1 Requirements considered for registration:

-------------------------------------

5.2.1	Requirements considered for registration

The following points are considered as requirements for the purpose of the registration procedures.

1.	The architecture shall allow for the Serving-CSCFs to have different capabilities or access to different capabilities. E.g. a VPN CSCF or CSCFs in different stages of network upgrade.

2.	The network operator shall not be required to reveal the internal network structure to another network. Association of the node names of the same type of entity and their capabilities and the number of nodes will be kept within an operator's network. However disclosure of the internal architecture shall not be prevented on a per agreement basis.

:

18.	When the UE determines that the radio conditions are suitable for IMS PS voice/video services (e.g. UE is in normal coverage or in CE mode A as defined in Annex E, clause E.1.2) then UE may register for IMS voice/video services.

x.	The UE may indicate the registered S-CSCF name and sends it to P-CSCF during IMS registration. When the I-CSCF receives the S-CSCF name from the P-CSCF, it may skip the Cx Query procedures.

-------------------------------------

2.1.7	UE > P/I/S-CSCF, HSS/UDM: Procedure enhancements for TS33.203 (3G security; Access security for IP-based services) (SA3)

Adding following procedures to clause 6.1.1 of TS 33.203:

-------------------------------------

6.1.1	Authentication of an IM-subscriber

Before a user can get access to the IM services at least one IMPU needs to be registered and the IMPI authenticated in the IMS at application level. In order to get registered the UE sends a SIP REGISTER message towards the SIP registrar server i.e. the S-CSCF, cf. figure 1, which will perform the authentication of the user. The message flows are the same regardless of whether the user has an IMPU already registered or not.

Skip

Figure 4: The IMS Authentication and Key Agreement for an unregistered IM subscriber and successful mutual authentication with no synchronization error

The detailed requirements and complete registration flows are defined in TS 24.229 [8] and TS 24.228 [11].

SMn stands for SIP Message n and CMm stands for Cx message m which has a relation to the authentication process:

SM1: REGISTER(IMPI, IMPU)

In SM1, The UE may indicate the registered S-CSCF name and send it to P/I-CSCF during IMS registration.

In SM2 and SM3 the P-CSCF and the I-CSCF respectively forwards the SIP REGISTER towards the S-CSCF.

If the I-CSCF retrieves the S-CSCF name with the designated security method such as message authentication (e.g. HMAC, CMAC) and encryption, it can determine the S‑CSCF address through a name-address resolution mechanism as define in TS 23.288[3], allowing it to skip the Cx-Selection-Info procedures.

2.1.8	UE > P/I-CSCF: Procedure and IE enhancements for TS24.229 (SDP/SIP; Stage 3) (CT1)

Adding the following procedures to clause 5.1.1 (Registration and authentication) of TS 24.229.

When the UE performs SIP Registration request, it contains a registered S-CSCF name in one of the following fields in SIP message and sends them to the P/I-CSCF.

- Contact parameter: It is possible to set multiple contact parameters and freely configure the string. Thus, there is no impact on RFC.

- Authorization header: It would be possible to create a URI for the S-CSCF name on the P/I-CSCF, and the URI could be specified on both the client and server side as a matter of operator policy. Thus, there is no impact on RFC.

- Security-Client: it would be highly versatile since it allows setting and sending parameters on the client side. It is assumed that extensions to RFC3329 are not necessary. However, the Security-Client's parameters are strictly defined, which may necessitate extensions.

2.2	Solution #2: Efficient USIM authentication for IMS AKA

The parts enclosed in red in the table below are introduced in this clause.

2.2.1	UE > P/I/S-CSCF, HSS/UDM: Procedure enhancements for TS33.203 (3G security; Access security for IP-based services) (SA3)

Adding following procedures to clause 6.1.1 of TS 33.203:

-------------------------------------

6.1.1	Authentication of an IM-subscriber

Before a user can get access to the IM services at least one IMPU needs to be registered and the IMPI authenticated in the IMS at application level. In order to get registered the UE sends a SIP REGISTER message towards the SIP registrar server i.e. the S-CSCF, cf. figure 1, which will perform the authentication of the user. The message flows are the same regardless of whether the user has an IMPU already registered or not.

Skip

Figure 4: The IMS Authentication and Key Agreement for an unregistered IM subscriber and successful mutual authentication with no synchronization error

The detailed requirements and complete registration flows are defined in TS 24.229 [8] and TS 24.228 [11].

SMn stands for SIP Message n and CMm stands for Cx message m which has a relation to the authentication process:

SM1: REGISTER(IMPI, IMPU)

In SM2 and SM3 the P-CSCF and the I-CSCF respectively forwards the SIP REGISTER towards the S-CSCF.

For non-IPSec environment, if the S-CSCF retrieves the S-CSCF name with the designated security method such as message authentication (e.g. HMAC, CMAC) and encryption, it can determine the S‑CSCF address through a name-address resolution mechanism as define in TS 23.288[3], allowing it to skip subsequent authentication procedures (CM1 - SM9)

:

:

Upon receiving SM9 containing the response, the S-CSCF retrieves the active XRES for that user and uses this to check the authentication response sent by the UE as described in RFC 3310 [17] and RFC 4169 [65]. If the check is successful then the user has been authenticated and the IMPU is registered in the S-CSCF. If the IMPU was not currently registered, the S-CSCF shall send a Cx-Put to update the registration-flag to registered. If the IMPU was currently registered the registration-flag is not altered.

NOTE:	For non-IPSec environment, the mutual authentication with CK and IK is not necessary. If the S-CSCF can identify the USIM authentication, the procedure to compare RES and XRES can be skipped.

-------------------------------------

2.2.2	UE > P/I/S-CSCF, HSS/UDM: Procedure and IE enhancements for TS24.229 (SDP/SIP; Stage 3) (CT1)

Adding the following procedures to clause 5.1.1 (Registration and authentication) of TS 24.229.

For non-IPSec environment, if the I-CSCF retrieves the S-CSCF name with the  designated security method such as message authentication (e.g. HMAC, CMAC) or encryption, it can identify that the USIM authentication in the UDM is complete, allowing it to skip 401 Auth Challenge and subsequent SIP Register procedures.

2.2.3	I/S-CSCF, HSS/UDM: Procedure enhancements for TS29.228 (Cx and Dx interfaces; Signalling flows and message contents; Stage 3) (CT4)

Adding the following procedures to clause 6.1(Location management procedures) and clause 6.3 (Authentication procedures) of TS 29.228.

For non-IPSec environment, if the I-CSCF retrieves the S-CSCF name with the designated security method such as message authentication (e.g. HMAC, CMAC) or encryption, it can identify that the USIM authentication in the HSS is complete, allowing it to skip Cx-AV procedures (Clause 6.3) and Cx-Query and Cx-Select-Pull procedures (Clause 6.1).

2.3	Solution #3: Efficient IMS AKA with a proof of USIM auth/ roaming status

The parts enclosed in red in the table below are introduced in this clause.

2.3.1	UDM > AMF > UE: Procedure enhancements for TS 23.501 (SA2)

Adding following new clause to clause 5.16.3 of TS 23.501:

-------------------------------------

5.16.3.y (new)	Proof of USIM authentication delivery

For the UE registration procedures, the UDM may check the UE registration status. If already registered, the UDM may generate a proof of USIM authentication (i.e., a token valid for about 1 minute) and sends them to AMF with Nudm_UECM_Registration Response.

The AMF may send the proof of USIM authentication to the UE with Registration Accept.

-------------------------------------

Adding following description to clause 5.16.3.1:

-------------------------------------

5.16.3	IMS support

5.16.3.1	General

IP-Connectivity Access Network specific concepts when using 5GS to access IMS can be found in TS 23.228 [15].

5GS supports IMS with the following functionality:

-	Indication toward the UE if IMS voice over PS session is supported.

-	Capability to transport the P-CSCF address(es) to UE.

-	Paging Policy Differentiation for IMS as defined in TS 23.228 [15].

-	IMS emergency service as defined in TS 23.167 [18].

-	Domain selection for UE originating sessions.

-	Terminating domain selection for IMS voice.

-	Support of P-CSCF restoration procedure (clause 5.16.3.9).

-	NRF based P-CSCF discovery (clause 5.16.3.11).

NOTE:	The NRF based P-CSCF discovery has no impact on the UE, i.e. the UE does not need to know how P-CSCF IP address(es) is discovered in the network.

-	NRF based HSS discovery (clause 5.16.3.12).

- Capability to transport the proof of USIM authentication delivery (clause 5.16.3.y)

-------------------------------------

2.3.2	UDM > AMF > UE: Procedure and IE enhancements for TS 23.502 (SA2)

Adding the following procedures to clause 4.2.2.2.2 (General Registration) of TS 23.502:

-------------------------------------

- Step 14a: The UDM may check the UE registration status. If already registered, the UDM shall generates a proof of USIM authentication (i.e., a token valid for about 1 minute) and sends them to AMF with Nudm_UECM_Registration Response.

- Step 21: If the AMF receives the proof of USIM authentication in Step 14a, the AMF shall include them in the Registration Accept message.

- Step 21: AMF sends the proof of USIM authentication to the UE with Registration Accept.

-------------------------------------

Adding a registered S-CSCF name to UE with Registration Accept in step 21 of clause 4.2.2.2.2 (General Registration).

-------------------------------------

21. New AMF to UE: Registration Accept (5G-GUTI, Registration Area, [Mobility restrictions], [PDU Session status], [Allowed NSSAI], [Mapping Of Allowed NSSAI], [Partially Allowed NSSAI], [Mapping Of Partially Allowed NSSAI], [TAI List for S-NSSAIs in Partially Allowed NSSAI], [Configured NSSAI for the Serving PLMN], [Mapping Of Configured NSSAI], [NSSRG Information], [NSAG Information], [rejected S-NSSAIs], [TAI List for any rejected S-NSSAI Partially in the RA], [Pending NSSAI], [Mapping Of Pending NSSAI], [Periodic Registration Update timer], [Active Time], [Strictly Periodic Registration Timer Indication], [LADN Information], [MICO Indication], [IMS Voice over PS session supported Indication], [Emergency Service Support indicator], [Accepted DRX parameters for E-UTRA and NR], [Accepted DRX parameters for NB-IoT], [extended idle mode DRX parameters], [Paging Time Window], [Network support of Interworking without N26], [Access Stratum Connection Establishment NSSAI Inclusion Mode], [Network Slicing Subscription Change Indication], [Operator-defined access category definitions], [List of equivalent PLMNs], [Enhanced Coverage Restricted information], [Supported Network Behaviour], [Service Gap Time], [PLMN-assigned UE Radio Capability ID], [PLMN-assigned UE Radio Capability ID deletion], [WUS Assistance Information], [AMF PEIPS Assistance Information], [Truncated 5G-S-TMSI Configuration], [Connection Release Supported], [Paging Cause Indication for Voice Service Supported], [Paging Restriction Supported], [Reject Paging Request Supported], [Paging Restriction Information acceptance / rejection], ["List of PLMN(s) to be used in Disaster Condition"], [Disaster Roaming wait range information], [Disaster Return wait range information], [Forbidden TAI(s)], [List of equivalent SNPNs], [Registered NID], [Unavailability Period Support], [MBSR authorization information], [Return To Coverage Notification Not Required], [Unavailability Period Duration], [Start of Unavailability Period], [S-NSSAI location availability information], [Mapping Of Alternative NSSAI], [Slice Usage Policy], [Maximum Time Offset], [Proof of USIM authentication]).

2.3.3	UDM > AMF: Procedure and IE enhancements for TS29.503 (UDM Services; Stage 3) (CT4)

Adding the following procedures to step 2a of clause 5.3.2.2.2 (AMF registration for 3GPP access) : When UDM confirms the UE Registration, it generates a proof of USIM authentication (i.e., a token valid for about 1 minute) and sends them to AMF with Nudm_UEContextManagement response.

Adding a proof of USIM authentication to Table 6.2.6.2.2-1 (Definition of type Amf3GppAccessRegistration) of TS 29.503:

2.3.4	AMF > UE: Procedure enhancements for TS24.501 (NAS protocol for 5GS; Stage 3) (CT1)

Adding the following procedure to clause 5.5.1.3 (Registration procedure for mobility and periodic registration update):

- AMF sends a proof of USIM authentication (i.e., a token valid for about 1 minute) to the UE in Registration accept or TAU procedures.

Adding a proof of USIM authentication to Table 8.2.7.1.1 (REGISTRATION ACCEPT message content) of TS 24.501.

-------------------------------------

Table 8.2.7.1.1: REGISTRATION ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
| : : |  |  |  |  |  |
| 63 | RAT utilization control | RAT utilization control 9.11.3.110 | O | TLV | 4 |
| x | a proof of USIM authentication | T.B.D | T.B.D | T.B.D | T.B.D |

-------------------------------------

2.3.5	AMF > UE: IE enhancements for TS24.301 (NAS protocol for EPS; Stage 3) (CT1)

Adding a proof of USIM authentication to Table 8.2.26.1 (TRACKING AREA UPDATE ACCEPT message content) of TS 24.301:

-------------------------------------

Table 8.2.26.1: TRACKING AREA UPDATE ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
| : ; |  |  |  |  |  |
| xx | RAT utilization control | RAT utilization control 9.9.3.3x | O | TLV | 4-n |
| yy | a proof of USIM authentication | T.B.D | T.B.D | T.B.D | T.B.D |

-------------------------------------

2.3.6	UE > P/I/S-CSCF, HSS/UDM: Procedure enhancements for TS33.203 (3G security; Access security for IP-based services) (SA3)

The S-CSCF identifies the roaming status by using the following steps.

Adding following procedures to clause 6.1.1 of TS 33.203:

-------------------------------------

6.1.1	Authentication of an IM-subscriber

Before a user can get access to the IM services at least one IMPU needs to be registered and the IMPI authenticated in the IMS at application level. In order to get registered the UE sends a SIP REGISTER message towards the SIP registrar server i.e. the S-CSCF, cf. figure 1, which will perform the authentication of the user. The message flows are the same regardless of whether the user has an IMPU already registered or not.

Skip

Figure 4: The IMS Authentication and Key Agreement for an unregistered IM subscriber and successful mutual authentication with no synchronization error

The detailed requirements and complete registration flows are defined in TS 24.229 [8] and TS 24.228 [11].

SMn stands for SIP Message n and CMm stands for Cx message m which has a relation to the authentication process:

SM1: REGISTER(IMPI, IMPU)

In SM1, the UE may indicate a proof of USIM authentication (i.e., a token valid for about 1 minute) in SIP message and sends them to the P/I-CSCF.

In SM2 and SM3 the P-CSCF and the I-CSCF respectively forwards the SIP REGISTER towards the S-CSCF.

For non-IPSec environment, if the S-CSCF identifies a proof of USIM authentication and Home PLMN in SIP Register message from I-CSCF (SM3), Cx-AV and the subsequent authentication procedures can be skipped.

The S-CSCF identifies the roaming status by using the combination of the following three methods:

- Identify the MNC/MCC in the PANI header included in the SIP message (SM3). If the roaming status is Home PLMN, the S-CSCF skips from Cx-Put to SM9. If not, S-CSCF does not skip them and continues 2xx Auth OK procedures.

- Identify the serving network information generated by the HSS to Cx-Pull message. If the roaming status is Home PLMN, the S-CSCF continues normal procedures. If not, the S-CSCF performs error operation.

- Identify the roaming status in User-Data-Request. If the roaming status is Home PLMN, the S-CSCF continues normal procedures. If not, S-CSCF performs error operation.

After receiving SM3, if the IMPU is not currently registered at the S-CSCF, the S-CSCF needs to set the registration flag at the HSS to initial registration pending. This is done in order to handle UE terminated calls while the initial registration is in progress and not successfully completed. The registration flag is stored in the HSS together with the S-CSCF name and user identity, and is used to indicate whether a particular IMPU of the user is unregistered or registered at a particular S-CSCF or if the initial registration at a particular S-CSCF is pending. The registration flag is set by the S-CSCF sending a Cx-Put to the HSS. If the IMPU is currently registered, the S-CSCF shall leave the registration flag set to registered. At this stage the HSS has performed a check that the IMPI and the IMPU belong to the same user.

:

:

Upon receiving SM9 containing the response, the S-CSCF retrieves the active XRES for that user and uses this to check the authentication response sent by the UE as described in RFC 3310 [17] and RFC 4169 [65]. If the check is successful then the user has been authenticated and the IMPU is registered in the S-CSCF. If the IMPU was not currently registered, the S-CSCF shall send a Cx-Put to update the registration-flag to registered. If the IMPU was currently registered the registration-flag is not altered.

NOTE:	For non-IPSec environment, the mutual authentication with CK and IK is not necessary. If the S-CSCF can identify the USIM authentication, the procedure to compare RES and XRES can be skipped.

If the HSS retrieve the token in the proof of USIM authentication received in Cx-Put message, the HSS verifies it against the token held by the HSS.

-------------------------------------

2.3.7	UE > P/I/S-CSCF, HSS/UDM: Procedure and IE enhancements for TS24.229 (SDP/SIP; Stage 3) (CT1)

Adding the following procedures to clause 5.1.1 (Registration and authentication) of TS 24.229.

When the UE starts SIP Registration request, it performs one of the following 2 options:

- Opt.1: If the UE receives a proof of USIM authentication from the AMF, it contains them in SIP message and sends them to the P/I-CSCF.

- Opt.2: If the UE receives Registration accept or TAU messages from AMF, it contains a proof of USIM authentication in SIP message and sends it to the P/I-CSCF.

The IEs above can contain one of the following fields in SIP message.

- Contact parameter : It is possible to set multiple contact parameters and freely configure the string. Thus, there is no impact on RFC.

- Authorization header: It would be possible to create a URI for the S-CSCF name on the P/I-CSCF, and the URI could be specified on both the client and server side as a matter of operator policy. Thus, there is no impact on RFC.

- Security-Client: it would be highly versatile since it allows setting and sending parameters on the client side. It is assumed that extensions to RFC3329 are not necessary. However, the Security-Client's parameters are strictly defined, which may necessitate extensions.

For non-IPSec environment, if the S-CSCF retrieves the proof of USIM authentication (i.e., a token valid for about 1 minute), it can identify that the USIM authentication in the UDM is complete, allowing it to skip 401 Auth Challenge and subsequent SIP Register procedures.

2.3.8	I/S-CSCF, HSS/UDM: Procedure enhancements for TS29.228 (Cx and Dx interfaces; Signalling flows and message contents; Stage 3) (CT4)

Adding the following procedures to clause 6.1(Location management procedures) and clause 6.3 (Authentication procedures) of TS 29.228:

- If the S-CSCF identifies a proof of USIM authentication (i.e., a token valid for about 1 minute) and Home PLMN in SIP Register message from I-CSCF, allowing it to skip Cx-AV procedures (Clause 6.3) and Cx-Query and Cx-Select-Pull procedures (Clause 6.1).

- S-CSCF identifies the roaming status by using the combination of the following three methods (Clause 6.1:

- Identify the MNC/MCC in the PANI header included in the SIP message. If the roaming status is Home PLMN, the S-CSCF skips Cx-AV procedures. If not, S-CSCF do not skip them and continues normal procedures.

- Identify the serving network information generated by the HSS to Cx-Pull message. If the roaming status is Home PLMN, the S-CSCF continues normal procedures. If not, S-CSCF performs error operation.

- Identify the roaming status in User-Data-Request. If the roaming status is Home PLMN, the S-CSCF continues normal procedures. If not, S-CSCF performs error operation.

- For non-IPSec environment, the mutual authentication with CK and IK is not necessary. If the S-CSCF can identify the USIM authentication, the procedure to compare RES and XRES can be skipped.

2.4	Solution #4: Efficient IMS AKA with access type/ roaming status

The parts enclosed in red in the table below are introduced in this clause.

2.4.1	UE > P/I/S-CSCF, HSS/UDM: Procedure enhancements for TS33.203 (3G security; Access security for IP-based services) (SA3)

Adding following procedures to clause 6.1.1 of TS 33.203:

-------------------------------------

6.1.1	Authentication of an IM-subscriber

Before a user can get access to the IM services at least one IMPU needs to be registered and the IMPI authenticated in the IMS at application level. In order to get registered the UE sends a SIP REGISTER message towards the SIP registrar server i.e. the S-CSCF, cf. figure 1, which will perform the authentication of the user. The message flows are the same regardless of whether the user has an IMPU already registered or not.

Skip

Figure 4: The IMS Authentication and Key Agreement for an unregistered IM subscriber and successful mutual authentication with no synchronization error

The detailed requirements and complete registration flows are defined in TS 24.229 [8] and TS 24.228 [11].

SMn stands for SIP Message n and CMm stands for Cx message m which has a relation to the authentication process:

SM1: REGISTER(IMPI, IMPU)

In SM2 and SM3 the P-CSCF and the I-CSCF respectively forwards the SIP REGISTER towards the S-CSCF.

For non-IPSec environment, the S-CSCF determines roaming status using the MNC/MCC in the PANI header in the SIP Registration message. The S-CSCF also determines access network type using PANI header in the SIP Registration message. When the S-CSCF identifies Home PLMN and 3GPP access, the S-CSCF skips the subsequent authentication procedures (Cx-Put - SM9).

:

:

Upon receiving SM9 containing the response, the S-CSCF retrieves the active XRES for that user and uses this to check the authentication response sent by the UE as described in RFC 3310 [17] and RFC 4169 [65]. If the check is successful then the user has been authenticated and the IMPU is registered in the S-CSCF. If the IMPU was not currently registered, the S-CSCF shall send a Cx-Put to update the registration-flag to registered. If the IMPU was currently registered the registration-flag is not altered.

NOTE:	For non-IPSec environment, the mutual authentication with CK and IK is not necessary. If the S-CSCF can identify the USIM authentication, the procedure to compare RES and XRES can be skipped.

2.4.2	UE > P/I/S-CSCF, HSS/UDM: Procedure enhancements for TS24.229 (SDP/SIP; Stage 3) (CT1)

Adding the following procedures to clause 5.1.1 (Registration and authentication) of TS 24.229:

For non-IPSec environment, the S-CSCF performs a combination of following methods and when determined as Home PLMN + 3GPP access, it can identify that the USIM authentication in the UDM is complete, allowing it to skip 401 Auth Challenge and subsequent SIP Register procedures (Clause 5.1.1).

- Determine roaming status using the MNC/MCC in the PANI header in the SIP Registration message.

- Determine access network type using PANI header in the SIP Registration message.

2.4.3	I/S-CSCF, HSS/UDM: Procedure enhancements for TS29.228 (Cx and Dx interfaces; Signalling flows and message contents; Stage 3) (CT4)

Adding the following procedures to clause 6.1(Location management procedures) and clause 6.3 (Authentication procedures) of TS 29.228:

For non-IPSec environment, the S-CSCF performs a combination of following methods and when determined as Home PLMN + 3GPP access, the S-CSCF skips Cx-AV procedures (Clause 6.3) and Cx-Query and Cx-Select-Pull procedures (Clause 6.1).

- Determine roaming status using the MNC/MCC in the PANI header in the SIP Registration message.

- Determine access network type using PANI header in the SIP Registration message.