3GPP SA WG2 Meeting #167	S2-25xxxxx

Greece, Athens, 17 - 21 February, 2025

Source:	KDDI

Title:	Draft CRs for mitigating IMS failures of IMS resiliency

Document for:	Discussion

Agenda Item:	30.8

Work Item / Release:	T.B.D / Rel-20

Abstract: This paper provides draft CRs based on the potential solutions for mitigating IMS failures in the IMS resiliency discussion paper.

1. Introduction

The contribution provides draft CRs based on the potential solutions for mitigating IMS failures of IMS resiliency discussion paper.

2. Draft CRs for mitigating IMS failures

The additional updates are highlighted in blue.

2.1	Solution #6, #9: IMS/PCRF failures notification to UE with PCO/ePCO

2.1.1	TS 23.501 (SA2)

Adding a new clause for IMS status descriptions and update in clause 5.16.3.1 and clause 5.16.3.4 of TS 23.501:

-------------------------------------

5.16.3	IMS support

5.16.3.1	General

IP-Connectivity Access Network specific concepts when using 5GS to access IMS can be found in TS 23.228 [15].

5GS supports IMS with the following functionality:

-	Indication toward the UE if IMS voice over PS session is supported.

-	Capability to transport the P-CSCF address(es) to UE.

-	Paging Policy Differentiation for IMS as defined in TS 23.228 [15].

-	IMS emergency service as defined in TS 23.167 [18].

-	Domain selection for UE originating sessions.

-	Terminating domain selection for IMS voice.

-	Support of P-CSCF restoration procedure (clause 5.16.3.9).

-	NRF based P-CSCF discovery (clause 5.16.3.11).

NOTE:	The NRF based P-CSCF discovery has no impact on the UE, i.e. the UE does not need to know how P-CSCF IP address(es) is discovered in the network.

-	NRF based HSS discovery (clause 5.16.3.12).

-	SMF base IMS status notification (clause 5.16.3.x).

5.16.3.4	P-CSCF address delivery

At PDU Session Establishment procedure related to IMS, SMF shall support the capability to send the P-CSCF address(es) to UE. The SMF is located in VPLMN if LBO is used. This is sent by visited SMF if LBO is used. For Home routed, this information is sent by the SMF in HPLMN. P-CSCF address(es) shall be sent transparently through AMF and in the case of Home Routed also through the SMF in VPLMN. The P-CSCF IP address(es) may be locally configured in the SMF, or discovered using NRF as described in clause 5.16.3.11.

NOTE 1:	Other options to provide P-CSCF to the UE as defined in TS 23.228 [15] is not excluded.

NOTE 2:	PDU Session for IMS is identified by "APN" or "DNN".

In the case of SNPN access the SMF is always located in the serving SNPN (no support for Home Routed traffic); therefore, the serving SMF sends the P-CSCF address(es) to the UE.

If a failure in the P-CSCF is detected, the SMF may send the P-CSCF address as Null to the UE.

5.16.3.X	SMF base IMS status notification

At PDU Session Establishment procedure related to IMS, if the SMF receives the IMS status indication from the AMF, the SMF shall support the capability to send the IMS status, Request Suppression Timer, Recovery timer and Ack required to UE for IMS failure/recovery.

In order to identify the UE which supports the IMS status feature, the UE shall include the IMS status indicator within the SM container. The SMF shall store this UE capability.

If a failure in the P-CSCF is detected, the following applies:

- The PCF may send the IMS failure notification to the SMF.

- The SMF shall send the IMS status (e.g. Not applicable) and the Request Suppression Timer to the UE which supports the IMS status feature.

NOTE:	In the event of IMS failure, the UE will try to prioritize the restoration of MMTEL service and attempt initial registration, but this action can be suppressed.

If a revovery in the P-CSCF is detected, the following applies:

- The PCF may send the IMS recovery notification to the SMF.

- The SMF shall send the IMS status (e.g. Applicable) and the Recovery Timer and may sends the Ack required to the UE which supports the IMS status feature. If the UE receives the notification with Ack required from the SMF, it shall send an ACK message.

2.1.2	TS 23.502 (SA2)

Adding IMS status IEs to step 1 and step 11 in clause 4.3.2.2.1 of TS 23.502:

-------------------------------------

4.3.2.2	UE Requested PDU Session Establishment

4.3.2.2.1	Non-roaming and Roaming with Local Breakout

The procedure assumes that the UE has already registered on the AMF thus unless the UE is Emergency Registered the AMF has already retrieved the user subscription data from the UDM.

1.	From UE to AMF: NAS Message (S-NSSAI(s), [Alternative S-NSSAI], UE Requested DNN, PDU Session ID, Request type, Old PDU Session ID, N1 SM container (PDU Session Establishment Request, [Port Management Information Container])).

:

:

If UE supports to report URSP rule enforcement to network and the URSP rule that triggered this PDU Session Establishment Request included the Indication for reporting URSP rule enforcement, the UE may provide URSP rule enforcement report as described in clause 6.6.2.4 of TS 23.503 [20].

If the UE is establishing a PDU session for IMS and the UE supports to notify IMS status, the UE shall include an IMS status indicator within the SM container.

:

:

11.	SMF to AMF: Namf_Communication_N1N2MessageTransfer (PDU Session ID, N2 SM information (PDU Session ID, QFI(s), QoS Profile(s), CN Tunnel Info, S-NSSAI from the Allowed NSSAI or Partially Allowed NSSAI, Session-AMBR, PDU Session Type, User Plane Security Enforcement information, UE Integrity Protection Maximum Data Rate, RSN, PDU Session Pair ID, TL-Container), N1 SM container (PDU Session Establishment Accept ([QoS Rule(s) and associated UL Protocol Description(s) (if available), QoS Flow level QoS parameters if needed for the QoS Flow(s) associated with the QoS rule(s)], selected SSC mode, S-NSSAI(s), UE Requested DNN, allocated IPv4 address, interface identifier, Session-AMBR, selected PDU Session Type, [Reflective QoS Timer] (if available), [P-CSCF address(es)], [Control Plane Only indicator], [Header Compression Configuration], [Always-on PDU Session Granted], [Small Data Rate Control parameters], [Small Data Rate Control Status], [Serving PLMN Rate Control], [PVS FQDN(s) and/or PVS IP address(es)], [Non-3GPP QoS Assistance Information Container], [IMS status], [Request Suppression Timer], [Recovery timer], [Ack required]))). If multiple UPFs are used for the PDU Session, the CN Tunnel Info contains tunnel information related with the UPFs that terminate N3.

:

:

The N1 SM container contains the PDU Session Establishment Accept that the AMF shall provide to the UE. If the UE requested P-CSCF discovery then the message shall also include the P-CSCF IP address(es) as determined by the SMF and as described in clause 5.16.3.4 of TS 23.501 [2]. The PDU Session Establishment Accept includes S-NSSAI from the Allowed NSSAI or Partially Allowed NSSAI. The S-NSSAI value of the Alternative S-NSSAI is included in the PDU session Establishment Accept if the SMF has received the Alternative S-NSSAI from the AMF. For LBO roaming scenario, the PDU Session Establishment Accept includes the S-NSSAI from the Allowed NSSAI or Partially Allowed NSSAI for the VPLMN and also it includes the corresponding S-NSSAI of the HPLMN from the Mapping Of Allowed NSSAI or Mapping Of Partially Allowed NSSAI that SMF received in step 3. If the SMF has received the VPLMN Alternative S-NSSAI from the AMF, the PDU Session Establishment Accept includes the VPLMN Alternative S-NSSAI. If the SMF has received the HPLMN Alternative S-NSSAI from the AMF, the PDU Session Establishment Accept includes the HPLMN Alternative S-NSSAI. If the PCF, based on the local configuration, provides the PCC rules with Protocol Descriptions for UL in step 7b or step 9, the SMF may additionally provide the Protocol Description for UL with the associated QoS rule as described in clause 5.37.5.1 of TS 23.501 [2]. If the UE requested IMS status indication, the message may include the IMS status, Request Suppression Timer, Recovery Timer and Ack required as described in clause 5.16.3.x of TS 23.501 [2].

-------------------------------------

2.1.4	TS 23.380 (CT4)

Adding following new procedures to clause 5.x of TS 23.380:

-------------------------------------

5.x	PGW/GGSN-based IMS status notification

5.x.1	Introduction

The PGW/GGSN-based IMS status notification is an optional mechanism.

This mechanism is executed when the PGW/GGSN informs the UE about the suppression and resumption of IMS registration due to a P-CSCF failure.

The PGW/GGSN-based IMS status notification consists of following methods:

-	IMS failure/recovery detection, as described in clause 5.x.2.

-	IMS failure notification, as described in clause 5.x.3.

-	IMS recovery notification as described in clause 5.x.4.

-	Deadlock prevention as described in clause 5.x.5.

5.x.2	IMS failure/recovery detection

For the IMS failure/recovery detection by the PGW/GGSN, the following three options are assumed. It is possible to use a combination of these three options, or to use just one according to operator policy.

- Option.1: Notification via PCRF.

- The PCF determines the IMS failure/recovery based on the status of Rx (Diameter). When the PCF detects the IMS failure/recovery, the PCF sends a new IE (IMS failure/IMS recovery) in the Event-Trigger AVP in Gx RAR to the PDN GW/GGSN.

- Option.2: Detect the IMS failure/recovery through liveness monitoring.

- The PGW/GGSN conducts liveness monitoring on the P-CSCF (e.g, ping)

NOTE: The monitoring configuration of the PGW depends on operator and implementation policy.

- Option.3: Manual operation by the Operator.

5.x.3	IMS failure notification

For the IMS failure notification by the PGW, the following two options are assumed:

- Option.1: Notify the IMS failure with dedicated IEs within PCO as described in clause 5.x.3.1.

- Option.2: Notify the IMS failure with the P-CSCF address as Null within PCO as described in clause 5.x.3.2.

5.x.3.1	IMS failure notification with dedicated IEs

The procedure for PGW/GGSN-based IMS failure notification with dedicated IEs is described in figure 5.x.3.1-1.

Figure 5.x.3.1-1: IMS failure notification with dedicated IEs

1.	The UE initiates an IP-CAN session. The GGSN/PDN-GW identifies the support of IMS status indication in the PCO IE of the CreatePDPContextResponse/ CreateBearerRequest.

3.	The UE performs an initial SIP registration towards the P-CSCF.

4.	With the methods described in clause 5.x.2, a failure in P-CSCF is detected.

5.	The PDN GW/GGSN sends a PCO IE with the IMS status and Request Suppression Timer to the UE which supports the IMS status feature.

6.	The UE acknowledges the request.

7. The UE identifies that the IMS is under failure condition. Then, the UE sets the timer based on the Suppression Timer in the PCO IE.

NOTE:	In the event of IMS failure, the UE will try to prioritize the restoration of MMTEL service and attempt initial registration, but this action can be suppressed.

8.	When the timer is elapsed, the UE sends an initial SIP registration towards the P-CSCF.

5.x.3.2	IMS failure notification with P-CSCF address as Null

The procedure for PGW/GGSN-based IMS status notification with P-CSCF address as Null is described in figure 5.x.3.2-1.

Figure 5.x.3.2-1: IMS status failure and notification with P-CSCF address as Null

1.	The UE initiates an IP-CAN session. The GGSN/PDN-GW identifies the support of P-CSCF Restration in the PCO IE of CreatePDPContextResponse / CreateBearerRequest.

3.	The UE performs an initial SIP registration towards the P-CSCF.

4.	With the methods described in clause 5.x.2, a failure in P-CSCF is detected.

7.	The PDN GW/GGSN sends a PCO IE with the P-CSCF address as Null to the UE which supports the P-CSCF Restration.

8.	The UE acknowledges the request.

9. The UE identifies that the IMS is under failure condition. Then, the UE does not send an initial SIP registration towards the P-CSCF.

NOTE:	In the event of IMS failure, the UE will try to prioritize the restoration of MMTEL service and attempt initial registration, but this action can be suppressed.

5.x.4	IMS recovery notification

For the IMS recovery notification, the following two options are assumed:

- Option.1: Notify the IMS recovery with dedicated IEs within PCO as described in clause 5.x.4.1.

- Option.2: Notify the IMS recovery with valid P-CSCF address within PCO as described in clause 5.x.4.2.

- Option.3: Disconnect the IMS PDN as described in clause 5.x.4.3.

5.x.4.1	IMS recovery notification with dedicated IEs

The procedure for PGW/GGSN-based IMS recovery notification with dedicated IEs is described in figure 5.x.4.1-1.

Figure 5.x.4.1-1: IMS recovery notification with dedicated IEs

Steps from 1 to 8 are the same as explained in figure 5.x.3.1-1 above.

9.	With the methods described in clause 5.x.2, a recovery in P-CSCF is detected.

10.	The PDN GW/GGSN sends a PCO IE with the IMS status and Recovery Timer to the UE which supports the IMS status indication.

11.	The UE acknowledges the request.

12. The UE identifies that the IMS is recovered. Then, the UE sets the timer based on the Recovery Timer in the PCO IE. This timer is designed to stagger the timing of each UE's Registration requests, preventing all UEs from making requests simultaneously (e.g., a random value to avoid simultaneous requests).

13.	When the timer is elapsed, the UE sends an initial SIP registration towards the P-CSCF.

5.x.4.2	IMS recovery notification with valid P-CSCF address

The procedure for PGW/GGSN-based IMS recovery notification with valid P-CSCF address is described in figure 5.x.4.2-1.

Figure 5.x.4.2-1: IMS recovery notification with valid P-CSCF address

Steps from 1 to 7 are the same as explained in figure 5.x.3.2 above.

8.	With the methods described in clause 5.x.2, a recovery in P-CSCF is detected.

9.	The PDN GW/GGSN sends a valid P-CSCF address to the UE which supports the P-CSCF Restoration.

10.	The UE acknowledges the request.

11. The UE identifies that the IMS is recovered.

12.	Tthe UE sends an initial SIP registration towards the P-CSCF.

5.x.4.3	IMS PDN Disconnection triggered by PCRF

The procedure for PGW/GGSN-based IMS recovery notification with IMS PDN Disconnection is described in figure 5.x.4.3-1. This option is reused to the P-CSCF restoration mechanism, allowing the IMS PDN reconnection and registration triggered by the IMS PDN disconnection. The advantage of this option is that if the PCRF has established junk sessions, it can prompt the UE to reconnect according to the operator's policy.

Figure 5.x.4.3-1: IMS PDN Disconnection triggered by PCRF

Steps from 9 to 13 and steps from 8 to 12 are the same as explained in figure 5.x.4.1 and figure 5.x.4.2 above.

14.	The PCRF determined the IMS PDN Disconnection according to the operator's policy and sends a Gx RAR including an IMS PDU disconnect request.

15.	The PDN GW/GGSN send a Gx RAA to the PCRF.

16.	Upon reception of the IMS PDU disconnect request, the P-GW/S-GW/GGSN shall release the IMS PDN connection/PDP context by sending a Delete Bearer Request / Delete PDP Context Request to the MME/SGSN with GTP cause "reactivation requested".

17.	Upon reception of a Delete Bearer Request / Delete PDP Context Request, the MME/SGSN shall send a Delete EPS Bearer Context Request / Delete PDP Context Request to the UE with the NAS cause "reactivation requested", then once the PDN connection is released, the UE shall re-activate the IMS PDN connection.

18.	The UE initiate IMS PDN re-establishment procedures.

19.	The UE sends an initial SIP registration towards the P-CSCF.

5.x.5	Deadlock prevention

Due to some issues, the recovery notification was dropped, and the UE does not send a SIP Registration. There are two methods to prevent the situation. These options can be combined based on the deadlock status.

- Option.1: Re-establish the IMS PDN connection.

- When the Request Suppression Timer as described in clause 5.x.3.1 expires, the IMS PDN connection will be re-established by using the IMS PDN Disconnection as described in clause 5.x.4. By doing this method, the UE will then obtain the latest status of the IMS and PCRF.

- Option.2: New IE can be defined in clause 10.5.6.3 (Protocol configuration options) of TS 24.008/

- Ack required: If the PGW sends a notification of the IMS Recovery, it includes this IE in PCO. If the UE receives the notification from the PGW, it shall send an ACK message.

Editor's note:	Whether to define a new ID for ACK message is FFS.

5.x.6	PCRF failure handling with IMS status notification

This clause introduces PCRF failure handling by reusing the method described in clause 5.x.4.

When the PCRF is down, the Gx connection is lost. If the IMS PDN connection is maintained and the PGW notifies through the dedicated IEs in the PCO as described in clause 5.x.4.1 or the P-CSCF address as Null in the PCO as described in clause 5.x.4.2, it is possible to prevent frequent SIP Registration from the UE and the load on the PCRF can be mitigated.

When the PCRF is recovered and the Gx connection is up, the PGW disconnect and re-establish the IMS PDN.

5.x.7	UE indication of support for "Update PDP context/bearer at IMS status"

This optional extension is based on the possibility for the P-GW/GGSN to identify whether or not the UE supports "Update PDP context/bearer at IMS status".

The UE shall indicate this capability to the P-GW/GGSN at the activation of the IMS PDN connection /PDP context, in the PCO parameter 00xx (UE support IMS status) as described in 3GPP TS 24.008 [4]. The P-GW/GGSN shall store this UE capability.

This method has no impact on the MME/SGSN or SGW, as PCO information is transparently transferred through these network elements.

The PGW/GGSN-based P-CSCF basic mechanism is optionally extended by reusing part of the "Update PDP context/bearer at P-CSCF failure" mechanism described in clause 5.1. This extension is based on the possibility for the PCRF to know whether or not the UE supports the "Update PDP context/bearer at P-CSCF failure" mechanism. This is described in clause 5.x.3.2.

5.y	SMF-based IMS status notification for 5GC

5.y.1	Introduction

The clause 5.y describes solutions to support SMF-based IMS status notification in 5GC for UEs with an IMS PDU session.

If IMS service is required through 5GC, the UE performs Registration procedure and requests PDU Session Establishment procedure for IMS service, as specified in 3GPP TS 23.501 [28] and 3GPP TS 23.502 [29]. The UE selects P-CSCF via PDU Session Establishment procedure or via DHCP procedure and performs initial IMS registration (see 3GPP TS 23.228 [27]).

IMS failure may be detected during an originating SIP procedure or terminating SIP procedure. When IMS failure is detected, various mechanisms for IMS status notification can be invoked (by e.g. SMF, UDM or PCF), depending on the mechanism utilized.

5.y.2	IMS failure/recovery detection

For the IMS failure/recovery detection, the following three options are assumed. It is possible to use a combination of these three options, or to use just one according to operator policy.

- Option.1: Notification via PCF

- The PCF determines the IMS failure/recovery based on the status of Rx (Diameter). When the PCF detects the IMS failure/recovery, the PCF sends a new IE for IMS failure/recovery in the PolicyControlRequestTrigger data type as described in TS 29.512 to the SMF.

- Option.2: Detect the IMS failure/recovery through liveness monitoring.

- The SMF conducts liveness monitoring on the P-CSCF (e.g, ping)

NOTE: The monitoring configuration of the SMF depends on operator and implementation policy.

- Option.3: Manual operation by the Operator.

5.y.3	IMS failure notification

For the IMS failure notification, the following two options are assumed:

- Option.1: Notify the IMS failure with dedicated IEs within PCO as described in clause 5.y.3.1.

- Option.2: Notify the IMS failure with the P-CSCF address as Null within PCO as described in clause 5.y.3.2.

5.y.3.1	IMS failure notification with dedicated IEs via SMF

The procedure for SMF-based IMS failure/recovery notification with dedicated IEs is described in figure 5.y.3.1-1.

Figure 5.y.3.1-1: IMS failure notification with dedicated IEs via SMF

1.	The UE sends a PDU Session Establishment Request towards the AMF, to establish a PDU session for IMS service. If the UE supports to notify IMS status, the UE shall include an IMS status indicator within the SM container.

2.	The AMF invokes Nsmf_PDUSession_CreateSMContext service operation to the SMF, to request the SMF to create PDU session for IMS service.

3.	The SMF invokes Npcf_SMFPolicyControl_Get service operation to the PCF, to retrieve session related policy.

4.	The SMF selects appropriate UPF for IMS service and requests the UPF to establish N4 session.

5.	When PDU session is created, the SMF invokes Namf_Communication_N1N2MessageTransfer service operation to the AMF, to inform the request of PDU session establishment. The support of IMS status indication is also included in the ePCO as specified in 3GPP TS 24.008 [4], clause 10.5.6.3A.

6.	The AMF sends PDU Establishenment Response message to the UE, with the support of IMS status indication included in the ePCO as specified in 3GPP TS 24.008 [4], clause 10.5.6.3A.

7.	The UE performs an initial SIP registration towards the P-CSCF.

8.	With the methods described in clause 5.y.2, a failure in P-CSCF is detected.

9.	The SMF invokes Namf_Communication_N1N2MessageTransfer service operation, to trigger the PDU Session Modification procedure. Within the message, the SMF includes the IMS status and Request Suppression Timer in the ePCO IE as specified in 3GPP TS 24.008 [4], clause 10.5.6.3A.

10.	The AMF triggers the PDU Session Modification procedure to the UEbwhich supports the IMS status indication, to deliver the IMS status and Request Suppression Timer to the UE.

11. The UE identifies that the IMS is under failure condition. Then, the UE sets the timer based on the Suppression Timer in the PCO IE.

NOTE:	In the event of IMS failure, the UE will try to prioritize the restoration of MMTEL service and attempt initial registration, but this action can be suppressed.

12.	When the timer is elapsed, the UE sends an initial SIP registration towards the P-CSCF.

5.y.3.2	IMS failure notification with P-CSCF address as Null via SMF

The procedure for SMF-based IMS status notification with P-CSCF address as Null is described in figure 5.y.3.2-1.

Figure 5.y.3.2-1: IMS status failure and notification with P-CSCF address as Null via SMF

1.	The UE sends a PDU Session Establishment Request towards the AMF, to establish a PDU session for IMS service. If the UE supports P-CSCF restoration, the UE shall include a P-CSCF restoration indicator within the SM container.

2.	The AMF invokes Nsmf_PDUSession_CreateSMContext service operation to the SMF, to request the SMF to create PDU session for IMS service.

3.	The SMF invokes Npcf_SMFPolicyControl_Get service operation to the PCF, to retrieve session related policy.

4.	The SMF selects appropriate UPF for IMS service and requests the UPF to establish N4 session.

5.	When PDU session is created, the SMF invokes Namf_Communication_N1N2MessageTransfer service operation to the AMF, to inform the request of PDU session establishment. The support of P-CSCF restoration is also included in the ePCO as specified in 3GPP TS 24.008 [4], clause 10.5.6.3A.

6.	The AMF sends PDU Establishenment Response message to the UE, with the support of P-CSCF restoration icluded in the ePCO as specified in 3GPP TS 24.008 [4], clause 10.5.6.3A.

7.	The UE performs an initial SIP registration towards the P-CSCF.

8.	With the methods described in clause 5.y.2, a failure in P-CSCF is detected.

9.	The SMF invokes Namf_Communication_N1N2MessageTransfer service operation, to trigger the PDU Session Modification procedure. Within the message, the SMF includes the P-CSCF address as Null in the ePCO IE as specified in 3GPP TS 24.008 [4], clause 10.5.6.3A.

10.	The AMF triggers the PDU Session Modification procedure to the UE which supports the P-CSCF Restoration, to deliver the P-CSCF address as Null to the UE.

11. The UE identifies that the IMS is under failure condition. Then, the UE does not send an initial SIP registration towards the P-CSCF.

NOTE: 	In the event of IMS failure, the UE will try to prioritize the restoration of MMTEL service and attempt initial registration, but this action can be suppressed.

5.y.4	SMF based IMS recovery notification

For the IMS recovery notification, the following two options are assumed:

- Option.1: Notify the IMS recovery with dedicated IEs within PCO as described in clause 5.y.4.1.

- Option.2: Notify the IMS recovery with valid P-CSCF address within PCO as described in clause 5.y.4.2.

- Option.3: Disconnect the IMS PDU as described in clause 5.x.4.3.

5.y.4.1	IMS recovery notification with dedicated IEs via SMF

The procedure for SMF-based IMS recovery notification with dedicated IEs is described in figure 5.y.4.1-1.

Figure 5.y.4.1-1: IMS recovery notification with dedicated IEs via SMF

Steps from 1 to 12 are the same as explained in figure 5.y.3.1-1 above.

13.	With the methods described in clause 5.y.2, a recovery in P-CSCF is detected.

14.	The SMF invokes Namf_Communication_N1N2MessageTransfer service operation, to trigger the PDU Session Modification procedure. Within the message, the SMF includes the IMS status and Recovery Timer in the ePCO IE as specified in 3GPP TS 24.008 [4], clause 10.5.6.3A.

15.	The AMF triggers the PDU Session Modification procedure to the UE which supports the IMS status indication, to deliver the IMS status and Recovery Timer to the UE.

16. The UE identifies that the IMS is recovered. Then, the UE sets the timer based on the Recovery Timer in the PCO IE. This timer is designed to stagger the timing of each UE's Registration requests, preventing all UEs from making requests simultaneously (e.g., a random value to avoid simultaneous requests).

17.	When the timer is elapsed, the UE sends an initial SIP registration towards the P-CSCF.

5.y.4.2	IMS recovery notification with valid P-CSCF address via SMF

The procedure for SMF-based IMS recovery notification with valid P-CSCF address is described in figure 5.y.4.2-1.

Figure 5.y.4.2-1: IMS recovery notification with valid P-CSCF address via SMF

Steps from 1 to 11 are the same as explained in figure 5.y.4.2 above.

12.	With the methods described in clause 5.y.2, a recovery in P-CSCF is detected.

13.	The SMF invokes Namf_Communication_N1N2MessageTransfer service operation, to trigger the PDU Session Modification procedure. Within the message, the SMF includes the valid P-CSCF address in the ePCO IE as specified in 3GPP TS 24.008 [4], clause 10.5.6.3A.

14.	The AMF triggers the PDU Session Modification procedure to the UE which supports the P-CSCF Restoration, to deliver the valid P-CSCF address to the UE.

15. The UE identifies that the IMS is recovered.

16.	Tthe UE sends an initial SIP registration towards the P-CSCF.

5.y.4.3	IMS PDU Disconnection triggered by SMF

The procedure for SMF-based IMS recovery notification with IMS PDU Disconnection is described in figure 5.y.4.3-1. This option is reused to the P-CSCF restoration mechanism as defined in TS 23.380, allowing the IMS PDU reconnection and registration triggered by the IMS PDU disconnection. The advantage of this option is that if the PCF has established junk sessions, it can prompt the UE to reconnect according to the operator's policy.

Figure 5.y.4.3-1: IMS PDU Disconnection triggered by SMF

Steps from 14 to 19 and steps from 13 to 18 are the same as explained in figure 5.y.4.1 and figure 5.y.4.2 above.

18.	The PCF determined the IMS PDU Disconnection according to the operator's policy and invokes Npcf_SMPolicyControl_Update Request including an IMS PDU disconnect request.

19.	The network initiates a PDU session release with reactivation indication of the IMS PDU session according to 3GPP TS 23.502 [29], clause 4.3.4.

20.	The UE initiate IMS PDU re-establishment procedures.

21.	The UE sends an initial SIP registration towards the P-CSCF.

5.y.5	Deadlock prevention

Due to some dead lock issues such as the recovery notification was dropped, and the UE does not send a SIP Registration. There are two methods to prevent the situation. These options can be combined based on the deadlock status.

- Option.1: Re-establish the IMS PDU connection.

- When the Request Suppression Timer as described in clause 5.y.3.1 expires, the IMS PDU connection will be re-established by using the IMS PDU Disconnection as described in clause 5.x.4. By doing this method, the UE will then obtain the latest status of the IMS and PCRF.

- Option.2: New IE can be defined in clause 10.5.6.3 (Protocol configuration options) of TS 24.008/

- Ack required: If the SMF sends a notification of the IMS Recovery, it includes this IE in PCO. If the UE receives the notification from the SMF, it shall send an ACK message.

Editor's note:	Whether to define a new ID for ACK message is FFS.

5.y.6	PCF failure handling with IMS status notification

This clause introduces PCF failure handling by reusing the method described in clauses 5.y.2, 5.y.3, 5.y.4 and 5.y.5.

For the PCF failure, If the IMS PDU connection is maintained and the SMF notifies through the dedicated IEs in the PCO as described in clause 5.y.3.1 or the P-CSCF address as Null in the PCO as described in clause 5.y.3.2, it is possible to prevent frequent SIP Registration from the UE and the load on the PCF can be mitigated.

When the PCF is recovered, the SMF disconnect and re-establish the IMS PDU.

5.y.7	UE indication of support for "Update PDP context/bearer at IMS status"

This optional extension is based on the possibility for the SMF to identify whether or not the UE supports "Update PDP context/bearer at IMS status".

The UE shall indicate this capability to the SMF at the activation of the IMS PDN connection /PDP context, in the PCO parameter 00xx (UE support IMS status) as described in 3GPP TS 24.008 [4]. The SMF shall store this UE capability.

This method has no impact on the AMF and SMF, as PCO information is transparently transferred through these network elements.

The PCRF-based P-CSCF basic mechanism is optionally extended by reusing part of the "Update PDP context/bearer at P-CSCF failure" mechanism described in clause 5.1. This extension is based on the possibility for the PCRF to know whether or not the UE supports the "Update PDP context/bearer at P-CSCF failure" mechanism. This is described in clause 5.y.3.2.

2.1.5	TS 24.008 (CT1)

Updating table 10.5.154 in clause 10.5.6.3 of TS 24.008:

-------------------------------------

Table 10.5.154/3GPP TS 24.008: Protocol configuration options information element

Additional parameters list (octets w+1 to za) MS to network direction: - 0001H (P-CSCF IPv6 Address Request); - : - 0056H (UE policy container with the length of two octets); - XXXXH (URSP provisioning in EPS support indicator); XXXXX: (UE support IMS status) - FF00H to FFFFH reserved for operator specific use. Network to MS direction: - 0001H (P-CSCF IPv6 Address); - - XXXXH (URSP provisioning in EPS support indicator); - XXXXX: (Request Suppression Timer); - XXXXX: (IMS status); - XXXXX: (Recovery timer); - XXXXX: (Ack Required); and - FF00H to FFFFH reserved for operator specific use. If the additional parameters list contains a container identifier that is not supported by the receiving entity the corresponding unit shall be ignored.

The container identifier field is encoded as the protocol identifier field and the length of container identifier contents field is encoded as the length of the protocol identifier contents field. : When the container identifier indicates P-CSCF IPv6 Address, the container identifier contents field contains one IPv6 address corresponding to a P-CSCF address (see 3GPP TS 24.229 [95]). This IPv6 address is encoded as a 128-bit address according to IETF RFC 4291 [99]. When there is a need to include more than one P-CSCF IPv6 address, then more logical units with the container identifier indicating P-CSCF IPv6 Address are used. If more than 3 instances of the P-CSCF IPv6 Address logical unit are received by the MS, then the MS may ignore all but the first 3 instances of the P-CSCF IPv6 Address logical unit received. When the container identifier contents field is empty, the MS may suppress to attempt initial SIP registration. : When the container identifier indicates P-CSCF IPv4 Address, the container identifier contents field contains one IPv4 address corresponding to the P-CSCF address to be used. When there is a need to include more than one P-CSCF IPv4 address, then more logical units with the container identifier indicating P-CSCF IPv4 Address are used. If more than 3 instances of the P-CSCF IPv4 Address logical unit are received by the MS, then the MS may ignore all but the first 3 instances of the P-CSCF IPv4 Address logical unit received. When the container identifier contents field is empty, the MS may suppress to attempt initial registration. : When the container identifier indicates UE support IMS status, the container identifier contents field is empty and the length of container identifier contents indicates a length equal to zero. If the container identifier contents field is not empty, it shall be ignored. This information indicates that the UE supports IMS status. When the container identifier indicates IMS status, the container identifier contents field contains IMS status parameters. When the container identifier indicates UE support Request Suppression Timer, the container identifier contents field contains the time until the MS initiates a SIP Registration Request. When the container identifier indicates Recovery timer, the container identifier contents field contains the time for resuming the SIP Registration from the MS. This timer is designed to stagger the timing of each MS's Registration requests, preventing all MSs from making requests simultaneously (e.g., a random value to avoid simultaneous requests). When the container identifier indicates Ack Required, the container identifier contents field contains true or false. If the PGW sends a notification of the IMS Recovery, it may include this IE. If the MS receives the notification from the PGW and contents field contains true, it shall send an ACK message.

-------------------------------------

2.1.6	TS 24.501 (CT1)

Adding following procedures to clause 6.3.2.2 of TS 24.501:

6.3.2.2	Network-requested PDU session modification procedure initiation

In order to initiate the network-requested PDU session modification procedure, the SMF shall create a PDU SESSION MODIFICATION COMMAND message.

:

If the PDU session was a single access PDU session established over 3GPP access with IP PDU session type and based on operator policy the SMF determines to provide the protocol description for UL PDU set handling to the UE, the SMF may include the Protocol description IE in the PDU SESSION MODIFICATION COMMAND message.

When UE has the support of IMS status indication, if the network-requested PDU session modification procedure is triggered for IMS status notification, the SMF shall include the IMS status (e,g, Not applicable, applicable) and the Request Suppression Timer or Recovery timer and may include Ack required in the Extended protocol configuration options IE in the PDU SESSION MODIFICATION COMMAND message as specified in subclause 5.x of TS 23.380 [54].

-------------------------------------