3GPP TSG WG Meeting 167	S2-2501775

Athens, 17 – 21 Feb. 2025	revision of S2-2500748

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5996 | rev | 1 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving ENs in Clause 5.51.5 and 5.51.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., vivo, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For resolving Editor’s notes listed below: - Editor's note: Whether the Energy Saving indicator is also stored in the UDR is FFS. 2 Editor's note: A notification for the energy related information, if any, should not be provided from the UDM/UDR. Whether the notification for the energy related information from the EIF is provided to the PCF and what are the notification details, are FFS. - Since the value is accessed by AMF, it should be stored in UDR for AMF to retrive - The PCF can be notified by EIF with the energy related information and to see whether the it needs to re-make the policy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To resolve ENs based on below changes: - The energy saving indicator can be stored in UDR - The PCF can be notified by EIF with the energy related information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not complete feature for EnegySys |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.5, 5.51.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.51.5	Subscription information aspects

The Subscription data for a UE may include an Energy Saving indicator that the UE is subject to network energy saving operation. The Energy Saving indicator is stored in UDM/UDR.The AMF retrieves the Energy Saving indicator in the UE Subscription data. The AMF forwards the Energy Saving indicator to the NG-RAN and to the PCF.

Editor's note:	The indicator is expected to select an energy saving strategy for the UE in the system. Whether there are multiple values of the indicator is FFS.

Editor's note:	Whether the indicator is provided to the NG-RAN andhow the indicator it is used in the NG-RAN is FFS.

5.51.6	Policy control aspects

During AM policy association establishment procedure, the AMF sends to the PCF the Energy Saving indicator as described in clause 5.51.5. The PCF may also subscribe to the notifications from EIF for energy consumtion information. Based on the Energy Saving indicator and EIF notifications, the PCF may make policy decisions reusing the existing parameters based on the operator policy.