3GPP TSG-SA2 Meeting #167	S2-2501784

# Athens, Greece, February 17 – 21, 2025	(revision of S2-2500559)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3855 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transitioning between operation in S&F mode and normal mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For 5GGSAT_PH3-ARC, following work task is included in Work Item Exception (2b) – Functional specification of expected UE and network behaviour when transitioning between operation in S&F mode and operation in normal mode As described in TS 23.401, when both the service link and feeder link are available, all services can be provided. However, user plane establishment and user plane data transfer cannot be supported, when operating in S&F Mode in the split MME deployment. Therefore, the UE and the MME need to re-negotiate their capabilities when trantransition between the S&F and normal operation modes. To do this, Attach or TAU procedure need to be performed. The operation mode of a satellite depends on the connectivity of the feeder link. We propose that when a satellite switching between the S&F and normal operation modes depending on the feeder link availability, the MME-onboard may decide to trigger the S1 release procedure with the existing cause "load balancing TAU required" or detach procedure so that the UE to initiate TAU or Attach procedure by considering, e.g. the UE's preferred Network Behaviour, MT data to be delivered, and/or subscription data. For idle UEs, the MME may wait until the UE become ECM-CONNECTED, or may paging the UE to be CONNECTED, and the MME can control that the procedures are not triggered simultaneously for serving UEs to avoid signaling congestion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MME-onboard may trigger S1 release procedure with the existing cause "load balancing TAU required" or detach procedure when transition between the S&F mode and normal mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE and network expected behaviour when transitioning between operation in S&F mode and non-S&F remains undefined and may lead to mis-operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.13.9.X	Transition between S&F and normal operation modes

When the satellite switches between the S&F and normal operation modes depending on feeder link availability (from S&F mode to normal mode or from S&F mode to normal mode), the MME may decide to initiate the S1 Release procedure with release cause "load balancing TAU required" (clause 5.3.5) or detach procedure, e.g. determined by considering the Network Behaviour the UE can support, MT data to be delivered, and/or subscription data. The S1 and RRC connections are released, and the UE is detached if the MME triggered detach procedure. Then, the UE initiates a TAU or an Attach.

NOTE X:	Depending on the operator policy or regulation requirement, the satellite may change the operation mode immediately or at a later time when the feeder link availability changes.

* * * * End of Changes * * * *