3GPP SA WG2 Meeting #167	S2-2501792

# Athens, Greece, 17-21 February 2025							               	  (was S2-2500173)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1534 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on third-party user ID provisioning procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The use of third-party user identity can be authorized for both originating UE and IP-PBX in a session. However, there may be differences in how a third-party identity information is provisioned in the SIP INVITE: - When the session request is triggered by a UE, the third-party user identity information is inserted by the originating IMS network. - When the session request is triggered by an IP-PBX, the third-party user identity information may be inserted by the IP-PBX. In this case, the originating IMS does not need to fetch the third-party user identity information from the HSS/RCD server |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the differences in third-party user identity information provisioning for UE and IP-PBX. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is confusion about when and how to provide third-party user identity in the SIP INVITE from UE and IP-PBX. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AF.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES <<<<

# AF.3	Procedure for signing and verification of third party user identity information in IMS

Figure AF.3-1 depicts the procedure for signing and verification of third party user identity information in IMS.

Figure AF.3-1: Procedure for signing and verification of third party user identity information in IMS

1-2.	If the SIP INVITE is received from UE-A, based on operator’s policy and user subscription data, the orginating IMS network determines whether to include third party user identity information.

If the SIP INVITE is received from the IP-PBX and the third party network is providing third party user identity information in the outgoing initial SIP INVITE request and is authorized to use third party user identity information, and based on operator policies, S-CSCF, IMS AS or IBCF invoke signing of the provided third party user identity information with a signing AS and steps 3-5 are skipped.

3.	The originating IMS AS may retrieve RCD server address, RCD URL, or RCD information from the HSS, based on the originating IMPU or wildcarded IMPU.

4.	The originating IMS AS may receive RCD server address, RCD URL, or RCD information from the HSS.

5.	If the IMS AS has retrieved an RCD URL, the IMS AS, based on configuration, may retrieve the RCD information from the RCD server using the RCD URL. The IMS AS includes either the RCD URL or, if retrieved from the RCD server, the RCD information in the outgoing SIP INVITE request.

If the IMS AS has retrieved an RCD server address, the IMS AS provides the originating IMPU or wildcarded IMPU to the RCD server, to retrieve the RCD information. The IMS AS includes the retrieved RCD information in the outgoing SIP INVITE request.

NOTE 1:	How RCD URL or RCD information are carried in SIP signalling is defined in TS 24.229 [10a].

6-8.	The originating IMS network invokes signing of the RCD information or RCD URL included in the outgoing SIP INVITE request with a signing AS.

NOTE 2:	Based on operator policies, either the S-CSCF, IMS AS or IBCF in the originating network invoke signing of the RCD information or RCD URL.

9-10.	Based on operator policies, the S-CSCF, IMS AS or IBCF in the terminating IMS network invoke signature verification of the RCD information or RCD URL included in the incoming SIP INVITE request with a verification AS. The S-CSCF, IMS AS or IBCF in the terminating IMS network may reject the call when the signature verification is unsuccessful.

11-12.	The terminating UE may present third party user identity information of the calling-party to the called party retrieved from RCD information, if present, in the SIP INVITE request. The called party may use the RCD URL, if present, to retrieve RCD information from the RCD server to download third party user identity information.

# >>>>END OF CHANGES<<<<