3GPP TSG-WG SA2 Meeting #167	S2-2501831

# Athens, Greece, 17th Feb – 21st Feb, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6121 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UCMF discovery in case of Indirect Network Service (INS) deployment deployment-23.501. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_NetShare |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In case of Indirect network sharing , when the UE is registerin to the participating network, the UE radio capability will be assigned by the UCMF of the participating network. However, how the serving AMF of the host operator network discover and select the UCMF of the participating network is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that the serving AMF of the host network includes the PLMN ID of the selected PLMN in Nnrf_NFDiscovery Request message and follow the procedure in subclause 4.17.5 NF/NF service discovery across PLMNs in the case of discovery made by NF service consumer of TS 23.502 to discover UCMF of the participating operator. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how the UCMF of the participating operator is discovered and selected. UE radio capability |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### . 6.3.15	UCMF Discovery and Selection

The AMF, MME, NEF, AF, SCEF, SCS/AS may utilize the NRF to discover UCMF instance(s) unless UCMF information is available by other means, e.g. locally configured in UCMF services consumers.

In the case of delegated discovery and selection in SCP, the NF consumer shall forward the request towards SCP.

- The PLMN ID (MCC and MNC) of the selected PLMN in case of Indirect Network Sharing (INS). The PLMN ID may include MCC and MNC of the hosting operator network or the MCC and MNC of the GUAMI of the 5G-GUTI assigned to the UE when the UE has registered to the operator PLMN (i.e. MCC and MNC of the participating operator network).

In case of INS when a UE registers to a hosting network with a UCMF in the participating network, the consumer NF (e.g AMF) sends a Nnrf_NFDiscovery Request message containing home PLMN ID set to the PLMN ID of the selected PLMN of the UE (i.e. Participating PLMN ID) and serving PLMN ID set to the host PLMN ID to the NRF of the host PLMN. The NRF of the host PLMN selects participating PLMN NRF corresponding to the host PLMN ID received in the Nnrf_NFDiscovery Request message and sends the Nnrf_NFDiscovery Request to participating PLMN NRF. The participating PLMN NRF sends the IP address or FQDN of the UCMF of the participating PLMN to the AMF via NRFs in the participating PLMN and the host PLMN. The NF consumer and the NRF follows the procedure as described in sub clause 4.17.5 of TS 23.502. The AMF uses this UCMF to get the UE radio capability ID corresponding to the UE radio access capability information and vice versa.

# * * * * End of Change * * * *