3GPP SA WG2 Meeting #167	S2-2501899

# Athens, 17 – 21 February 2025

# Title:	[Draft] LS Reply on emergency services and eDRX

# Response to:	LS (S2-2501367/R2-2411041)

# Release:	Rel-17

# Work Item:	NR_newRAT-Core, NR_redcap-Core

Source:	[Nokia to be] SA2

To:	RAN2

Cc:	RAN3, CT1

Contact Person:

Name:	Dongjoo Kim

E-mail Address:	dongjoo.kim@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.501 Rel-17 CR 6132, TS 23.501 Rel-18 CR 6133, TS 23.501 Rel-19 CR 6134

1. Overall Description:

SA2 thanks RAN2 for the LS on emergency services and eDRX (S2-2501367/R2-2411041) and would like to provide the following feedback:

- RAN2 question: RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed?

- SA2 Answer: SA2 has analysed the identified issue and provides the following outcomes:

- The mechanism outlined in TS 23.503 Clause 6.1.3.10 prevents the termination of an emergency PDU session but does not prevent the deactivation of the UP connection associated with the session.

- The mechanism specified in TS 23.501 Clause 5.6.8 does not prevent the deactivation of the UP connection for an emergency PDU session.

- Currently, the gNB is only aware of an existing emergency PDU session if the UP connection for the session is activated, as indicated by NGAP signaling. If the gNB detects the activated UP connection for the emergency PDU session, there is no issue, as the implementation can prevent the eDRX configuration.

- However, as mentioned in point 2, an emergency PDU session can still exist without an activated UP connection. This scenario is problematic, as the gNB might mistakenly consider the PDU session inactive and release the UE to the RRC_INACTIVE state with a RAN eDRX cycle.

- To address this issue, the attached CRs were agreed upon, which modify TS 23.501 Clause 5.31.7.2.1 to prevent the use of RAN eDRX by the AMF, either by not sending the NR eDRX information in the CNAI for RRC_INACTIVE to the gNB or by removing it if it was previously sent.

2. Actions:

To RAN2 group:

ACTION:  SA2 kindly requests RAN2 to consider the provided answers in their further work.

3. Date of Next SA2 Meetings:

SA2#168                                    7th – 11th April 2025	 		Goteborg, Sweden

SA2#169                                    19th – 23rd May 2025	 		Fukuoka, Japan