3GPP TSG-WG SA2 Meeting #167	S2-2501901

Athens, 17th Feb 2025 - 21st Feb 2025	(revision of S2-24xxxxx)

Source:	Nokia

Title:	Conclusions for KI#1 on the aggregation aspects (Topic#3)

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusions for KI#1 on the aggregation aspects are proposed.

# 1. Introduction/Discussion

This contribution is prepared to update the conclusions for KI#1 on the aggregation aspect related to the EN below.

Editor's note:	How the aggregation can be done is FFS.

It is proposed for the AIOTF to determine the aggregation assistance information based on some factors and provide it to the AIoT RAN Reader so that it can use the information to determine whether and how to aggregate the data read from multiple AIoT devices.

S2-2500463 was noted in SA2#166AH-e, where it was proposed that AIOTF could provide "aggregation assistance information" to guide how AIoT RAN nodes and/or BS readers perform aggregation.

In our view, this feature should be supported in Release 19, as aggregation is a critical component for most AIoT use cases. It is impractical to expect the reader and/or AIoT RAN to forward messages from AIoT devices every time data is received. For example, in services like inventory management, it is typically assumed that the reader would provide a list of AIoT device IDs.

Some companies suggested that aggregation in the reader could be supported through implementation-specific solutions. However, we believe that relying solely on implementation is insufficient, given that AIoT RAN and AIOTF may be developed by different vendors. Without a standardized mechanism, coordinating aggregation methods would become overly complex.

The “aggregation assistance information” may include several parameters:

- Per time window: The reader periodically reports aggregated data at each time window. This differs from the latency requirement specified by AF, which mandates responding to data requests from all or part of the target AIoT devices within a specific period. In contrast, the responses in this scenario would be provided multiple times within each time window.

- Per reader: The AIoT RAN node aggregates data on a per BS reader basis. Once it confirms that a BS reader has completed its reporting, the AIoT RAN node aggregates all the data from that reader and forwards it to the AIOTF.

- Per AIoT device type: The reader and/or the AIoT RAN node can aggregate the data from the same types of AIoT devices (e.g., sensor, tag, actuator, etc).

- Per priority level: In case the AF specified a specific device IDs with a priority, the reader and/or the AIoT RAN node can first aggregate the data from those AIoT devices and report it to the AIOTF.

- Any combination of above.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13 v1.3.0.

FIRST CHANGE

## 8.1	Conclusion on Key Issue #1

### 8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

In this release, normative work will take place for Topology 1 and no normative work will take place for Topology 2.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

1.	A new core network function is introduced to support Ambient IoT (e.g. AIOTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIOTF registers itself in the NRF with its NF profile, this is to enable the discovery of AIOTF instances e.g. by an NEF. The NF profile at least includes the AIOTF ID/address, NF type, information used to allow the NEF to discover AIOT instances, e.g. based on the target area information in AF request. Other details of the NF profile, if needed, will be completed in normative phase.

b.	For topology 1, the AIOTF selects AIOT RAN nodes and optionally a list of the BS readers. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

For topology 1, NGAP is terminated at an AIOT RAN node. An AIOT RAN node that supports one or more BS readers, may report its supported BS reader information e.g., BS reader IDs and their corresponding service areas. The OAM may configure the AIOTF with BS Reader information including e.g., their corresponding service area when the BS Readers are integrated in the network.

NOTE 2:	Whether RAN3 will specify BS reader information (e.g. BS reader ID, service area) reporting to AIOTF over NGAP depends on RAN3 decision and if so SA2 may include this feature in SA2 specifications.

NOTE 3:	If the request from the AIOTF to the AIOT RAN does not include a BS reader list, then AIOT RAN can use all available BS readers.

NOTE 4:	The BS reader’s service area needs to coordinate with RAN WG(s).

c.	The AIOTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIOTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

e.	The AIOTF may provide the following assistance information to AIoT RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

NOTE 5:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 6:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

NOTE 7:	Further assistance information can be added during the normative phase and in cooperation with other WGs if necessary.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

f.	The AIOTF may provide the AIoT service requirements, if provided by AF, to the AIoT RAN. Aggregation assistance information, if included, is used to assist in the aggregation of AIoT data received from multiple AIoT devices. The AIOTF may determine the aggregation assistance information based on operator policy and/or request from the AF, which includes:

- 	Aggregation per AIoT device type (e.g., sensor, tag)

- 	Aggregation per time window (e.g., repeatedly aggregate and report data per a time period)

-	Aggregation per reader (e.g., AIoT RAN node aggregates data per BS reader)

-	Aggregation per priority level (e.g., if particular AIoT devices are requested with priority by the AF)

-	Aggregation per number of AIoT devices (e.g., for inventory, threshold for the maximum number of AIoT device IDs required for reporting)

-	Any combination of above (if needed)

NOTE X: The AIoT service operations being aggregated from the AIoT devices share the same service identifier.

NOTE X:	If the AIOTF does not provide aggregation assistance information, the aggregation process in the AIoT RAN becomes implementation-dependent. Consequently, how AIOTF or AF interprets the aggregated data is also left to individual implementation decisions.

g.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 8:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIOTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

5.	The AIOTF may store and manage the AIoT device related information (also known as device context information) locally that includes e.g., the AIOT device permanent ID, the last known reader information of the AIoT device. The last known reader information can be used to support the AIOTF to select the serving reader to forward the message towards the specific AIoT device(s).

NOTE 9:	The storage of security related information at the AIOTF is up to the coordination with SA WG3 later.

Second CHANGES

### 8.3.5	Information provided by the AF

The AF provides one or some of the following information:

-	Information about AIoT services:

-	Service type indicating e.g. inventory or command (e.g. read or write).

-	Information to be used for AIoT reader selection:

-	At least one of the UE reader ID or the target area information.

-	Optionally, Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

-	Optionally, Information to be used for resource allocation that is detailed in TR 38.769 [8]:

-	Approximate number of AIoT devices.

-	Approximate D2R message size.

NOTE:	Information to be used for resource allocation will be aligned with RAN WGs during the normative phase.

- Optionally, Information about AIoT service requirements:

-	Aggregation requirements (e.g., per device type, per time window, per priority level, minimum number of devices for reporting, any combination of the requirements)

END OF CHANGES